3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220982

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2698 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 6.3.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Keeping coordinate flag as true by default in f_IMS_EmergencyCallSetup_NR5GC_A_6() affects coordination between IMS PTC and NR5GC PTC and test case gets stuck in various steps of IMS MO Emergency call release procdure as per Table 4.9.12A.2.2-1. Also, f_IMS_EmergencyCallAndReleaseWithoutRegistration() has already handled those cor-ordination message to inform and wait from NR5GC PTC for f_NR5GC_EmergencyCallPDUSession() and f_NR5GC_EmergencyCallRelease(). - During f_Check_NG_PDUSessionReleaseReq call v_PDUSession will get updated and thus some of the stored param like v_PDUSession.PdnIndex will be omitted during the f_IP_RtpRtcpLoopback() call and will lead to run time CompulsoryValueOmittedException error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Set p_WaitForTrigger and p_SessionModification as false in f_IMS_EmergencyCallSetup_NR5GC_A_6() call. - Store v_PDUSession.PdnIndex into a separate variable before passing it to subsequent handling functions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | These changes are needed after implementation of R5s220789 (MCCTF160 implementation) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to f_IMS_EmergencyCallAndReleaseWithoutRegistration	3

2.2	Correction to function f_NR5GC_EmergencyCallRelease	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to f_IMS_EmergencyCallAndReleaseWithoutRegistration

| Function name | f_IMS_EmergencyCallAndReleaseWithoutRegistration |
| --- | --- |
| Reason for change | Keeping coordinate flag as true by default in f_IMS_EmergencyCallSetup_NR5GC_A_6() affects coordination between IMS PTC and NR5GC PTC and test case gets stuck in various steps of IMS MO Emergency call release procdure as per Table 4.9.12A.2.2-1. Also, f_IMS_EmergencyCallAndReleaseWithoutRegistration() has already handled those cor-ordination message to inform and wait from NR5GC PTC for f_NR5GC_EmergencyCallPDUSession() and f_NR5GC_EmergencyCallRelease(). |
| Summary of change | Set p_WaitForTrigger and p_SessionModification as false in f_IMS_EmergencyCallSetup_NR5GC_A_6() call. |
| TTCN module | \Common\IMS\IMS_NR5GC_Functions.ttcn |
| MCC160 Comment | Alternative solution implemented: correct the appropriate co-ordination messages on the NR5GC side |

Before Change:

function f_IMS_EmergencyCallAndReleaseWithoutRegistration (INVITE_A_2_1_Context_Type p_Context := A_2_1_A6) runs on IMS_PTC { var INVITE_Request v_InviteRequest; // Emergency call setup: 34.229-5 Annex A.6 v_InviteRequest := f_IMS_EmergencyCallSetup_NR5GC_A_6(p_Context); // @sic R5s210875, R5-217857 sic@ // Inform NR: Emergency Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //Release Emergency Call MO f_IMS_EmergencyCallRelease(v_InviteRequest); // Inform NR: Emergency Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

After Change:

function f_IMS_EmergencyCallAndReleaseWithoutRegistration (INVITE_A_2_1_Context_Type p_Context := A_2_1_A6) runs on IMS_PTC { var INVITE_Request v_InviteRequest; // Emergency call setup: 34.229-5 Annex A.6 v_InviteRequest := f_IMS_EmergencyCallSetup_NR5GC_A_6(p_Context, -, -, false, false); // @sic R5s210875, R5-217857 sic@ WA#WI=988512 // Inform NR: Emergency Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //Release Emergency Call MO f_IMS_EmergencyCallRelease(v_InviteRequest); // Inform NR: Emergency Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

#### MCC160 Implementation

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC

{

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11);    //@sic R5-204410 sic@

var O2_Type v_AccessTechId :='0800'O;

var octetstring v_Data := v_PlmnID_2 & v_AccessTechId;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

//@siclog "Step 1" siclog@

//Power on the UE

//@siclog "Step 1A1-1A13" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed    //@sic R5-204410 sic@

f_NR5GC_Preamble_Steps1_13(nr_Cell11);

//@siclog "Step 1B" siclog@

//SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes.    //@sic R5-204410 sic@

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B)))));

//@siclog "Step 1C" siclog@

//The SS releases the RRC connection    //@sic R5-204410 sic@

f_NR_RRCRelease(nr_Cell11);

//@siclog "Step 2" siclog@

//The UE is made to establish an emergency call. This can be done by an AT/MMI command

f_UT_RequestIMSEmergencyCall (UT);

//@siclog "Step 3" siclog@

// The UE transmits an RRCSetupRequest message on NR Cell 11 with establishmentCause set to 'emergency'.

SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency)))); //@sic R5s201466 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received"); //@sic R5s200305 sic@

//@siclog "Steps 4-5" siclog@

//Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed

//@siclog "Step 6" siclog@

//The UE transmit a REGISTRATION REQUEST message with IE 5GS registration type set to "Emergency registration" or "Initial registration"//@sic R5-210807 R5s201466 sic@

//@siclog "Steps 7-14" siclog@

//Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 15" siclog@

//The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing

//steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception

f_NR5GC_SorRegistration(nr_Cell11,v_Data,v_Header,true,Initial_Secure, tsc_SHT_IntegrityProtected);//@sic R5s210236 R5s201455 R5-205361 sic@//using Initial_Secure to indicate the security context state.

//@siclog "Step 16" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container

f_NR5GC_Registration_Complete(nr_Cell11);

//@siclog "Step 17" siclog@

//Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13

//@siclog "Step 18-22" siclog@//@sic R5-216165 sic@

//Steps 9 to 13 of the procedure of "IMS Emergency call establishment in 5GC with IMS emergency registration" described in TS 38.508-1 [4] Table 4.9.11.2.2-1 are performed.

f_NR5GC_EmergencyCallPDUSession(nr_Cell11,tsc_NR_RbId_SRB1);//@sic R5s211479 sic@

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

//@siclog "Step 23" siclog@//@sic R5-216165 sic@

//Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed.

f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5s211479 sic@

// The above function won't finish until IMS messages have been sent and PDU session has been released @sic R5s220982 sic@

//@siclog "Step 24" siclog@//@sic R5-216165 sic@

//The SS releases the RRC connection.

f_NR_RRCRelease(nr_Cell11);

}//End of fl_TC_6_3_1_7_TestBody

## Correction to function f_NR5GC_EmergencyCallRelease

| Function name | function f_NR5GC_EmergencyCallRelease |
| --- | --- |
| Reason for change | During f_Check_NG_PDUSessionReleaseReq call v_PDUSession will get updated and thus some of the stored param like v_PDUSession.PdnIndex will be omitted during the f_IP_RtpRtcpLoopback() call and will lead to run time CompulsoryValueOmittedException error. |
| Summary of change | Store v_PDUSession.PdnIndex into a separate variable before passing it to subsequent handling functions. |
| TTCN module | Common\NR5GC\NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Already implemented under R5s220829 |

Before Change:

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId, boolean p_MOTriggerRequired := true, boolean p_IMSMsgsNotYetSent := true) runs on NR5GC_PTC //WA#WI=984946 R5s220789 TF160 Comments { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var boolean v_WaitForReq := true; // @sic R5s220524 sic@ timer t_Wait := 5.0; if (p_MOTriggerRequired) { // @sic R5s220531 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s220039 sic@ } t_Wait.start; alt { // @sic R5-216829 sic@ [v_WaitForReq] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp // @sic R5s211524 sic@ { if (not f_Check_NG_PDUSessionReleaseReq (v_PDUSession, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request failed"); } t_Wait.stop; v_WaitForReq := false; repeat; } [p_IMSMsgsNotYetSent] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger) // @sic R5s220039 sic@ //WA#WI=984946 R5s220789 TF160 Comments { t_Wait.stop } [] t_Wait.timeout{ // @sic R5s220039 sic@ if (p_IMSMsgsNotYetSent){ repeat; } } } f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }

After Change:

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId, boolean p_MOTriggerRequired := true, boolean p_IMSMsgsNotYetSent := true) runs on NR5GC_PTC //WA#WI=984946 R5s220789 TF160 Comments { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var PDN_Index_Type v_PDN_Index := v_PDUSession.PdnIndex; // WA#WI=988512 var boolean v_WaitForReq := true; // @sic R5s220524 sic@ timer t_Wait := 5.0; if (p_MOTriggerRequired) { // @sic R5s220531 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s220039 sic@ } t_Wait.start; alt { // @sic R5-216829 sic@ [v_WaitForReq] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp // @sic R5s211524 sic@ { if (not f_Check_NG_PDUSessionReleaseReq (v_PDUSession, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request failed"); } t_Wait.stop; v_WaitForReq := false; repeat; } [p_IMSMsgsNotYetSent] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger) // @sic R5s220039 sic@ //WA#WI=984946 R5s220789 TF160 Comments { t_Wait.stop } [] t_Wait.timeout{ // @sic R5s220039 sic@ if (p_IMSMsgsNotYetSent){ repeat; } } } f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDN_Index, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ WA#WI=988512 f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }