3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220983

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2699 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of test case 9.1.9.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 9.1.9.3 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.1.9.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches executed	5

4	Execution Log Files	6

4.1	Samsung SM-G996B	6

5	References	6

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.1.9.3 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

  - Verification Test Summary

Test Case: 	9.1.9.3

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Samsung SM-G996B

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | function f_TC_9_1_9_3_NR5GC() |
| --- | --- |
| Reason for change | PLMN and TAC of cell A are accessed fetched before cell has been initialised, giving a run-time error |
| Summary of change | Moved PLMN and TAC information access after cell initialisation. |
| TTCN module | RACS_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change

function f_TC_9_1_9_3_NR5GC() runs on NR5GC_PTC { //RACS / PLMN change within registration area / From NW assigned to Manufacturer assigned UE Radio Capability ID var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NAS_PlmnId v_PLMN:= f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; f_NR5GC_Init_NAS(NR_2); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellB); f_NR_CellConfig_Def(v_NGC_NASCellE); v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13 (v_NGC_NASCellA); if (not match (v_GMM_MobilityInfo.GMMCap, cdr_NG_GMM_Cap_RACS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Registration Accepted ………………………….. }

After Change

function f_TC_9_1_9_3_NR5GC() runs on NR5GC_PTC { //RACS / PLMN change within registration area / From NW assigned to Manufacturer assigned UE Radio Capability ID var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; f_NR5GC_Init_NAS(NR_2); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellB); f_NR_CellConfig_Def(v_NGC_NASCellE); v_PLMN:= f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13 (v_NGC_NASCellA); if (not match (v_GMM_MobilityInfo.GMMCap, cdr_NG_GMM_Cap_RACS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Registration Accepted …………………… }

  - Change 2

| Function name | template cr_UERadioCapId_Manu |
| --- | --- |
| Reason for change | The id field is “octetstring” type so in the template each “?” charecter in this field will match one octet (=2 digits). For Manufacturer-assigned Radio Capability Id, the id will consisit of 1-digit TF (=0), 8-digit vendor id and 11 digit RCI field (as 23.003 29.2). |
| Summary of change | Changed expected id to '00?????????'O |
| TTCN module | RACS_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change

template (present) UERadioCapId cr_UERadioCapId_Manu (IEI8_Type p_IEI) := { /* @status APPROVED (NR5GC_IRAT) */ iei := p_IEI, iel := int2oct(10, 1), id := '00?????????????????'O };

After Change

template (present) UERadioCapId cr_UERadioCapId_Manu (IEI8_Type p_IEI) := { /* @status APPROVED (NR5GC_IRAT) */ iei := p_IEI, iel := int2oct(10, 1), id := '00?????????'O };

- Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

- Execution Log Files

  - Samsung SM-G996B

The Samsung SM-G996B UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_9_1_9_3_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_9_1_9_3_PIXIT.xml

- References

| [1] | R5s220984 : NR5GC FR1 : Supporting information for addition of test case 9.1.9.3 |
| --- | --- |