3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221088

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2743 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR_Derive_AddXDD_UE_NR_Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the current implementation of the function , f_NR_Derive_AddXDD_UE_NR_Capabilities pc_SFTD_MeasNR_Neigh_FDD and pc_SFTD_MeasNR_Neigh_TDD are checked. - But - as per 38306 , sftd-MeasNR-Neigh is not mandatory IE - as well as based on 38331 , sftd-MeasNR-Neigh ENUMERATED {supported} OPTIONAL, - This check will fail the UEs not supporting sftd-MeasNR-Neigh |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the check of pc_SFTD_MeasNR_Neigh_TDD and pc_SFTD_MeasNR_Neigh_FDD from the function f_NR_Derive_AddXDD_UE_NR_Capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the SA and ENDC cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the function f_NR_Derive_AddXDD_UE_NR_Capabilities	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_NR_Derive_AddXDD_UE_NR_Capabilities

| Template name | f_NR_Derive_AddXDD_UE_NR_Capabilities |
| --- | --- |
| Reason for change | - In the current implementation of the function , f_NR_Derive_AddXDD_UE_NR_Capabilities pc_SFTD_MeasNR_Neigh_FDD and pc_SFTD_MeasNR_Neigh_TDD are checked. - But - as per 38306 , sftd-MeasNR-Neigh is not mandatory IE - as well as based on 38331 , sftd-MeasNR-Neigh ENUMERATED {supported} OPTIONAL, - This check will fail the UEs not supporting sftd-MeasNR-Neigh |
| Summary of change | Remove the check of pc_SFTD_MeasNR_Neigh_TDD and pc_SFTD_MeasNR_Neigh_FDD from the function f_NR_Derive_AddXDD_UE_NR_Capabilities |
| TTCN module | NR_AuxiliaryCapCheckFunctions |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_Derive_AddXDD_UE_NR_Capabilities() return template UE_NR_CapabilityAddXDD_Mode { // @sic R5s190471 R5s201302 sic@ var template (present) UE_NR_CapabilityAddXDD_Mode v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode; if (f_NR_DualModeCheck() and (not pc_intraAndInterF_MeasAndReport) and (not pc_eventA_MeasAndReport) and (not pc_handoverInterF) and (not (pc_SFTD_MeasNR_Neigh_FDD or pc_SFTD_MeasNR_Neigh_TDD))) { v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode_Support; } else if ((f_NR_DualModeCheck() and pc_intraAndInterF_MeasAndReport and pc_eventA_MeasAndReport and pc_handoverInterF and (pc_SFTD_MeasNR_Neigh_FDD or pc_SFTD_MeasNR_Neigh_TDD)) or (pc_nrFDD and (not pc_nrTDD)) or ((not pc_nrFDD) and pc_nrTDD)) { v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode_Omit; } else { f_SetVerdict(inconc,__FILE__, __LINE__, "PICS do not match condition for UE-NR-CapabilityAddXDD-Mode") } return v_UE_NR_CapabilityAddXDD_Mode ifpresent; };

After Change:

function f_NR_Derive_AddXDD_UE_NR_Capabilities() return template UE_NR_CapabilityAddXDD_Mode { // @sic R5s190471 R5s201302 sic@ var template (present) UE_NR_CapabilityAddXDD_Mode v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode; if (f_NR_DualModeCheck() and (not pc_intraAndInterF_MeasAndReport) and (not pc_eventA_MeasAndReport) and (not pc_handoverInterF)) //WA#WI=1006243 { v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode_Support; } else if ((f_NR_DualModeCheck() and pc_intraAndInterF_MeasAndReport and pc_eventA_MeasAndReport and pc_handoverInterF ) or (pc_nrFDD and (not pc_nrTDD)) or ((not pc_nrFDD) and pc_nrTDD)) //WA#WI=1006243 { v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode_Omit; } else { f_SetVerdict(inconc,__FILE__, __LINE__, "PICS do not match condition for UE-NR-CapabilityAddXDD-Mode") } return v_UE_NR_CapabilityAddXDD_Mode ifpresent; };