3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221098

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0863 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS Conference call test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the implementation of R5-225421, related to Annex A.4.2 PRACK, changes in ttcn-3 have been done to accommodate the prose CR. These changes are missing to differentiate between a “NORMAL” call and a “CONFERENCE” call, both expecting a different Route header. This can be fixed just adding an input parameter to function f_IMS_PrackRequest_MessageHeaderRX to populate Route header depending on the type of the call placed during test case execution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an input parameter to function f_IMS_PrackRequest_MessageHeaderRX to populate Route header PRACK depending on the type of the call placed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.31.NR5GC, 8.35.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Correction to function f_IMS_MOCallSetup_ReceivePRACK_Send200OK	3

2.2	Correction to function f_IMS_PrackRequest_MessageHeaderRX	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function f_IMS_MOCallSetup_ReceivePRACK_Send200OK

| Function name | f_IMS_MOCallSetup_ReceivePRACK_Send200OK |
| --- | --- |
| Reason for change | With the implementation of R5-225421, related to Annex A.4.2 PRACK, changes in ttcn3 have been done to accommodate the prose CR. These changes are missing to differentiate between a “NORMAL” call and a “CONFERENCE” call, both expecting a different Route header. |
| Summary of change | Added an input parameter to function f_IMS_PrackRequest_MessageHeaderRX to populate Route header PRACK depending on the type of the call placed. |
| TTCN module | Common\IMS\IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment | Accepted; similar change done for f_IMS_MOCallSetup_AnnexC21C25_Steps5_8 |

Before Change:

<<SKIPPED CODE>> function f_IMS_MOCallSetup_ReceivePRACK_Send200OK(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, template (value) MessageHeader p_MessageHeader_PrevReliableResponse, template ContentType p_ContentTypePRACK := omit, template SDP_Message p_SdpTemplatePRACK := omit, IMS_CallType_Type p_CallType := NORMAL_CALL) runs on IMS_PTC return template (omit) SDP_Message { /* @sic R5s201620: p_ContactUrl removed sic@ */ /* @sic R5-211439: Handling of SDP sic@ */ /* @sic R5-213531, R5-214884: p_CallType sic@ */ /* @sic R5-223463: return SDP_Message - SDP offer in PRACK sic@ */ var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) MessageHeader v_MessageHeaderPRACK; var template (present) SipUrl v_ContactUrl; var IMS_DATA_REQ v_IMS_DATA_REQ; var PRACK_Request v_PrackRequest; var template (value) MessageHeader v_MessageHeader200OK; var template (omit) MessageBody v_MessageBody200OK := omit; var template (omit) ContentType v_ContentType200OK := omit; var boolean v_PreconditionsRequired200OK := false; var template (omit) SDP_Message v_SdpOffer; var template (value) SDP_Message v_SdpAnswer; v_ContactUrl := f_MessageHeader_GetContactSipUrl(p_MessageHeader_PrevReliableResponse); /* @sic R5s201620: ContactUrl retrieved from response being sent reliable sic@ */ v_MessageHeaderPRACK := f_IMS_PrackRequest_MessageHeaderRX(p_InviteRequestWithSdp.Invite, p_MessageHeader_PrevReliableResponse, p_ContentTypePRACK); v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Prack_Request(cr_PRACK_Request(v_ContactUrl, v_MessageHeaderPRACK))); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); /* @sic R5s201620: RoutingInfo derived from PRACK but not from INVITE sic@ */ v_PrackRequest := v_IMS_DATA_REQ.Request.Prack; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_MOCallSetup_ReceivePRACK_Send200OK(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, template (value) MessageHeader p_MessageHeader_PrevReliableResponse, template ContentType p_ContentTypePRACK := omit, template SDP_Message p_SdpTemplatePRACK := omit, IMS_CallType_Type p_CallType := NORMAL_CALL) runs on IMS_PTC return template (omit) SDP_Message { /* @sic R5s201620: p_ContactUrl removed sic@ */ /* @sic R5-211439: Handling of SDP sic@ */ /* @sic R5-213531, R5-214884: p_CallType sic@ */ /* @sic R5-223463: return SDP_Message - SDP offer in PRACK sic@ */ var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) MessageHeader v_MessageHeaderPRACK; var template (present) SipUrl v_ContactUrl; var IMS_DATA_REQ v_IMS_DATA_REQ; var PRACK_Request v_PrackRequest; var template (value) MessageHeader v_MessageHeader200OK; var template (omit) MessageBody v_MessageBody200OK := omit; var template (omit) ContentType v_ContentType200OK := omit; var boolean v_PreconditionsRequired200OK := false; var template (omit) SDP_Message v_SdpOffer; var template (value) SDP_Message v_SdpAnswer; v_ContactUrl := f_MessageHeader_GetContactSipUrl(p_MessageHeader_PrevReliableResponse); /* @sic R5s201620: ContactUrl retrieved from response being sent reliable sic@ */ v_MessageHeaderPRACK := f_IMS_PrackRequest_MessageHeaderRX(p_InviteRequestWithSdp.Invite, p_MessageHeader_PrevReliableResponse, p_ContentTypePRACK, p_CallType); v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Prack_Request(cr_PRACK_Request(v_ContactUrl, v_MessageHeaderPRACK))); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); /* @sic R5s201620: RoutingInfo derived from PRACK but not from INVITE sic@ */ v_PrackRequest := v_IMS_DATA_REQ.Request.Prack; <<SKIPPED CODE>>

## Correction to function f_IMS_PrackRequest_MessageHeaderRX

| Function name | f_IMS_PrackRequest_MessageHeaderRX |
| --- | --- |
| Reason for change | With the implementation of R5-225421, related to Annex A.4.2 PRACK, changes in ttcn3 have been done to accommodate the prose CR. These changes are missing to differentiate between a “NORMAL” call and a “CONFERENCE” call, both expecting a different Route header. |
| Summary of change | Added an input parameter to function f_IMS_PrackRequest_MessageHeaderRX to populate Route header PRACK depending on the type of the call placed. |
| TTCN module | Common\IMS\IMS_Procedures_CallControlCommon.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> function f_IMS_PrackRequest_MessageHeaderRX(template (value) INVITE_Request p_InviteRequest, template (value) MessageHeader p_MessageHeader_PrevReliableResponse, template ContentType p_ContentType := *) runs on IMS_PTC return template (present) MessageHeader { /* p_MessageHeader_PrevReliableResponse .. MessageHeader of "the reliable response" (e.g. previous 183 response) */ /* @sic R5-160505: p_A5 := false per default sic@ */ /* @sic R5-174544: SessionId removed sic@ */ /* @sic R5s201657: p_A5 removed sic@ */ var MessageHeader v_MessageHeader_PrevReliableResponse := valueof(p_MessageHeader_PrevReliableResponse); var template (value) MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader; var template (present) MessageHeader v_MessageHeader_Prack := cr_MessageHeader_Dummy; var template (present) RouteBody_List v_RouteBodyList := f_IMS_Request_RouteSet_RouteBodyList_RX(); /* @sic R5-225421 sic@ */ var CSeq v_CSeq := v_MessageHeader_PrevReliableResponse.cSeq; var integer v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogRemote); /* @sic R5s130497 change 3; R5-133188 sic@ */ var integer v_ResponseNumber := v_MessageHeader_PrevReliableResponse.rSeq.responseNum; /* @sic R5s130333 change 5.2 sic@ */ v_MessageHeader_Prack.via := f_IMS_Via_AsInPrevInvite(v_MessageHeader_Invite.via); // @sic R5s120530 change 3.9 sic@ v_MessageHeader_Prack.route := cr_Route(v_RouteBodyList); v_MessageHeader_Prack.toField := f_IMS_RequestInDialog_ToHeaderRX(); /* @sic R5s130497 change 1 - MCC160 implementation; R5s140017 MCC160 implementation sic@ */ v_MessageHeader_Prack.fromField := f_IMS_RequestInDialog_FromHeaderRX(); /* @sic R5s130497 change 1 - MCC160 implementation; R5s140017 MCC160 implementation sic@ */ v_MessageHeader_Prack.callId := v_MessageHeader_Invite.callId; v_MessageHeader_Prack.cSeq := cr_CseqDef(v_CSeqValue, "PRACK"); v_MessageHeader_Prack.maxForwards := cr_MaxForwardsDef; v_MessageHeader_Prack.rAck := cr_RAckDef(v_ResponseNumber, v_CSeq.seqNumber, v_CSeq.method); v_MessageHeader_Prack.contentType := p_ContentType; if (not f_IMS_PTC_SecurityScheme_IsGiba()) { /* @sic R5s170099 sic@ */ v_MessageHeader_Prack.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); } else { v_MessageHeader_Prack.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()) ifpresent; } return v_MessageHeader_Prack; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_PrackRequest_MessageHeaderRX(template (value) INVITE_Request p_InviteRequest, template (value) MessageHeader p_MessageHeader_PrevReliableResponse, template ContentType p_ContentType := *, IMS_CallType_Type p_CallType := NORMAL_CALL ) runs on IMS_PTC return template (present) MessageHeader { /* p_MessageHeader_PrevReliableResponse .. MessageHeader of "the reliable response" (e.g. previous 183 response) */ /* @sic R5-160505: p_A5 := false per default sic@ */ /* @sic R5-174544: SessionId removed sic@ */ /* @sic R5s201657: p_A5 removed sic@ */ var MessageHeader v_MessageHeader_PrevReliableResponse := valueof(p_MessageHeader_PrevReliableResponse); var template (value) MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader; var template (present) MessageHeader v_MessageHeader_Prack := cr_MessageHeader_Dummy; var template (present) RouteBody_List v_RouteBodyList := f_IMS_Request_RouteSet_RouteBodyList_RX(p_CallType); /* @sic R5-225421 sic@ */ var CSeq v_CSeq := v_MessageHeader_PrevReliableResponse.cSeq; var integer v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogRemote); /* @sic R5s130497 change 3; R5-133188 sic@ */ var integer v_ResponseNumber := v_MessageHeader_PrevReliableResponse.rSeq.responseNum; /* @sic R5s130333 change 5.2 sic@ */ v_MessageHeader_Prack.via := f_IMS_Via_AsInPrevInvite(v_MessageHeader_Invite.via); // @sic R5s120530 change 3.9 sic@ v_MessageHeader_Prack.route := cr_Route(v_RouteBodyList); v_MessageHeader_Prack.toField := f_IMS_RequestInDialog_ToHeaderRX(); /* @sic R5s130497 change 1 - MCC160 implementation; R5s140017 MCC160 implementation sic@ */ v_MessageHeader_Prack.fromField := f_IMS_RequestInDialog_FromHeaderRX(); /* @sic R5s130497 change 1 - MCC160 implementation; R5s140017 MCC160 implementation sic@ */ v_MessageHeader_Prack.callId := v_MessageHeader_Invite.callId; v_MessageHeader_Prack.cSeq := cr_CseqDef(v_CSeqValue, "PRACK"); v_MessageHeader_Prack.maxForwards := cr_MaxForwardsDef; v_MessageHeader_Prack.rAck := cr_RAckDef(v_ResponseNumber, v_CSeq.seqNumber, v_CSeq.method); v_MessageHeader_Prack.contentType := p_ContentType; if (not f_IMS_PTC_SecurityScheme_IsGiba()) { /* @sic R5s170099 sic@ */ v_MessageHeader_Prack.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); } else { v_MessageHeader_Prack.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()) ifpresent; } return v_MessageHeader_Prack; } <<SKIPPED CODE>>