3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221112

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4691 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Step 126, the ESM Procedure Transaction Identity is acquired the one assigned to UE, while in 24301 clause 6.3.2 : "If the procedure was triggered by the transport of user data via the control plane, the network shall set the procedure transaction identity value in the message header of the EPS bearer context related request message to "no procedure transaction identity assigned" (see example in figure 6.3.2.5)." Also, in case the UE is run with Attach without PDN (px_DoAttachWithoutPDN = TRUE), then there wont be a PDN Index anf call to f_NBIOT_MobileInfo_GetPTI(v_PDNIndex) will cause a runtime error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the PTI to “no procedure transaction identity assigned” in cs_ESM_DATA_TRANSPORT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation may FAIL a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1. Correction to f_TC_22_5_8_NBIOT()	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## 1. Correction to f_TC_22_5_8_NBIOT()

| Function name | f_TC_22_5_8_NBIOT () |
| --- | --- |
| Reason for change | In Step 126, the ESM Procedure Transaction Identity is acquired the one assigned to UE, while in 24301 clause 6.3.2 : "If the procedure was triggered by the transport of user data via the control plane, the network shall set the procedure transaction identity value in the message header of the EPS bearer context related request message to "no procedure transaction identity assigned" (see example in figure 6.3.2.5)." Also, in case the UE is run with Attach without PDN (px_DoAttachWithoutPDN = TRUE), then there wont be a PDN Index anf call to f_NBIOT_MobileInfo_GetPTI(v_PDNIndex) will cause a runtime error. |
| Summary of change | Change the PTI to “no procedure transaction identity assigned” in cs_ESM_DATA_TRANSPORT |
| TTCN module | NBIOT\ 22_5\ NBIOT_NAS_TrackingArea.ttcn |
| MCC160 Comment | Accepted |

Before Change:

// ==========================TC 22.5.8 ===================================== /* * @desc REFERENCE TS 36.523-1 clause 22.5.8 * @status APPROVED (NBIOT) */ function f_TC_22_5_8_NBIOT() runs on NBIOT_PTC { <<>CODE SKIPPED>> //@siclog "Step 126" siclog@ // The SS transmits one IP packet to the UE SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell50, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, f_NBIOT_MobileInfo_GetPTI(v_PDNIndex), cs_UserDataContainer(v_UserData))))); <<CODE SKIPPED>> }

After Change:

// ==========================TC 22.5.8 ===================================== /* * @desc REFERENCE TS 36.523-1 clause 22.5.8 * @status APPROVED (NBIOT) */ function f_TC_22_5_8_NBIOT() runs on NBIOT_PTC { <<>CODE SKIPPED>> //@siclog "Step 126" siclog@ // The SS transmits one IP packet to the UE SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell50, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData))))); <<CODE SKIPPED>> }