3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221130

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2774 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.5.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN Implementation of the function f_TC_8_1_5_9_1_NR5GC() - The call of function to set the T311 to 10000ms is misplaced. It should have been ahead of the function f_NR_CellConfig_Def(nr_Cell1) to take effect In the current TTCN Implementation of the function f_TC_8_1_5_9_1_NR5GC_TestBody () - At Step 1, SS transmits a UECapabilityEnquiry message to enquire max capability and with v_CapRAT_RequestList set to Nr and Eutra_Nr. In response if UE follows Step 2b1 , UE capability Information is expecting only cr_UE_CapabilityRAT_Container_NR(nr) but not cr_UE_CapabilityRAT_Container_NR(eutra_nr). It is proposed to add cr_UE_CapabilityRAT_Container_NR(eutra_nr) as part of Ue capability Information as UE is requested for both NR and EUTRA_NR capabilities - At step 5 , SS transmits periodic grant for every 10ms – but this 10ms is leading to the stability issues where , by the time cr_ULDedicatedMessageSegment_r16(0) message is processed , 10msec would have been elapsed and another grant would have been sent which lead to the unexpected reception of cr_ULDedicatedMessageSegment_r16(1). Hence it is proposed to increase the periodicity to 25ms to improve the stability |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the function f_TC_8_1_5_9_1_NR5GC() Placed the f_NR_Set_RLF_TimersAndConstants_t311(nr_Cell1) ahead of the function f_NR_CellConfig_Def(nr_Cell1) to take effect. - In the function f_TC_8_1_5_9_1_NR5GC_TestBody () , at Step 2b1 UeCapabilityInformation is modified to expect both NR as well EUTRA_NR container - f_TC_8_1_5_9_1_NR5GC_TestBody (), at step 5 Periodic grant transmission is scheduled for every 25ms instead of 10ms to improve the test case stability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the function f_ f_TC_8_1_5_9_1_NR5GC	3

2.2	Correction to the function f_TC_8_1_5_9_1_NR5GC_TestBody	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_ f_TC_8_1_5_9_1_NR5GC

| Template name | f_TC_8_1_5_9_1_NR5GC |
| --- | --- |
| Reason for change | In the function f_TC_8_1_5_9_1_NR5GC In the current TTCN,The call of function to set the T311 to 10000ms is misplaced. It should have been ahead of the function f_NR_CellConfig_Def(nr_Cell1) to take effect |
| Summary of change | In the function f_TC_8_1_5_9_1_NR5GC Placed the f_NR_Set_RLF_TimersAndConstants_t311(nr_Cell1) ahead of the function f_NR_CellConfig_Def(nr_Cell1) to take effect. |
| TTCN module | RRC_Others_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function f_TC_8_1_5_9_1_NR5GC() runs on NR5GC_PTC {// RACS / UL Message Segment transfer / UECapabilityInformation f_NR5GC_Init(NR_1); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_Set_RLF_TimersAndConstants_t311(nr_Cell1); //@sic R5-220330 sic@ // Preamble: according to TS 38.508-1, clause 4.4A.2 Table 4.4A.2-1 f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); f_TC_8_1_5_9_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } //end of f_TC_8_1_5_9_1_NR5GC

After Change:

function f_TC_8_1_5_9_1_NR5GC() runs on NR5GC_PTC {// RACS / UL Message Segment transfer / UECapabilityInformation f_NR5GC_Init(NR_1); f_NR_Set_RLF_TimersAndConstants_t311(nr_Cell1); //@sic R5-220330 sic@//WA#WI=1008674 // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Preamble: according to TS 38.508-1, clause 4.4A.2 Table 4.4A.2-1 f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); f_TC_8_1_5_9_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } //end of f_TC_8_1_5_9_1_NR5GC

## Correction to the function f_TC_8_1_5_9_1_NR5GC_TestBody

| Template name | f_TC_8_1_5_9_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_TC_8_1_5_9_1_NR5GC_TestBody () - At Step 1, SS transmits a UECapabilityEnquiry message to enquire max capability and with v_CapRAT_RequestList set to Nr and Eutra_Nr. In response if UE follows Step 2b1 , UE capability Information is expecting only cr_UE_CapabilityRAT_Container_NR(nr) but not cr_UE_CapabilityRAT_Container_NR(eutra_nr). It is proposed to add cr_UE_CapabilityRAT_Container_NR(eutra_nr) as part of Ue capability Information as UE is requested for both NR and EUTRA_NR - At step 5 , SS transmits periodic grant for every 10ms – but this 10ms is leading to the stability issues where , by the time cr_ULDedicatedMessageSegment_r16(0) message is processed, 10msec would have been elapsed and another grant would have been sent which lead to the unexpected reception of cr_ULDedicatedMessageSegment_r16(1). Hence it is proposed to increase the periodicity to 25ms to improve the stability |
| Summary of change | - In the function f_TC_8_1_5_9_1_NR5GC_TestBody () , at Step 2b1 UeCapabilityInformation is modified to expect both NR as well EUTRA_NR container - f_TC_8_1_5_9_1_NR5GC_TestBody (), at step 5 Periodic grant transmission is scheduled for every 25ms instead of 10ms to improve the test case stability |
| TTCN module | RRC_Others_NR5GC.ttcn |
| MCC160 Comment | Accepted, conditional to approval of prose CR. |

Before Change:

function f_TC_8_1_5_9_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200933 R5-204456 R5-216276 sic@ var NR_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var NR_SRB_COMMON_IND v_Received_DedicatedMessageSegment; var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15) var template(value) FreqBandList v_FreqBandList := f_NR_Build_FreqBandList_MaxCap(tsc_ListFreqBandIndicator); // Acc. Table 8.1.5.9.1.3.3-3 var octetstring v_EncodeCapRequestNR := bit2oct(encvalue(cs_38508_UECapabilityRequestFilterNR(v_FreqBandList))); var template (value) UE_CapabilityRAT_RequestList v_CapRAT_RequestList:= {cs_UE_CapabilityRAT_Request(nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestNR)}; //@sic R5-223425 sic@ var octetstring v_EncodeUECapabilityEnquiry_v1560_RACS := bit2oct(encvalue(cs_UECapabilityEnquiry_v1560_RACS)); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var SubFrameTiming_Type v_TimingInfoStep4; //Step 4 var SubFrameTiming_Type v_TimingInfoStep5; //Step 5 timer t_Watchdog := 5.0; //@siclog "Step 1" siclog@ // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ // IF UE's encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else step 2b1 will be performed alt { // EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicates rrc-MessageSegmentType-r16 = lastSegment [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(v_MessageSegmentRx))) ) -> value v_Received_DedicatedMessageSegment { if (v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentType_r16 == lastSegment){ // no more segments to receive v_Completed_AllSegments := true; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact last message } else { // notLastSegment received if (v_MessageSegmentRx == 15) { f_NR_SetVerdictFailOrInconc(__FILE__,__LINE__, "Step 2a1: Received a notLastSegment as 16th segment from the UE"); } v_MessageSegmentRx := v_MessageSegmentRx + 1; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact message repeat; } } //Step 2b1: Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UeCapabilityInformation(tsc_NR_RRC_TI_Def, {cr_UE_CapabilityRAT_Container_NR(nr)}))) -> value v_ReceivedAspForUeCapabilityInfo { if (v_MessageSegmentRx>0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE radio access capability information received after a segment"); // VerdictFail case: step 2b1 is mutually exclusive with 2ax } f_NR_8_1_5_9_1_Steps2b2_2b7(); //@sic R5s220330 sic@ } };//end alt // Decode and store the UE-CapabilityRAT-Container if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments if (lengthof(v_Rx_SegmentOct) < tsc_Max_NR_PDCP_SDU_size) { //<9000 @sic R5s220330 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size"); } f_NR_DecodeULDcchMsg(v_Rx_SegmentOct); //@sic R5s220330 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a1"); } //@siclog "Step 3" siclog@ // SS stops regular grant transmission upon scheduling request. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 4" siclog@ // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. v_TimingInfoStep4 := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep4), cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ //@siclog "Step 5" siclog@ //The SS transmits periodic grant from (80*2microsec+1+TDL) slots after step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. v_TimingInfoStep5 := fl_CalculateTiming_8_1_5_9_1(v_TimingInfoStep4); f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep5), valueof (cs_NR_ResourceAllocationDef), 10); //periodic grant every 10ms << SKIPPED CODE >> }

After Change:

function f_TC_8_1_5_9_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200933 R5-204456 R5-216276 sic@ var NR_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var NR_SRB_COMMON_IND v_Received_DedicatedMessageSegment; var octetstring v_Rx_SegmentOct := ''O; var boolean v_Completed_AllSegments := false; var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15) var template(value) FreqBandList v_FreqBandList := f_NR_Build_FreqBandList_MaxCap(tsc_ListFreqBandIndicator); // Acc. Table 8.1.5.9.1.3.3-3 var octetstring v_EncodeCapRequestNR := bit2oct(encvalue(cs_38508_UECapabilityRequestFilterNR(v_FreqBandList))); var template (value) UE_CapabilityRAT_RequestList v_CapRAT_RequestList:= {cs_UE_CapabilityRAT_Request(nr, v_EncodeCapRequestNR), cs_UE_CapabilityRAT_Request(eutra_nr, v_EncodeCapRequestNR)}; //@sic R5-223425 sic@ var octetstring v_EncodeUECapabilityEnquiry_v1560_RACS := bit2oct(encvalue(cs_UECapabilityEnquiry_v1560_RACS)); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var SubFrameTiming_Type v_TimingInfoStep4; //Step 4 var SubFrameTiming_Type v_TimingInfoStep5; //Step 5 timer t_Watchdog := 5.0; //@siclog "Step 1" siclog@ // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ // IF UE's encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else step 2b1 will be performed alt { // EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicates rrc-MessageSegmentType-r16 = lastSegment [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16(v_MessageSegmentRx))) ) -> value v_Received_DedicatedMessageSegment { if (v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentType_r16 == lastSegment){ // no more segments to receive v_Completed_AllSegments := true; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact last message } else { // notLastSegment received if (v_MessageSegmentRx == 15) { f_NR_SetVerdictFailOrInconc(__FILE__,__LINE__, "Step 2a1: Received a notLastSegment as 16th segment from the UE"); } v_MessageSegmentRx := v_MessageSegmentRx + 1; v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16; //compact message repeat; } } //Step 2b1: Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UeCapabilityInformation(tsc_NR_RRC_TI_Def, {cr_UE_CapabilityRAT_Container_NR(nr),cr_UE_CapabilityRAT_Container_NR(eutra_nr)}))) -> value v_ReceivedAspForUeCapabilityInfo //WA#WI=1008674 { if (v_MessageSegmentRx>0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE radio access capability information received after a segment"); // VerdictFail case: step 2b1 is mutually exclusive with 2ax } f_NR_8_1_5_9_1_Steps2b2_2b7(); //@sic R5s220330 sic@ } };//end alt // Decode and store the UE-CapabilityRAT-Container if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments if (lengthof(v_Rx_SegmentOct) < tsc_Max_NR_PDCP_SDU_size) { //<9000 @sic R5s220330 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size"); } f_NR_DecodeULDcchMsg(v_Rx_SegmentOct); //@sic R5s220330 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a1"); } //@siclog "Step 3" siclog@ // SS stops regular grant transmission upon scheduling request. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 4" siclog@ // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. v_TimingInfoStep4 := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep4), cs_38508_UECapabilityEnquiryMultipleEntries(tsc_NR_RRC_TI_Def, v_CapRAT_RequestList, v_EncodeUECapabilityEnquiry_v1560_RACS))); //@sic R5-223425 sic@ //@siclog "Step 5" siclog@ //The SS transmits periodic grant from (80*2microsec+1+TDL) slots after step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. v_TimingInfoStep5 := fl_CalculateTiming_8_1_5_9_1(v_TimingInfoStep4); f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep5), valueof (cs_NR_ResourceAllocationDef), 25); //periodic grant every 25ms //WA#WI=1008674 << SKIPPED CODE >> }