3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221151

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2789 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC RRC test case 8.1.4.3.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 8.1.4.3.1 to the NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC testcase 8.1.4.3.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1  Overview	2

1.1 Verification Test Summary	2

2 Corrections required	3

2.1 Correction to the function f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS	3

2.2 Correction to the function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback	6

2.3 Correction to the function f_NR5GC_DAPS_HO_CheckDataPath_NoReply	8

2.4 Correction to the function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig	10

2.5 Correction to the function fl_TC_8_1_4_3_1And4_TestBody	12

3 Branches executed	15

4 Execution Log Files	15

4.1 Hisilicon Balong 5000 UE	15

5 References	16

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.4.3.1 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Ting Gao

gaoting@starpointcomm.com

## Verification Test Summary

Test Case: 	8.1.4.3.1.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	Starpoint SP9500 Test System

UE used:	Hisilicon Balong 5000 UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS |
| --- | --- |
| Reason for change | According to rach-ConfigGeneric’s description in core Spec, When CFRA is configured, UE shall ignore preambleReceivedTargetPower, preambleTransMax, powerRampingStep, ra-ResponseWindow signaled within rach-ConfigGeneric and use the corresponding values provided in RACH-ConfigCommon. The preambleTransMax in RACH-ConfigCommon should be configured to n200, rather than rach-ConfigGeneric. |
| Summary of change | Set preambleTransMax in RACH-ConfigCommon to n200. |
| TTCN module | NR5GC\NR5GC_Handover.ttcn |
| MCC160 Comment | Accepted, conditional to approval of corresponding prose CR at RAN5#97. In addition to the proposed change, the setting of preambleTransMax to n200 in rach_ConfigDedicated has to be removed to be in line with the prose. See below. |

Before Change:

function f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(NR_CellId_Type p_TargetCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template(value) PDCP_Config p_PDCP_Config := cs_38508_PDCP_Config, boolean p_RBConfig_KeyChange := true ) runs on NR5GC_PTC return template (value) DL_DCCH_Message { var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (value) CellGroupConfig v_CellGroupConfig; //Get relevant info from NR5GC_Global and build RadioBearerConfig to be included in RRCReconfiguration message var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(p_RBConfig_KeyChange, p_PDCP_Config, -, true); // @sic R5-222040 sic@ //Get RLC-Config from NR5GC_Global and set ReestablishRLC according to parameter var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList; var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); if (p_RBConfig_KeyChange == true) { v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(); //Initialise RLC_BearerConfig with no re-establishment @sic R5-222040 sic@ //Configure the first DRB (DRB#n) as DAPS DRB v_RadioBearerConfig.drb_ToAddModList[0].daps_Config_r16 := true_; v_RadioBearerConfig.drb_ToAddModList[0].reestablishPDCP := omit; // @sic R5-222040 sic@ } else {//@sic R5-222043 sic@ v_RLC_BearerToAddModList[0]:= v_ExistingDRBs[0].RLC_Config; v_RLC_BearerToAddModList[0].reestablishRLC := omit; //Configure the first DRB (DRB#n) as DAPS DRB v_RadioBearerConfig.drb_ToAddModList[0].daps_Config_r16 := true_; v_RadioBearerConfig.drb_ToAddModList[0].recoverPDCP := omit; //@sic R5-222043 sic@ v_RLC_BearerToAddModList[0].servedRadioBearer.drb_Identity := tsc_NR_DRB1; } //CellGroupCongfig with condition PCell_change v_CellGroupConfig := f_NR_GetCellGroupConfigHO (p_TargetCellId, v_RLC_BearerToAddModList, p_RNTI_Value, p_RACH_ConfigDedicated); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig)), p_MasterKeyUpdate, omit, omit); v_RRCReconfiguration := cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_RRCReconfiguration_v1530); return v_RRCReconfiguration; }

After Change:

function f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(NR_CellId_Type p_TargetCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template(value) PDCP_Config p_PDCP_Config := cs_38508_PDCP_Config, boolean p_RBConfig_KeyChange := true ) runs on NR5GC_PTC return template (value) DL_DCCH_Message { var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (value) CellGroupConfig v_CellGroupConfig; //Get relevant info from NR5GC_Global and build RadioBearerConfig to be included in RRCReconfiguration message var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(p_RBConfig_KeyChange, p_PDCP_Config, -, true); // @sic R5-222040 sic@ //Get RLC-Config from NR5GC_Global and set ReestablishRLC according to parameter var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList; var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); if (p_RBConfig_KeyChange == true) { v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(); //Initialise RLC_BearerConfig with no re-establishment @sic R5-222040 sic@ //Configure the first DRB (DRB#n) as DAPS DRB v_RadioBearerConfig.drb_ToAddModList[0].daps_Config_r16 := true_; v_RadioBearerConfig.drb_ToAddModList[0].reestablishPDCP := omit; // @sic R5-222040 sic@ } else {//@sic R5-222043 sic@ v_RLC_BearerToAddModList[0]:= v_ExistingDRBs[0].RLC_Config; v_RLC_BearerToAddModList[0].reestablishRLC := omit; //Configure the first DRB (DRB#n) as DAPS DRB v_RadioBearerConfig.drb_ToAddModList[0].daps_Config_r16 := true_; v_RadioBearerConfig.drb_ToAddModList[0].recoverPDCP := omit; //@sic R5-222043 sic@ v_RLC_BearerToAddModList[0].servedRadioBearer.drb_Identity := tsc_NR_DRB1; } //CellGroupCongfig with condition PCell_change v_CellGroupConfig := f_NR_GetCellGroupConfigHO (p_TargetCellId, v_RLC_BearerToAddModList, p_RNTI_Value, p_RACH_ConfigDedicated); v_CellGroupConfig.spCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n200; v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig)), p_MasterKeyUpdate, omit, omit); v_RRCReconfiguration := cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_RRCReconfiguration_v1530); return v_RRCReconfiguration; }

MCC TF160 Additional changes:

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(NR_CellId_Type p_SourceCellId,

NR_CellId_Type p_TargetCellId,

SubFrameTiming_Type p_Timing,

boolean p_RBConfig_KeyChange := true) runs on NR5GC_PTC return template (value) NR_RadioBearerList_Type

{

var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric;

var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList();

var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList);

var template (value) DL_DCCH_Message v_RRCReconfiguration;

var template (omit) MasterKeyUpdate v_MasterKeyUpdate := omit;

var SubFrameTiming_Type v_Timing1;

var SubFrameTiming_Type v_Timing2;

v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5);  // time for sending RRCReconfiguration

v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell

//Acc. Table 8.1.4.3.1.3.3-1

v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id);

if (p_RBConfig_KeyChange == true)  { v_MasterKeyUpdate := cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); }

v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId,

-,

v_MasterKeyUpdate,

cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))),

-,

p_RBConfig_KeyChange);

## Change 2

| Function name | f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback |
| --- | --- |
| Reason for change | In the function, source cell send DRB data to check if UE transmit HARQ feedback after the DAPS handover complete. Once UE send back HARQ feedback, the next action must be sent back DRB data in CloseUETestLoop mode.Hence it is proposed to add DRB data receive procedure to make sure the procedure’s integrity. |
| Summary of change | In the function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback, DRB data receive procedure is added. |
| TTCN module | NR5GC\NR5GC_Handover.ttcn |
| MCC160 Comment | Accepted, conditional to approval of corresponding prose CR at RAN5#97. |

Before Change:

function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(NR_CellId_Type p_CellId, charstring p_StepNo := "") runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var IntegerList_Type v_DrbList; var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 5.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_DrbList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionInfoList[0].SessionId)); // Get list of DRBs for the first PDU session v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DrbList[0]); //Get the QosFlow ID information from the DRB ID // Configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Send one IP packet on the DAPS DRB (first DRB of the first PDU session) DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(p_CellId, v_QosFlowId, {v_IPData})); // Check that the UE transmit a positive HARQ feedback SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_CellId, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, p_StepNo); t_Watchdog.stop; }

After Change:

function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(NR_CellId_Type p_CellId, charstring p_StepNo := "", NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var IntegerList_Type v_DrbList; var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 5.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_DrbList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionInfoList[0].SessionId)); // Get list of DRBs for the first PDU session v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DrbList[0]); //Get the QosFlow ID information from the DRB ID // Configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Send one IP packet on the DAPS DRB (first DRB of the first PDU session) DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(p_CellId, v_QosFlowId, {v_IPData})); // Check that the UE transmit a positive HARQ feedback SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_CellId, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, p_StepNo); DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(p_TargetCellId, v_QosFlowId, {v_IPData})); t_Watchdog.stop; }

## Change 3

| Function name | f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig |
| --- | --- |
| Reason for change | It is proposed to configure v_RadioBearerList[0] instead of v_RadioBearerList[2] . v_RadioBearerList is the exact set of DRB and the only exsisting DRB is DRB2. |
| Summary of change | Step1 Configuration of DRBs is corrected. |
| TTCN module | NR5GC\NR5GC_Handover.ttcn |
| MCC160 Comment | Accepted. In addition, the line “(v_RadioBearerList[2].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId) + 1;” is deleted, as it is not needed. |

Before Change:

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, SubFrameTiming_Type p_Timing, boolean p_RBConfig_KeyChange := true) runs on NR5GC_PTC return template (value) NR_RadioBearerList_Type { var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (omit) MasterKeyUpdate v_MasterKeyUpdate := omit; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell //Acc. Table 8.1.4.3.1.3.3-1 v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); v_RACH_ConfigGeneric.preambleTransMax := n200; // Table 8.1.4.3.1.3.3-4 if (p_RBConfig_KeyChange == true) { v_MasterKeyUpdate := cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, v_MasterKeyUpdate, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))), -, p_RBConfig_KeyChange); // (R5w210214r1 Step 1. Target Cell: Configuration of DRBs) v_RadioBearerList[2].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId := valueof (v_RadioBearerList[2].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId) + 1; //Configure DRB for DAPS f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, v_RadioBearerList, cs_TimingInfo_NR(p_Timing)); // (R5w210214r1 Step 2. Target Cell: Configuration of Target Cell for no RACH response transmission) f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, cs_TimingInfo_NR(p_Timing)); // (R5w210214r1 Step 3. Source Cell: Send RRCReconfiguration) SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_NR(v_Timing1), v_RRCReconfiguration)); ............................omit........................................ }

After Change:

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, SubFrameTiming_Type p_Timing, boolean p_RBConfig_KeyChange := true) runs on NR5GC_PTC return template (value) NR_RadioBearerList_Type { var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (omit) MasterKeyUpdate v_MasterKeyUpdate := omit; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell //Acc. Table 8.1.4.3.1.3.3-1 v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); v_RACH_ConfigGeneric.preambleTransMax := n200; // Table 8.1.4.3.1.3.3-4 if (p_RBConfig_KeyChange == true) { v_MasterKeyUpdate := cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, v_MasterKeyUpdate, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))), -, p_RBConfig_KeyChange); // (R5w210214r1 Step 1. Target Cell: Configuration of DRBs) v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId := valueof f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, v_RadioBearerList, cs_TimingInfo_NR(p_Timing)); // (R5w210214r1 Step 2. Target Cell: Configuration of Target Cell for no RACH response transmission) f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, cs_TimingInfo_NR(p_Timing)); // (R5w210214r1 Step 3. Source Cell: Send RRCReconfiguration) SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_NR(v_Timing1), v_RRCReconfiguration)); ............................omit........................................ }

## Change 4

| Function name | f_NR5GC_DAPS_HO_CheckDataPath_NoReply |
| --- | --- |
| Reason for change | In the function, to check if UE send the IP Packet on DRB#n of the first PDU session in the uplink on NR Cell 1 within the next 1s, the timer and function ALT is used. However the timer start is missed, it would results UE stuck in an infinite loop. |
| Summary of change | t_Watchdog.start is added to avoid infinite loop. |
| TTCN module | NR5GC\NR5GC_Handover.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function f_NR5GC_DAPS_HO_CheckDataPath_NoReply(NR_CellId_Type p_CellId) runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var IntegerList_Type v_DrbList; var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 1.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_DrbList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionInfoList[0].SessionId)); // Get list of DRBs for the first PDU session v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DrbList[0]); //Get the QosFlow ID information from the DRB ID // Send one IP packet on the DAPS DRB (first DRB of the first PDU session) DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(p_CellId, v_QosFlowId, {v_IPData})); // Check that the UE does not loopback the IP packet alt { [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(p_CellId, v_QosFlowId, ?)) { t_Watchdog.stop; f_SetVerdict(fail,__FILE__,__LINE__,"Unexpected Packet is received at Source cell"); } [] t_Watchdog.timeout{}; } }

After Change:

function f_NR5GC_DAPS_HO_CheckDataPath_NoReply(NR_CellId_Type p_CellId) runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var IntegerList_Type v_DrbList; var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 1.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_DrbList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionInfoList[0].SessionId)); // Get list of DRBs for the first PDU session v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DrbList[0]); //Get the QosFlow ID information from the DRB ID // Send one IP packet on the DAPS DRB (first DRB of the first PDU session) DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(p_CellId, v_QosFlowId, {v_IPData})); t_Watchdog.start; // Check that the UE does not loopback the IP packet alt { [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(p_CellId, v_QosFlowId, ?)) { t_Watchdog.stop; f_SetVerdict(fail,__FILE__,__LINE__,"Unexpected Packet is received at Source cell"); } [] t_Watchdog.timeout{}; } }

## Change 5

| Function name | fl_TC_8_1_4_3_1And4_TestBody |
| --- | --- |
| Reason for change | - Cell power level is absence that cell couldn’t turn on normally. - After exchange DRB data in step2, the UL and DL SN count maintained in UE will change into 1 and 1 respectively, hence the SN used for the next DRB loopback in target cell will be 1,but the PDCP count in target cell is still 0, so it is proposed to handover PDCP count from source cell to target cell to make sure target cell’s PDCP count will be conformant with UE. - At steps 8, the PDCP count of target cell is handovered from source cell in step 6-7, DL PDCP count(value: 2) is conformant with UE, but UL PDCP count is not. On the hand of UE, two DRB data loopbacks have been performed, First loopback is on source cell, Second loopback is on target cell, so SN number maintained in UE is 2; On the hand of SS, PDCP count of target cell is handovered from source cell, UE just loopback DRB data on source cell for one time, so the UL PDCP count value maintained in source cell is 1, target cell will also be 1 after handing over. It will result non comformance between target cell and UE. Hence it is proposed to add 1 to target cell’s UL PDCP count value. |
| Summary of change | 1)Cell power list and power set step is added 2)After step 2, Pdcp Count Handover step is provided 3)At steps 8, target cell’s UL PDCP count value should increase by 1 to keep conformance with UE. |
| TTCN module | NR5GC\8_1_4\RRC_DAPS_Handover_NR5GC.ttcn |
| MCC160 Comment | - Accepted but added in f_TC_8_1_4_3_1_NR5GC after the preamble. - Accepted, conditional to approval of corresponding prose CR. This change is only needed if the procedure in Change 2 is added. - Accepted in principle, but implemented slightly different. See below. |

Before Change:

function fl_TC_8_1_4_3_1And4_TestBody(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC { //This function follows the RRC procedure described in R5w210214r1, section 3.2 var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing3; var template (value) NR_RadioBearerList_Type v_RadioBearerList; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); v_Timing := f_NR_GetNextSendOccasion_DL(p_SourceCellId, 700); // 700 ms v_Timing3 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 15); // time for sending IP packets to source cell //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform //DAPS handover to NR Cell 2. DRB#n of the first PDU session is configured as DAPS bearer. v_RadioBearerList := f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(p_SourceCellId, p_TargetCellId, v_Timing); //R5w210214r1, 3.2, Steps 1-8 //@siclog "Step 2" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable // of exchanging IP data on DRB#n of the first PDU session on NR Cell 1? f_NR5GC_DAPS_HO_CheckDataPath(p_SourceCellId, "Step 2", cs_TimingInfo_NR(v_Timing3)); //R5w210214r1, 3.2, Steps 9-10 //@siclog "Steps 4-5" siclog@ f_NR5GC_DAPS_HO_RACHProcedure_RRCReconfigurationComplete(p_TargetCellId, //R5w210214r1, 3.2, Steps 11-14 "Step 5", -, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); //@siclog "Steps 5A-5B" siclog@ f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(p_SourceCellId, "Step 5A"); //@siclog "Steps 6-7" siclog@ f_NR5GC_DAPS_HO_SourceRelease(p_SourceCellId, p_TargetCellId, "Step 7", v_RadioBearerList); //R5w210214r1, 3.2, Steps 15-17 //@siclog "Step 8" siclog@ f_NR5GC_DAPS_HO_CheckDataPath(p_TargetCellId, "Step 8"); //R5w210214r1, 3.2, Step 18 //@siclog "Steps 9-10" siclog@ f_NR5GC_DAPS_HO_CheckDataPath_NoReply(p_SourceCellId); //R5w210214r1, 3.2, Step 18 f_NR5GC_DAPS_HO_SourceRelease_SSConfig(p_SourceCellId); //R5w210214r1, 3.2, Steps 19-20 }

After Change:

function fl_TC_8_1_4_3_1And4_TestBody(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC { //This function follows the RRC procedure described in R5w210214r1, section 3.2 var template (value) NR_CellPowerList_Type v_CellPowerList; var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing3; var template (value) NR_RadioBearerList_Type v_RadioBearerList; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); v_Timing := f_NR_GetNextSendOccasion_DL(p_SourceCellId, 700); // 700 ms v_Timing3 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 15); // time for sending IP packets to source cell v_CellPowerList := {cs_NR_CellPower(p_SourceCellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_TargetCellId, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1,tsc_NR_NonSuitableOffCellSSS_EPRE)}; f_NR_SetCellPowerList (v_CellPowerList); //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform //DAPS handover to NR Cell 2. DRB#n of the first PDU session is configured as DAPS bearer. v_RadioBearerList := f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(p_SourceCellId, p_TargetCellId, v_Timing); //R5w210214r1, 3.2, Steps 1-8 //@siclog "Step 2" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable // of exchanging IP data on DRB#n of the first PDU session on NR Cell 1? f_NR5GC_DAPS_HO_CheckDataPath(p_SourceCellId, "Step 2", cs_TimingInfo_NR(v_Timing3)); //R5w210214r1, 3.2, Steps 9-10 f_NR_SS_PdcpCount_Handover(p_SourceCellId, p_TargetCellId, v_RadioBearerList, -, cs_TimingInfo_Now, false); //@siclog "Steps 4-5" siclog@ f_NR5GC_DAPS_HO_RACHProcedure_RRCReconfigurationComplete(p_TargetCellId, //R5w210214r1, 3.2, Steps 11-14 "Step 5", -, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); //@siclog "Steps 5A-5B" siclog@ f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(p_SourceCellId, "Step 5A",p_TargetCellId); //@siclog "Steps 6-7" siclog@ f_NR5GC_DAPS_HO_SourceRelease(p_SourceCellId, p_TargetCellId, "Step 7", v_RadioBearerList); //R5w210214r1, 3.2, Steps 15-17 var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList := f_NR_GetPdcpCountInfoListIntraNR_HO(p_SourceCellId, v_RadioBearerList, true, false); v_NR_PdcpCountInfoList[0].UL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList[0].UL.Value) + 1), 32); f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList, cs_TimingInfo_Now); //@siclog "Step 8" siclog@ f_NR5GC_DAPS_HO_CheckDataPath(p_TargetCellId, "Step 8"); //R5w210214r1, 3.2, Step 18 //@siclog "Steps 9-10" siclog@ f_NR5GC_DAPS_HO_CheckDataPath_NoReply(p_SourceCellId); //R5w210214r1, 3.2, Step 18 f_NR5GC_DAPS_HO_SourceRelease_SSConfig(p_SourceCellId); //R5w210214r1, 3.2, Steps 19-20 }

MCC160 proposed implementation for change 5-3:

//@siclog "Steps 6-7" siclog@

f_NR5GC_DAPS_HO_SourceRelease(p_SourceCellId, p_TargetCellId, "Step 7", v_RadioBearerList); //R5w210214r1, 3.2, Steps 15-17

//Adjust UL count to consider the data exchange in steps 5A-5C @sic R5s221151 sic@

v_NR_PdcpCountInfoList:=f_NR_SS_PdcpCount_Get(p_SourceCellId);

v_NR_PdcpCountInfoList[0].UL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList[0].UL.Value) + 1), 32);

f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList, cs_TimingInfo_Now);

# Branches executed

The test case was executed on NR band n41 uing NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Hisilicon Balong 5000 UE

The Hisilicon Balong 5000 UE passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_4_3_1_NR5GC_n41_PASS.spm

- PICS/PIXIT parameter file:

PCT_PICS_PIXIT.sppar

# References

| [1] | R5s221151：Addition of NR5GC RRC test case 8.1.4.3.1 in FR1 |
| --- | --- |