3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221178

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4694 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 Sep'22 baseline upgrade for LTE & NB-IoT Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2022-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to develop the Rel-17 test cases in TTCN-3, the current baseline shall be upgraded. RAN5 agreed to move baseline to September 2022 Rel-17 to all RAN5 TTCN-3 test suites. This CR is implementing this decision on 36.523-3 Test Suites. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline moving to the September 2022 Rel-17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 test cases cannot be implemented and verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All LTE & NB-IoT Test Suites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | MCC160Comments> Added clause 4.3.2. |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	3

1	Baseline Moving – 36.331 ASN.1	3

1.1	EUTRA_RRC_ASN1_Definitions	3

1.1.1	Type change of nonCriticalExtension	3

1.1.2	Name changes	4

1.1.3	Fields added with extension marker (ellipsis)	8

1.1.4	Further ASN.1 changes	11

1.2	NBIOT_RRC_ASN1_Definitions	11

1.2.1	Type change of nonCriticalExtension	11

1.2.2	Name changes	12

1.2.3	Fields added with extension marker (ellipsis)	12

1.2.4	Further ASN.1 changes	13

2	Baseline Moving – 24.334	13

3	Changes required to SAE/NAS/PDCP/RLC/MAC messages	14

3.1	Baseline Moving – 24.301	14

3.1.1	Modified Message Definitions	14

3.1.2	Modified Information Elements	15

3.1.3	New Information Elements	16

3.2	Baseline Moving – 36.323 (PDCP)	16

3.3	Baseline Moving – 36.322 (RLC)	16

3.4	Baseline Moving – 36.321 (MAC)	16

3.5	Baseline Moving – 23.038, 23.040, 24.011 (SMS)	17

3.6	Baseline Moving – 36.509 (Loopback)	17

4	Other Changes	17

4.1	Template restrictions	17

4.2	Changes to Capability Checking (Test Case 8.5.4.1)	19

4.2.1	Updated function fl_EUTRA_FullCapabilityCheck to account for Release 17	19

4.2.2	New  function f_EUTRA_MatchEutraCapMsgs_r17 for  Release 17 checking	21

4.2.3	New  templates	21

4.2.4	Updated templates	22

4.3	Further Changes to Capability Checking	25

4.3.1	f_EUTRA_Capability	25

4.4	Removal of non-inclusive terms	26

# Overview

This document lists all the changes needed for baseline moving based on iwd-TTCN3-B2020-09_D22wk37.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Baseline Moving – 36.331 ASN.1

ASN.1 modules is upgraded to 36.331-h20.

## EUTRA_RRC_ASN1_Definitions

### Type change of nonCriticalExtension

| ASN.1 type affected | New nonCriticalExtension type |
| --- | --- |
| LoggedMeasurementConfiguration-v1530-IEs | LoggedMeasurementConfiguration-v1700-IEs |
| Paging-v1610-IEs | Paging-v1700-IEs |
| RRCConnectionReconfiguration-v1610-IEs | RRCConnectionReconfiguration-v1700-IEs |
| RRCConnectionReconfigurationComplete-v1530-IEs | RRCConnectionReconfigurationComplete-v1700-IEs |
| RRCConnectionReestablishmentComplete-v1530-IEs | RRCConnectionReestablishmentComplete-v1710-IEs |
| RRCConnectionRelease-v1610-IEs | RRCConnectionRelease-v1650-IEs |
| RRCConnectionResume-v1610-IEs | RRCConnectionResume-v1700-IEs |
| RRCConnectionResumeComplete-v1610-IEs | RRCConnectionResumeComplete-v1710-IEs |
| RRCConnectionSetupComplete-v1610-IEs | RRCConnectionSetupComplete-v1690-IEs |
| SystemInformationBlockType1-v12j0-IEs | SystemInformationBlockType1-v15g0-IEs |
| SystemInformationBlockType1-v1610-IEs | SystemInformationBlockType1-v1700-IEs |
| UEAssistanceInformation-v1610-IEs | UEAssistanceInformation-v1700-IEs |
| UECapabilityEnquiry-v1610-IEs | UECapabilityEnquiry-v1710-IEs |
| UEInformationRequest-v1530-IEs | UEInformationRequest-v1710-IEs |
| UEInformationResponse-v1610-IEs | UEInformationResponse-v1710-IEs |
| UE-EUTRA-Capability-v1610-IEs | UE-EUTRA-Capability-v1630-IEs |

no changes in TTCN

### Name changes

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| DLDedicatedMessageSegment-r16-IEs | Field lateNonCriticalExtensions renamed to lateNonCriticalExtension | No impact |
| DLInformationTransfer-r15-IEs | Fields dedicatedInfoNAS-r15 dedicatedInfoCDMA2000-1XRTT-r15 dedicatedInfoCDMA2000-HRPD-r15 renamed respectively to dedicatedInfoNAS dedicatedInfoCDMA2000-1XRTT dedicatedInfoCDMA2000-HRPD | No impact |
| SecurityConfigHO-v1530 | Fields intra5GC-r15 fivegc-ToEPC-r15 epc-To5GC-r15 renamed respectively to intra5GC fivegc-ToEPC epc-To5GC | cs_SecurityConfigHO_From5GToEPC |
| RRCConnectionRelease-v1610-IEs | Field releaseIdleMeasConfig Renamed to releaseIdleMeasConfig-r16 | No impact |
| RAN-NotificationAreaInfo-r15 | Fields cellList ran-AreaConfigList renamed respectively to cellList-r15 ran-AreaConfigList-r15 | No impact |
| RRCConnectionRequest-5GC-r15-IEs | Fields ue-Identity establishmentCause Renamed respectively to ue-Identity-r15 establishmentCause-r15 | No impact |
| InitialUE-Identity-5GC | InitialUE-Identity-5GC-r15 | No impact |
| EstablishmentCause-5GC | EstablishmentCause-5GC-r15 | No impact |
| RRCConnectionResume-v1610-IEs | Fields restoreMCG-SCells restoreSCG nr-SecondaryCellGroupConfig renamed respectively to restoreMCG-SCells-r16 restoreSCG-r16 nr-SecondaryCellGroupConfig-r16 | No impact |
| RRCConnectionSetupComplete-v1610-IEs | Field iab-NodeIndication renamed to iab-NodeIndication-r16 | No impact |
| RedirectedCarrierInfo-r15-IEs | Fields eutra-r15 geran-r15 utra-FDD-r15 cdma2000-HRPD-r15 cdma2000-1xRTT-r15 utra-TDD-r15 renamed respectively to eutra geran utra-FDD cdma2000-HRPD cdma2000-1xRTT utra-TDD | No impact |
| FailureReportSCG-NR-r15 | Enumerated other-r16 renamed to dummy Enumerated bh-RLF, spare4 renamed respectively to bh-RLF-r16, beamFailure-r17 | No impact |
| SIB-Type-v12j0 | Values spare6, spare5, spare4 Renamed respectively to sibType30-v1700, sibType31-v1700, sibType32-v1700 | No impact |
| RLF-Report-r9 | Fields cellGlobalId-r16 pci-arfcn-r16 nrReconnectCellId-r16 eutraReconnectCellId-r16 renamed respectively to cellGlobalId pci-arfcn nrReconnectCellId eutraReconnectCellId | No impact |
| FlightPathInfoReport-r15 | nonCriticalExtension renamed to dummy | cr_FlightPathInfoReport_OneEntry_Any_r15 |
| SystemInformationBlockType4 | intraFreqBlackCellList renamed to intraFreqExcludedCellList | cs_508_SystemInformationBlockType4_Def |
| IntraFreqBlackCellList | Type renamed to IntraFreqExcludedCellList | cs_SIB4_Neigh_BlackCellList_1Entry renamed to cs_SIB4_Neigh_ExcludedCellList_1Entry EUTRA_ASN1_IntraFreqBlackCellList_Type renamed to EUTRA_ASN1_IntraFreqExcludedCellList_Type |
| maxCellBlack | Constant renamed to maxExcludedCell | No impact |
| InterFreqCarrierFreqInfo | Field interFreqBlackCellList renamed to interFreqExcludedCellList | cs_InterFreqCarrierFreq_Common cs_InterFreqCarrierFreq |
| InterFreqCarrierFreqInfo-r12 | Field interFreqBlackCellList-r12 renamed to interFreqExcludedCellList-r12 | cs_InterFreqCarrierFreqInfo_r12 |
| CarrierFreqNR-v1610 | whiteCellListNR-r16 renamed to allowedCellListNR-r16 | No impact |
| WhiteCellListNR-r16 | AllowedCellListNR-r16 | No impact |
| MeasObjectEUTRA | Fields blackCellsToRemoveList excludedCellsToAddModList whiteCellsToRemoveList-r13 whiteCellsToAddModList-r13 renamed respectively to excludedCellsToRemoveList excludedCellsToAddModList allowedCellsToRemoveList-r13 allowedCellsToAddModList-r13 | cs_MeasObject_measObjectEUTRA_Common cds_MeasObject_measObjectEUTRA_Common_RMTC |
| BlackCellsToAddModList | Type renamed to ExcludedCellsToAddModList | cds_MeasObject_measObjectEUTRA_Common_RMTC cs_ExcludedCellsToAddModList_1Entry |
| BlackCellsToAddMod | Type renamed to ExcludedCellsToAddMod | No impact |
| WhiteCellsToAddModList-r13 | AllowedCellsToAddModList-r13 | No impact |
| WhiteCellsToAddMod-r13 | AllowedCellsToAddMod-r13 | No impact |
| MeasObjectNR-r15 | Fields blackCellsToRemoveList-r15 blackCellsToAddModList-r15 renamed respectively to excludedCellsToRemoveList-r15 excludedCellsToAddModList-r15 | cs_MeasObject_measObjectNR |
| SSB-PositionQCL-CellsToAddNR-r16 | Field ssb-PositionQCL-r16 renamed to ssb-PositionQCL-NR-r16 | No impact |
| ReportConfigEUTRA | Field useWhiteCellList-r13 renamed to useAllowedCellList-r13 | cs_ReportConfig_reportConfigEUTRA cs_ReportConfig_reportConfigEUTRA_Wlan |
| MIMO-UE-ParametersPerTM-v1430 | Enumerated values ffs1, ffs2, ffs3, ffs4 renamed respectively to n1, n2, n4, n8 | No impact |
| BandCombinationParameters-v1610 | Field measGapInfoNR renamed to measGapInfoNR-r16 | No impact |
| BandParameters-v1610 | Field intraFreqMultiUL-TransmissionDAPS-r16 renamed to dummy | No impact |
| MeasParameters-v1310 | Field whiteCellList-r13 renamed to allowedCellList-r13 | f_Derive_MeasParameters_v1310 |
| MeasParameters-v1610 | Fields ca-IdleInactiveMeasurements-r16 endc-IdleInactiveMeasFR1-r16 endc-IdleInactiveMeasFR2-r16 renamed respectively to eutra-IdleInactiveMeasurements-r16 nr-IdleInactiveMeasFR1-r16 nr-IdleInactiveMeasFR2-r16 | No impact |
| InterRAT-BandListNR | Type renamed to InterRAT-BandListNR-r16 | No impact |
| InterRAT-BandInfoNR | Type renamed to InterRAT-BandInfoNR-r16 | No impact |
| SL-Parameters-v1610 | Field v2x-SupportedBandCombinationListNR-r16 renamed to dummy | No impact |
| V2X-SupportedBandCombinationNR-r16 | Type renamed to V2X-SupportedBandCombinationEUTRA-NR-r16 | No impact |
| MBSFN-AreaInfo-r16 | Enumerated spare4 renamed to kHz15-v1710 | No impact |
| maxCellWhiteNR-r16 | Constant renamed to maxCellAllowedNR-r16 | No impact |

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| RLF-Report-r9 | measResultListNR_v1640 measResultListExtNR_r16 | Added as * in cr_RLF_Report_1Entry |
| LogMeasInfo-r10 | measResultListNR-v1640 measResultListExtNR-r16 uncomBarPreMeasResult-r17 | Added as * in cr_LogMeasInfo_r10, cr_UEInformationResponse_Wlan |
| ConnEstFailReport-r11 | measResultListNR-v1640 measResultListExtNR-r16 | Added as * in cr_UEInformationResponse_v1130_IEs |
| MeasResultFreqListNR-r16 | measResultListNR-v1640 measResultListExtNR-r16 | No impact |
| SystemInformationBlockType3 | t-Service-r17 | Added as omit in cs_508_SystemInformationBlockType3_Def cs_508_SystemInformationBlockType3_Common |
| SystemInformationBlockType13-r9 | mbsfn-AreaInfoList-r17 | Added as omit in cs_508_SystemInformationBlockType13_Def |
| SystemInformationBlockType24-r15 | carrierFreqListNR-v1700 carrierFreqListNR-v1720 | Added as omit in cs_508_SystemInformationBlockType24_Def |
| SystemInformationBlockType25-r15 | uac-AC1-SelectAssistInfo-r16 uac-BarringInfoSetList-v1700 | No impact |
| MAC-MainConfig | offsetThresholdTA-r17 sr-ProhibitTimerOffset-r17 | Added as omit in cs_MAC_MainConfig_Common, cs_MAC_MainConfig_SCell_AddMod |
| PDCP-Config | discardTimerExt-r17 | Added as omit in cs_508_PDCP_Config_DRB_UM, cs_508_PDCP_Config_DRB_AM, cs_DRB_ConfigAM_PDCP_Mode_UDC |
| PhysicalConfigDedicated | pdsch-ConfigDedicated-v1700 ntn-ConfigDedicated-r17 uplinkSegmentedPrecompensationGap-r17 | Added as omit in cs_PhysicalConfigDedicated_Omit |
| PUR-Config-r16 | pur-PDSCH-maxTBS-r17 | No impact |
| RadioResourceConfigCommonSIB | pcch-Config-v1700 ntn-ConfigCommon-r17 | Added as omit in cs_508_RadioResourceConfigCommonSIB_Default |
| RadioResourceConfigCommon | ntn-ConfigCommon-r17 | Added as omit in cs_RadioResourceConfigCommon_Def, cs_RadioResourceConfigCommon_AllParams |
| SRB-ToAddMod | rlc-Config-v1700 | Added as omit in cs_508_SRB_ToAddMod_DEFAULT cs_508_SRB_ToAddMod_RECONFIG cs_SRB_ToAddMod_ReconfigFromNR cs_508_SRB4_ToAddMod_DEFAULT |
| DRB-ToAddMod | rlc-Config-v1700 | Added as omit in cs_DRB_ToAddMod_AllOmit |
| CondReconfigurationAddMod-r16 | triggerConditionSN-r17 | No impact |
| MobilityControlInfo | daps-Config-r16 | Added as omit in cs_MobilityControlInfo |
| MeasIdleCarrierNR-r16 | subcarrierSpacingSSB-r17 | No impact |
| RS-ConfigSSB-NR-r15 | subcarrierSpacingSSB-r17 ssb-PositionQCL-CommonNR-r17 ssb-PositionQCL-CellsToAddModListNR-r17 ssb-PositionQCL-CellsToRemoveListNR-r17 | Added as omit in cs_MeasObject_measObjectNR |
| RMTC-ConfigNR-r16 | rmtc-BandwidthNR-r17 measDurationNR-r17 refSCS-CP-NR-r17 | No impact |
| MeasResults | uncomBarPreMeasResult-r17 coarseLocationInfo-r17 | Added as * in cr_MeasurementReport_r10 cr_MeasurementReport cr_MeasurementReport_Wlan cr_MeasurementReport_r12 |
| CGI-InfoNR-r15 | plmn-IdentityInfoList-v1710 | No impact |
| ReportConfigEUTRA | includeUncomBarPreMeas-r17 coarseLocationReq-r17 | Added as omit in cs_ReportConfig_reportConfigEUTRA cs_ReportConfig_reportConfigEUTRA_Wlan |
| ReportConfigInterRAT | condReconfigurationTriggerNR-r17 | Added as omit in cs_ReportConfig_reportConfigInterRAT cs_ReportConfig_reportConfigInterRAT_Sftd |
| OtherConfig-r9 | measUncomBarPre-r17 scg-DeactivationPreferenceConfig-r17 | Added as omit in cs_OtherConfig_r9 |
| UE-RadioPagingInfo-r12 | inactiveStatePO-Determination-r17 | Added as * in cr_EutraCapInfo_r12 |

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| MasterInformationBlock | spare field split in 2 new field: - partEARFCN-17 - spare | New field added with 0 in cs_508_MasterInformationBlock_Init Value of the spare field replaced with ‘1’B in f_TC_8_1_2_15_EUTRA. |
| RedirectedCarrierInfo | New choice value added: nr-r17 | No impact |
| SystemInformation-r8-IEs | Added choice values sib30-v1700 sib31-v1700 sib32-v1700 | No impact |
| PosSystemInformation-r15-IEs | Added choice values posSib1-9-v1700 posSib1-10-v1700 | No impact |
| PosSIB-Type-r15 | Added choice values posSibType1-9-v1700, posSibType1-10-v1700 | No impact |
| AccessStratumRelease | Added choice value rel17 | New case added for rel-17 in f_ConvertAccessStratumReleaseToInteger f_EUTRAN_CheckReleaseIndicator |

## NBIOT_RRC_ASN1_Definitions

### Type change of nonCriticalExtension

| ASN.1 type affected | New nonCriticalExtension type |
| --- | --- |
| RRCConnectionReestablishmentComplete-NB-v1610-IEs | RRCConnectionReestablishmentComplete-NB-v1710-IEs |
| RRCConnectionRelease-NB-v1610-IEs | RRCConnectionRelease-NB-v1700-IEs |
| RRCConnectionResumeComplete-NB-v1610-IEs | RRCConnectionResumeComplete-NB-v1710-IEs |
| RRCConnectionSetupComplete-NB-v1610-IEs | RRCConnectionSetupComplete-NB-v1710-IEs |
| RRCEarlyDataComplete-NB-v1590-IEs | RRCEarlyDataComplete-NB-v1700-IEs |
| UE-Capability-NB-v1610-IEs | UE-Capability-NB-v16x0-IEs |
| SystemInformationBlockType1-NB-v1610 | SystemInformationBlockType1-NB-v1700 |

no changes in TTCN

### Name changes

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| SIB-Type-NB-v1530 | Enumerated value spare6, spare5 renamed respectively to sibType31-NB-r17, sibType32-NB-r17 | cs_508_SystemInformationBlockType3_NB_Def |
| SystemInformationBlockType4-NB-r13 | Field intraFreqBlackCellList-r13 renamed to intraFreqExcludedCellList-r13 | cs_508_SystemInformationBlockType4_NB_Def |
| InterFreqCarrierFreqInfo-NB-r13 | Field interFreqBlackCellList-r13 renamed to interFreqExcludedCellList-r13 | cs_NB_InterFreqCarrierFreq |
| InterFreqBlackCellList-NB-r13 | Type renamed to InterFreqExcludedCellList-NB-r13 | No impact |
| maxCellBlack | Constant renamed to maxExcludedCell | No impact |
| PUR-NRSRP-ChangeThreshold-r16 | Type renamed to PUR-NRSRP-ChangeThreshold-NB-r16 | No impact |
| ANR-Carrier-NB-r16 | Field blackCellList-r16 renamed to excludedCellList-r16 | No impact |
| ANR-BlackCellList-NB-r16 | Type renamed to ANR-ExcludedCellList-NB-r16 | No impact |
| UE-Capability-NB-v1610-IEs | Field meas-Parameters-r16 renamed to measParameters-r16 | No impact |
| Meas-Parameters-NB-r16 | Type renamed to MeasParameters-NB-r16 | No impact |
| AccessStratumRelease-NB-r13 | Enumerated value spare4 renamed to rel17 | Added rel17 branch in f_NBIOT_CheckReleaseIndicator f_NBIOT_ConvertAccessStratumReleaseToInteger |

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| SystemInformationBlockType3-NB-r13 | connMeasConfig-r17 t-Service-r17 | Added as omit in cs_508_SystemInformationBlockType3_NB_Def |
| SystemInformationBlockType22-NB-r14 | coverageBasedPagingConfig-r17 | Added as omit in cs_508_SystemInformationBlockType22_NB_Def |
| DL-ConfigCommon-NB-r14 | pcch-Config-r17 | Added as omit in cs_DL_ConfigCommon_NB_r14 cs_508_SystemInformationBlockType22_NB_Def |
| UL-ConfigCommon-NB-r14 | rsrp-ThresholdsPrachInfoList-r16 | Added as omit in cs_UL_ConfigCommon_NB_r14 |
| MAC-MainConfig-NB-r13 | offsetThresholdTA-r17 | Added as omit in cs_MAC_MainConfig_NB_SRB |
| PhysicalConfigDedicated-NB-r13 | ntn-ConfigDedicated-r17 npdsch-ConfigDedicated-v1700 uplinkPowerControlDedicated-v1700 uplinkSegmentedPrecompensationGap-r17 | Added as omit in cs_PhysicalConfigDedicated_NB_DEFAULT |
| PUR-Config-NB-r16 | pur-PhysicalConfig-v1650 pur-PhysicalConfig-v1700 | No impact |
| RadioResourceConfigCommonSIB-NB-r13 | ntn-ConfigCommon-r17 | Added as omit in cs_508_RadioResourceConfigCommonSIB_NB_Default |
| SRB-ToAddMod-NB-r13 | rlc-Config-v1700 | Added as omit in cs_SRB_ToAddMod_NB_DEFAULT cs_SRB_ToAddMod_NB_RLC_Config |
| DRB-ToAddMod-NB-r13 | rlc-Config-v1700 | Added as omit in cs_NB_DRB_ToAddMod_Common |
| SchedulingRequestConfig-NB-r15 | sr-WithoutHARQ-ACK-Config-v1700 | Added as omit in cs_SchedulingRequestConfig_NB_r15_def |

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| MasterInformationBlock-NB | spare field split in 2 new field: - partEARFCN-17 - spare | New field added with 0 in cs_508_MasterInformationBlock_Init cs_508_MasterInformationBlock_NB_Init |
| SystemInformation-NB-r13-IEs | Choice values added sib27-v1610 sib31-v1700 sib32-v1700 | No impact |

# Baseline Moving – 24.334

24.334 is upgraded to 24.334-h20.

No change is required in TTCN.

# Changes required to SAE/NAS/PDCP/RLC/MAC messages

## Baseline Moving – 24.301

24.301 is upgraded to 24.301-h80.

### Modified Message Definitions

| ASN.1 Definition | Changes Required | Ref: 3GPP 24.301 clause |
| --- | --- | --- |
| Attach Accept | New optional fields: Negotiated IMSI offset, Forbidden TAI(s) for the list of "forbidden tracking areas for roaming", Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | 8.2.1 |
| Attach Reject | New optional fields: Lower bound timer value, Forbidden TAI(s) for the list of "forbidden tracking areas for roaming", Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | 8.2.3 |
| Attach Request | New optional field: Requested IMSI offset | 8.2.4 |
| Detach Request | New optional fields: Lower bound timer value, Forbidden TAI(s) for the list of "forbidden tracking areas for roaming", Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | 8.2.11 |
| Extended Service Request | New optional fields: UE Request Type, Paging Restriction | 8.2.15 |
| Service Reject | New optional fields: Lower bound timer value, Forbidden TAI(s) for the list of "forbidden tracking areas for roaming", Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | 8.2.24 |
| TAU Accept | New optional fields: Negotiated IMSI offset, EPS Additional Request Result, Forbidden TAI(s) for the list of "forbidden tracking areas for roaming", Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | 8.2.26 |
| TAU Reject | New optional fields: Lower bound timer value, Forbidden TAI(s) for the list of "forbidden tracking areas for roaming", Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | 8.2.28 |
| TAU Request | New optional fields: Requested IMSI offset, UE Request Type, Paging Restriction | 8.2.29 |
| Control Plane Service Request | New optional fields: UE Request Type, Paging Restriction | 8.2.33 |
| Service Accept | New optional field: EPS Additional Request Result | 8.2.3.34 |

### Modified Information Elements

| ASN.1 Definition | Changes Required | Ref: 3GPP 24.301 clause |
| --- | --- | --- |
| EPS Network Feature Support | New optional fields: Spare, Paging Timing Collision Control, Paging Restriction, Reject Paging Request, Paging Indication for voice services, NAS Signalling Connection Release | 9.9.3.12A |
| UE Network Capability | New optional fields: Paging Timing Collision Control, Paging Restriction, Reject Paging Request, Paging Indication for voice services, NAS Signalling Connection Release | 9.9.3.34 |
| N1 UE Network Capability | New mandatory field: Ethernet Header Compression for Control Plane CIoT Removed field: Spare | 9.9.3.57 |

### New Information Elements

| ASN.1 Definition | Ref: 3GPP 24.301 clause |
| --- | --- |
| IMSI Offset | 9.9.3.64 |
| UE Request Type | 9.9.3.65 |
| Paging Restriction | 9.9.3.66 |
| EPS Additional Request Result | 9.9.3.67 |

## Baseline Moving – 36.323 (PDCP)

36.323 is upgraded to 36.323-h10.

No changes required in TTCN.

## Baseline Moving – 36.322 (RLC)

36.322 is upgraded to 36.322-h00.

No changes required in TTCN.

## Baseline Moving – 36.321 (MAC)

36.321 is upgraded to 36.321-h20.

No changes required in TTCN.

## Baseline Moving – 23.038, 23.040, 24.011 (SMS)

23.038 is upgraded to 23.038-h00.

23.040 is upgraded to 23.040-h20.

24.011 is upgraded to 24.011-h20

No changes required in TTCN.

## Baseline Moving – 36.509 (Loopback)

36.509 is upgraded to 36.509-h10.

No changes required in TTCN.

# Other Changes

## Template restrictions

The following templates shall be defined as template(present) as they are used to receive:

- cr_MeasResultListGERAN_r9_1Entry

- cr_LocationInfo_r10_LocationCoordinates

- cr_MeasResultNeighCells_r10

- cr_MeasResultNeighCells_r10_EUTRA_1Entry

- cr_MeasResultNeighCells_r9

- cr_MeasResultNeighCells_r9_EUTRA_1Entry

- cr_MeasResultNeighCells_r11

- cr_RLF_Report_1Entry

- cr_LogMeasReport_r10

- cr_MeasResultList2EUTRA_v9e0_1Entry

- cr_UEInformationResponse_v1130_IEs

- cr_UEInformationResponse_v930_IEs

- cr_MeasResultNeighCells_measResultListEUTRA_1Entry

- cr_MeasResultNeighCells_measResultListEUTRA_2Entries

- cr_Cgi_Info_EUTRA

- cr_AdditionalSI_Info_r9

- cr_MeasResultNeighCells_measResultListUTRA_1Entry_FDD

- cr_MeasResultNeighCells_measResultListEUTRA_1Entry

- cr_MeasResultNeighCells_measResultListEUTRA_2Entries

- cr_MeasResultNeighCells_measResultListUTRA_1Entry_TDD

- cr_Cgi_Info_UTRA

Change example:

template(present) LocationInfo_r10 cr_LocationInfo_r10_LocationCoordinates(template (present) LocationInfo_r10.locationCoordinates_r10 p_LocationCoordinates_r10,

template LocationInfo_r10.horizontalVelocity_r10 p_HorizontalVelocity_r10 := *,

template LocationInfo_r10.gnss_TOD_msec_r10 p_Gnss_TOD_msec_r10 := *) :=

{ /* @status    APPROVED (LTE_A_IRAT, LTE_A_R10_R11) */

locationCoordinates_r10 := p_LocationCoordinates_r10,

horizontalVelocity_r10 := p_HorizontalVelocity_r10,

gnss_TOD_msec_r10 := p_Gnss_TOD_msec_r10,

verticalVelocityInfo_r15 := *           /* @sic R5s180552 BASELINE MOVING 2018 sic@ */

};

## Changes to Capability Checking (Test Case 8.5.4.1)

Capability checking is updated to cope with Rel-17 UEs.

### Updated function fl_EUTRA_FullCapabilityCheck to account for Release 17

function fl_EUTRA_FullCapabilityCheck(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC

{

var UE_EUTRA_Capability v_ReceivedCapabilityMsg;

var template UE_EUTRA_Capability v_EutraCapability;

var template UE_EUTRA_Capability_v9a0_IEs v_Expected_UE_EUTRA_Capability_v9a0_IEs := omit;

// Transmit UE Capability Enquiry message to ask for EUTRA Capability information

SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry(p_RRC_TI,-,-,-,-,enabled))); // @sic R5-217903 sic@

// Receive UE Capability Information message for EUTRA capabilities and decode

// Either a container is received or segments from which the container is to be established (only Rel-16 onwards)

v_ReceivedCapabilityMsg := f_EUTRA_GetCaps(p_CellId, p_RRC_TI);  // @sic R5-217903 sic@

select (v_ReceivedCapabilityMsg.accessStratumRelease) { // @sic R5s120250 sic@

case (rel8) {

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r8);

// Check Rel 8 vs. IEs provided

if (isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension)) { // @sic R5-123750 sic@

// if mixed releases then Rel 9 IEs may be povided

v_EutraCapability.nonCriticalExtension := ?;

}

else {

v_EutraCapability.nonCriticalExtension := omit;

}

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r8(v_ReceivedCapabilityMsg);  // @sic R5-130706, R5s170597 sic@

}

case (rel9) {

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r9);

if (not isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension)) {

// no Rel 9 (UE-EUTRA-Capability-v940-IEs) IEs  and thus no additional Rel 9 FGIs provided

v_EutraCapability.nonCriticalExtension.nonCriticalExtension := omit; // @sic R5s120623 sic@

}

else {

// Rel 9 (UE-EUTRA-Capability-v940-IEs) IEs provided !!

if (isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension)) {

// Rel 9 (UE-EUTRA-Capability-v9a0-IEs) IEs provided !!

v_EutraCapability.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension := ?;

}

else {

v_EutraCapability.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension := omit;

}

if (isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) {

// Rel 10 (UE-EUTRA-Capability-v1020-IEs) IEs may be provided by Rel 9 UE, but not checked

v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := ?;  // @sic R5-150656  sic@

}

else {

v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := omit;

}

}

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r9(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@

}

case (rel10) {

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r10);

if (not isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { // @sic R5s130088 sic@

// no Rel 10 IEs provided although Rel 10 FGIs must be provided (36.331 Annex C.1)

f_SetVerdict(fail, __FILE__, __LINE__, "UE-EUTRA-Capability-v1020-IEs must be provided by Rel 10 UE");  // @sic R5-133612 sic@

}

else {

v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := ?;

}

if (not isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.featureGroupIndRel10_r10)) { // @sic R5s130269 sic@ @sic R5-133612 sic@

f_SetVerdict(fail, __FILE__, __LINE__, "UE-EUTRA-Capability-v1020-IEs FGIs must be provided by Rel 10 UE");

}

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r10(v_ReceivedCapabilityMsg);  // @sic R5-130706, R5s170597 sic@

}

case (rel11) { // @sic R5s130195 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r11); // @sic R5s160666, R5-166324 sic@

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r11(v_ReceivedCapabilityMsg);  // @sic R5-130706, R5s170597 sic@

}

case (rel12) { // @sic R5s150329 - Rel-12 Baseline move sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r12); // @sic R5s160666, R5-166324 sic@

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r12(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@

}

case (rel13) { // @sic R5s160711 BASELINE MOVING 2016 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r13); // @sic R5s160666, R5-166324 sic@

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r13(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@

}

case (rel14) { // @sic R5s170597 BASELINE MOVING 2017 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r14);

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r14(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@

}

case (rel15) { // @sic R5s180552 BASELINE MOVING 2018 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r15);

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r15(v_ReceivedCapabilityMsg); // @sic R5s180552 BASELINE MOVING 2018 sic@

}

case (rel16) { // @sic R5s201386 BASELINE MOVING 2020 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r16);

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r16(v_ReceivedCapabilityMsg); // @sic R5s201386 BASELINE MOVING 2020 sic@

}

case (rel17) { // @sic R5s221178 BASELINE MOVING 2022 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r17);

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r17(v_ReceivedCapabilityMsg); // @sic R5s221178 BASELINE MOVING 2022 sic@

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not testable"); // a UE implementing an unsupported release must not be failed !

}

}

// Match the received UECapabilityInformation with the expected UE_EUTRA_Capability template and, if applicable, the UE_EUTRA_Capability_v9a0_IEs template.

f_EUTRA_UE_Capability_MatchTemplates(v_ReceivedCapabilityMsg, v_EutraCapability, v_Expected_UE_EUTRA_Capability_v9a0_IEs);

} // fl_EUTRA_FullCapabilityCheck

### New  function f_EUTRA_MatchEutraCapMsgs_r17 for  Release 17 checking

function f_EUTRA_MatchEutraCapMsgs_r17(UE_EUTRA_Capability p_ReceivedCapabilityMsg)

{ /* @sic R5s22178 BASELINE MOVING 2022 sic@ */

// to check some Rel 8 + 9 + 10 + 11 + 12 + 13 + 14 + 15 + 16 UE capabilities

fl_EUTRA_CommonChecks_r16(p_ReceivedCapabilityMsg);

// further rel 17 specific checks may be needed

}

### New  templates

- cr_UE_EUTRA_Capability_v1630_IEs

- cr_UE_EUTRA_Capability_Baseline_2022

- cr_UeCapabilitiesRAT_Container_r17

template UE_EUTRA_Capability_v1630_IEs cr_UE_EUTRA_Capability_v1630_IEs :=

{ /* @sic R5s221178 BASELINE MOVING 2022 sic@ */

/* @status    */

rf_Parameters_v1630 := *,

sl_Parameters_v1630 := *,

earlySecurityReactivation_r16 := *,

mac_Parameters_v1630 := ?,

measParameters_v1630 := *,

fdd_Add_UE_EUTRA_Capabilities_v1630 := ?,

tdd_Add_UE_EUTRA_Capabilities_v1630 := ?,

nonCriticalExtension := {

otherParameters_v1650 := *,

nonCriticalExtension := {

irat_ParametersNR_v1660 := ?,

nonCriticalExtension := {

other_Parameters_v1690 := ?,

nonCriticalExtension := {

measParameters_v1700 := *,

ue_BasedNetwPerfMeasParameters_v1700 := *,

phyLayerParameters_v1700 := ?,

ntn_Parameters_r17 := *,

irat_ParametersNR_v1700 := *,

mbms_Parameters_v1700 := ?,

nonCriticalExtension := {

irat_ParametersNR_v1710 := ?,

neighCellSI_AcquisitionParameters_v1710 := *,

sl_Parameters_v1710 := *,

sidelinkRequested_r17 := *,

nonCriticalExtension := {

ntn_Parameters_v1720 := ?,

nonCriticalExtension := *

} ifpresent // UE-EUTRA-Capability-v1720-IEs

} ifpresent // UE-EUTRA-Capability-v1710-IEs

} ifpresent // UE-EUTRA-Capability-v1700-IEs

} ifpresent // UE-EUTRA-Capability-v1690-IEs

} ifpresent // UE-EUTRA-Capability-v1660-IEs

} ifpresent // UE-EUTRA-Capability-v1650-IEs

} ifpresent; // UE-EUTRA-Capability-v1630-IEs

template UE_EUTRA_Capability_v920_IEs cr_UE_EUTRA_Capability_Baseline_2022

/* @status     */

:= cr_UE_EUTRA_Capability_Baseline_2020 (cr_UE_EUTRA_Capability_v1630_IEs); /* @sic R5s221178 sic@ */

template (present) UE_EUTRA_Capability cr_UeCapabilitiesRAT_Container_r17 /* @sic R5s221178 BASELINE MOVING 2022 sic@ */

/* @status     */

:= cr_UE_EUTRA_Capability(?, -, cr_UE_EUTRA_Capability_Baseline_2022);

### Updated templates

- cr_UE_EUTRA_Capability_v1540_IEs

- cr_UeCapabilitiesRAT_Container_r16 using new template cr_UE_EUTRA_Capability_Baseline_2020

- cr_UeCapabilitiesRAT_Container_r15 using new template cr_UE_EUTRA_Capability_Baseline_2018

- cr_UeCapabilitiesRAT_Container_r14 using new template cr_UE_EUTRA_Capability_Baseline_2017

- cr_UeCapabilitiesRAT_Container_r13 using new template cr_UE_EUTRA_Capability_Baseline_2016

Note: new tempates introduced to reduce the levels of nesting

template UE_EUTRA_Capability_v920_IEs cr_UE_EUTRA_Capability_Baseline_2016 (template UE_EUTRA_Capability_v1330_IEs p_UE_EUTRA_Capability_v1330_IEs := *)

/* @status     */

:= cr_UE_EUTRA_Capability_v920_IEs(cr_UE_EUTRA_Capability_v940_IEs(cr_UE_EUTRA_Capability_v1060_IEs(cr_UE_EUTRA_Capability_v1180_IEs (cr_UE_EUTRA_Capability_v1280_IEs(p_UE_EUTRA_Capability_v1330_IEs))))); /* @sic R5s221178 sic@ */

template (present) UE_EUTRA_Capability cr_UeCapabilitiesRAT_Container_r13  /* @sic R5s160711 BASELINE MOVING 2016 sic@ */

/* @status     */

:= cr_UE_EUTRA_Capability(?, -, cr_UE_EUTRA_Capability_Baseline_2016(cr_UE_EUTRA_Capability_v1330_IEs));

template UE_EUTRA_Capability_v920_IEs cr_UE_EUTRA_Capability_Baseline_2017 (template UE_EUTRA_Capability_v1440_IEs p_UE_EUTRA_Capability_v1440_IEs := *)

/* @status     */

:= cr_UE_EUTRA_Capability_Baseline_2016 (cr_UE_EUTRA_Capability_v1330_IEs(p_UE_EUTRA_Capability_v1440_IEs)); /* @sic R5s221178 sic@ */

template (present) UE_EUTRA_Capability cr_UeCapabilitiesRAT_Container_r14 /* @sic R5s170597 BASELINE MOVING 2017 sic@ */

/* @status     */

:= cr_UE_EUTRA_Capability(?, -, cr_UE_EUTRA_Capability_Baseline_2017(cr_UE_EUTRA_Capability_v1440_IEs));

template UE_EUTRA_Capability_v920_IEs cr_UE_EUTRA_Capability_Baseline_2018 (template UE_EUTRA_Capability_v1540_IEs p_UE_EUTRA_Capability_v1540_IEs := *)

/* @status     */

:= cr_UE_EUTRA_Capability_Baseline_2017(cr_UE_EUTRA_Capability_v1440_IEs(p_UE_EUTRA_Capability_v1540_IEs)); /* @sic R5s221178 sic@ */

template (present) UE_EUTRA_Capability cr_UeCapabilitiesRAT_Container_r15 /* @sic R5s180552 BASELINE MOVING 2018 sic@ */

/* @status     */

:= cr_UE_EUTRA_Capability(?, -, cr_UE_EUTRA_Capability_Baseline_2018(cr_UE_EUTRA_Capability_v1540_IEs));

template UE_EUTRA_Capability_v1540_IEs cr_UE_EUTRA_Capability_v1540_IEs (template UE_EUTRA_Capability_v1630_IEs p_UE_EUTRA_Capability_v1630_IEs := *) :=

{ /* @sic R5s201386 BASELINE MOVING 2020 sic@ */

/* @status    APPROVED (LTE) */

phyLayerParameters_v1540 := *,

otherParameters_v1540 := ?,

fdd_Add_UE_EUTRA_Capabilities_v1540 := *,

tdd_Add_UE_EUTRA_Capabilities_v1540 := *,

sl_Parameters_v1540 := *,

irat_ParametersNR_v1540 := *,

nonCriticalExtension := {

neighCellSI_AcquisitionParameters_v1550 := *,

phyLayerParameters_v1550 := ?,

mac_Parameters_v1550 := ?,

fdd_Add_UE_EUTRA_Capabilities_v1550 := ?,

tdd_Add_UE_EUTRA_Capabilities_v1550 := ?,

nonCriticalExtension := {

pdcp_ParametersNR_v1560 := ?,

irat_ParametersNR_v1560 := ?,

appliedCapabilityFilterCommon_r15 := *,

fdd_Add_UE_EUTRA_Capabilities_v1560 := ?,

tdd_Add_UE_EUTRA_Capabilities_v1560 := ?,

nonCriticalExtension := {

rf_Parameters_v1570 := *,

irat_ParametersNR_v1570 := *,

nonCriticalExtension := {

neighCellSI_AcquisitionParameters_v15a0 := ?,

eutra_5GC_Parameters_r15 := *,

fdd_Add_UE_EUTRA_Capabilities_v15a0 := *,

tdd_Add_UE_EUTRA_Capabilities_v15a0 := *,

nonCriticalExtension := {

highSpeedEnhParameters_v1610 := *,

neighCellSI_AcquisitionParameters_v1610 := *,

mbms_Parameters_v1610 := *,

pdcp_Parameters_v1610 := *,

mac_Parameters_v1610 := *,

phyLayerParameters_v1610 := *,

measParameters_v1610 := *,

pur_Parameters_r16 := *,

eutra_5GC_Parameters_v1610 := *,

otherParameters_v1610 := *,

dl_DedicatedMessageSegmentation_r16 := *,

mmtel_Parameters_v1610 := ?,

irat_ParametersNR_v1610 := *,

rf_Parameters_v1610 := *,

mobilityParameters_v1610 := *,

ue_BasedNetwPerfMeasParameters_v1610 := ?,

sl_Parameters_v1610 := *,

fdd_Add_UE_EUTRA_Capabilities_v1610 := *,

tdd_Add_UE_EUTRA_Capabilities_v1610 := *,

nonCriticalExtension := p_UE_EUTRA_Capability_v1630_IEs // @sic R5s221178 BASELINE MOVING 2022 sic@

} ifpresent // UE-EUTRA-Capability-v1610-IEs

} ifpresent // UE-EUTRA-Capability-v15a0-IEs

} ifpresent // UE-EUTRA-Capability-v1570-IEs

} ifpresent // UE-EUTRA-Capability-v1560-IEs

} ifpresent // UE-EUTRA-Capability-v1550-IEs

} ifpresent; // UE-EUTRA-Capability-v1540-IEs

template UE_EUTRA_Capability_v920_IEs cr_UE_EUTRA_Capability_Baseline_2020 (template UE_EUTRA_Capability_v1630_IEs p_UE_EUTRA_Capability_v1630_IEs := *)

/* @status     */

:= cr_UE_EUTRA_Capability_Baseline_2018 (cr_UE_EUTRA_Capability_v1540_IEs(p_UE_EUTRA_Capability_v1630_IEs)); /* @sic R5s221178 sic@ */

template (present) UE_EUTRA_Capability cr_UeCapabilitiesRAT_Container_r16 /* @sic R5s201386 BASELINE MOVING 2020 sic@ */

/* @status     */

:= cr_UE_EUTRA_Capability(?, -, cr_UE_EUTRA_Capability_Baseline_2020(cr_UE_EUTRA_Capability_v1630_IEs));

## Further Changes to Capability Checking

### f_EUTRA_Capability

function f_EUTRA_Capability(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC

{

var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo;

var UE_EUTRA_Capability v_EutraCapability;

SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry(p_RRC_TI)));

SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, cr_UeCapabilityInformation_Any(p_RRC_TI)))

-> value v_ReceivedAspForUeCapabilityInfo;

// Decode and store the ueCapabilitiesRAT-Container

v_EutraCapability := f_EUTRA_DecodeEutraCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.c1.ueCapabilityInformation_r8.ue_CapabilityRAT_ContainerList[0].ueCapabilityRAT_Container);

f_EUTRA_MobileInfo_SetUECapability (v_EutraCapability);

// Check the UE release

// @sic R5-123750 sic@

f_EUTRAN_CheckReleaseIndicator (v_EutraCapability.accessStratumRelease);

select (v_EutraCapability.accessStratumRelease) { // @sic R5-121847 sic@ @sic R5-123750 sic@

case (rel8) {

if (f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 30)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 31 not set to 0");

}

if (f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 31)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 32 not set to 0");

}

}

case (rel9) { // @sic R5-123750 sic@

/* @sic R5s120653 sic@

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 4)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 5 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 5)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 6 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 15)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 16 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 16)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 17 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 17)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 18 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 19)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 20 not set to 1");

}

if (f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 30)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 31 not set to 0");

}

if (f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 31)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 32 not set to 0");

}

*/

}

case (rel10) { // @sic R5s130064 sic@

}

case (rel11) { // @sic R5s130195 sic@

}

case (rel12) {

}

case (rel13) { // @sic R5s160975 sic@

}

case (rel14) { // @sic R5s170597 BASELINE MOVING 2017 sic@

}

case (rel15) {   /* @sic R5s180271 BASELINE MOVING 2018 Phase 1 sic@ */

}

case (rel16) {   /* @sic R5s201386 BASELINE MOVING 2020 sic@ */

}

case (rel17) {   /* @sic R5s221178 BASELINE MOVING 2022 sic@ */

}

case else{

FatalError (__FILE__, __LINE__, "accessStratumRelease not yet considered");

}

}

### NB-IoT Capability Checking Testcase 22.4.13

function f_TC_22_4_13_NBIOT() runs on NBIOT_PTC

{

...

select (v_UECapability.accessStratumRelease_r13) { // @sic R5-181277 sic@

case (rel13) {

//do nothing

}

case (rel14) {

f_NBIOT_MatchCapMsgs_r14(v_UECapabilityInformation_r13); // @sic R5-217957 sic@

}

case (rel15, rel16, rel17) { // @sic R5s180552 BASELINE MOVING 2018, R5s201386 BASELINE MOVING 2020 sic@

f_NBIOT_MatchCapMsgs_r15(v_UECapabilityInformation_r13); // @sic R5-217957 sic@

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not testable"); // a UE implementing an unsupported release must not be failed !

}

}

f_NBIOT_TestBody_Set(false);

//Switch/power off UE

f_NBIOT_Postamble ( nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED );

}

## Removal of non-inclusive terms

Use Allowed CSG list instead of non-inclusive term CSG whitelist

function f_TC_9_2_1_1_18_EUTRA() runs on EUTRA_PTC

{ /* @desc     Attach / Rejected / Not authorized for this CSG */

var CSG_Identity v_CSG_Identity_CellB := '000000000000000000000000010'B;

var template (value) CellPowerList_Type v_CellPowerList;

f_EUTRA_NAS_Init(LTE_AllCellsOnSamePLMN, c2);   // @sic R5-121742 sic@

…

// Add the CSG ID of Cell B in CSG list in USIM

if (pc_Manual_CSG_Selection) { //@sic R5-134565 sic@

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available");

// delete the UE's CSG Identity of Cell B from the Allowed CSG list

f_EUTRA_EmptyACSGL (eutra_CellB, {v_CSG_Identity_CellB}, EPS_Only);  // @sic R5s120102 sic@ @sic R5s120480 sic@ @sic R5s150994 sic@

// Configure Sysinfo for CSG Cell eutra_CellB with CSG Identity v_CSG_Identity_CellB

f_EUTRA_ConfigCSG_SysInfos (eutra_CellB, v_CSG_Identity_CellB);  //@sic R5s140155, R5s140279 sic@

// UE gets EPS attached and registers on cell B using manual CSG selection

f_EUTRA_Manual_CSG_Selection (eutra_CellB, v_CSG_Identity_CellB, EPS_Only); // @sic R5s120013 sic@  @sic R5s120102 sic@ @sic R5s120480 sic@

} else {

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available and UE's Allowed CSG list contains the CSG ID of Cell B"); // @sic R5s221178 sic@

// Configure Sysinfo for CSG Cell eutra_CellB with CSG Identity v_CSG_Identity_CellB

f_EUTRA_ConfigCSG_SysInfos (eutra_CellB, v_CSG_Identity_CellB);  //@sic R5s140155, R5s140279 sic@

}

…

}

function f_TC_9_2_1_2_14_EUTRA() runs on EUTRA_PTC

{ /* Combined attach / Rejected / Not authorized for this CSG */

var CSG_Identity v_CSG_Identity_CellB := '000000000000000000000000010'B;

var template (value) CellPowerList_Type v_CellPowerList;

f_EUTRA_NAS_Init(LTE_CellsOnDifferentPLMN, c7);

…

// Add the CSG ID of Cell B in CSG list in USIM

if (pc_Manual_CSG_Selection) { //@sic R5-134565 sic@

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available");

// delete the UE's CSG Identity of Cell B from the Allowed CSG list

f_EUTRA_EmptyACSGL (eutra_CellB, {v_CSG_Identity_CellB}, Combined);  // @sic R5s120102 sic@ @sic R5s120480 sic@ @sic R5s150995 sic@

// Configure Sysinfo for CSG Cell eutra_CellB with CSG Identity v_CSG_Identity_CellB

f_EUTRA_ConfigCSG_SysInfos (eutra_CellB, v_CSG_Identity_CellB);  //@sic R5s140155, R5s140279 sic@

// UE gets EPS attached and registers on cell B using manual CSG selection

f_EUTRA_Manual_CSG_Selection (eutra_CellB, v_CSG_Identity_CellB, Combined); // @sic R5s120480 sic@ UE registers on cell B using manual CSG selection

} else {

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available and UE's Allowed CSG list contains the CSG ID of Cell B"); // @sic R5s221178 sic@

// Configure Sysinfo for CSG Cell eutra_CellB with CSG Identity v_CSG_Identity_CellB

f_EUTRA_ConfigCSG_SysInfos (eutra_CellB, v_CSG_Identity_CellB);  //@sic R5s140155, R5s140279 sic@

}

…

}

function f_TC_9_2_3_1_20_EUTRA() runs on EUTRA_PTC

{

var template (value) CellPowerList_Type v_CellPowerList;

var CSG_Identity v_CSG_Identity_CellB := '000000000000000000000000010'B;

f_EUTRA_NAS_Init(LTE_AllCellsOnSamePLMN, c1);

…

// Add the CSG ID of Cell B in CSG list in USIM

if (pc_Manual_CSG_Selection) { //@sic R5-134565 sic@

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available");

// delete the UE's CSG Identity of Cell B from the Allowed CSG list

f_EUTRA_EmptyACSGL (eutra_CellB, {v_CSG_Identity_CellB}, EPS_Only);  // @sic R5s120480 sic@ @sic R5s150997 sic@

// Configure Sysinfo for CSG Cell eutra_CellB with CSG Identity v_CSG_Identity_CellB

f_EUTRA_ConfigCSG_SysInfos (eutra_CellB, v_CSG_Identity_CellB);  //@sic R5s140155, R5s140279 sic@

// UE gets EPS attached and registers on cell B using manual CSG selection

f_EUTRA_Manual_CSG_Selection (eutra_CellB, v_CSG_Identity_CellB, EPS_Only); // @sic R5s120480 sic@ UE registers on cell B using manual CSG selection

} else {

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available and UE's Allowed CSG list contains the CSG ID of Cell B"); // @sic R5s221178 sic@

// Configure Sysinfo for CSG Cell eutra_CellB with CSG Identity v_CSG_Identity_CellB

f_EUTRA_ConfigCSG_SysInfos (eutra_CellB, v_CSG_Identity_CellB);  //@sic R5s140155, R5s140279 sic@

}
…

}

function f_TC_9_2_3_2_16_EUTRA() runs on EUTRA_PTC

{ /* Combined tracking area update / Rejected / Not authorized for this CSG */

var template (value) CellPowerList_Type v_CellPowerList;

var CSG_Identity v_CSG_Identity_CellB := '000000000000000000000000010'B;

f_EUTRA_NAS_Init(LTE_AllCellsOnSamePLMN, c3);

…

// Add the CSG ID of Cell B in CSG list in USIM

if (pc_Manual_CSG_Selection) { //@sic R5-134565 sic@ @sic R5s131003 sic@

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available");

// delete the UE's CSG Identity of Cell B from the Allowed CSG list

f_EUTRA_EmptyACSGL (eutra_CellB, {v_CSG_Identity_CellB}, Combined);  // @sic R5s120480 sic@ @sic R5s150998 sic@

// Configure Sysinfo for CSG Cell eutra_CellB with CSG Identity v_CSG_Identity_CellB

f_EUTRA_ConfigCSG_SysInfos (eutra_CellB, v_CSG_Identity_CellB);  //@sic R5s140155, R5s140279 sic@

f_EUTRA_Manual_CSG_Selection (eutra_CellB, v_CSG_Identity_CellB, Combined); // @sic R5s120480 sic@ UE registers on cell B using manual CSG selection

} else {

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available and UE's Allowed CSG list contains the CSG ID of Cell B"); // @sic R5s221178 sic@

// Configure Sysinfo for CSG Cell eutra_CellB with CSG Identity v_CSG_Identity_CellB

f_EUTRA_ConfigCSG_SysInfos (eutra_CellB, v_CSG_Identity_CellB);  //@sic R5s140155, R5s140279 sic@

}

…

}

function f_TC_9_3_1_18_EUTRA() runs on EUTRA_PTC

{

var CSG_Identity v_CSG_Identity_CellA := '000000000000000000000000010'B; //@sic R5s150370 sic@

f_EUTRA_NAS_Init(LTE_AllCellsOnSamePLMN, c3);

…

// Add the CSG ID of Cell A in CSG list in USIM

if (pc_Manual_CSG_Selection) { //@sic R5-134565 sic@

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available");

// delete the UE's CSG Identity of Cell A from the Allowed CSG list

f_EUTRA_EmptyACSGL (eutra_CellA, {v_CSG_Identity_CellA});  // @sic R5s150999 sic@

// Configure Sysinfo for CSG Cell eutra_CellA with CSG Identity v_CSG_Identity_CellA

f_EUTRA_ConfigCSG_SysInfos (eutra_CellA, v_CSG_Identity_CellA);  //@sic R5s140155, R5s140279 sic@

f_EUTRA_Manual_CSG_Selection(eutra_CellA, v_CSG_Identity_CellA); // UE registers on cell A using manual CSG selection

} else {

// Insert a USIM

f_UT_USIM_Insert (UT, "USIM with service n86 available and UE's Allowed CSG list contains the CSG ID of Cell A"); // @sic R5s221178 sic@

// Configure Sysinfo for CSG Cell eutra_CellA with CSG Identity v_CSG_Identity_CellA

f_EUTRA_ConfigCSG_SysInfos (eutra_CellA, v_CSG_Identity_CellA);  //@sic R5s140155, R5s140279 sic@

}

…

}