3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221186

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2808 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC capability check test case 8.1.5.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For a UE supporting both FR1 and FR2, when the IE fr2-Add-UE-NR-Capabilities is included, then the lower level sub IE ss_SINR_Meas, csi_RSRP_AndRSRQ_MeasWithSSB, csi_RSRP_AndRSRQ_MeasWithoutSSB, handoverInterF may not be included, because they have been reported via measAndMobParameters-> measAndMobParametersFRX-Diff Based on Note 3 in Table 8.1.5.1.1.3.3-3 - pc_intraAndInterF_MeasAndReport, pc_eventA_MeasAndReport, pc_handoverInterF are optional and independent capabilities and a UE may support none, any or all of them. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add ifpresent to allow some IEs to be omitted. - Correct some condition checking |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

The below changes are required in addition to the CRs R5s221155, R5s221147, R5s221100 and R5s221089.

## Change 1

| Function name | Template MeasAndMobParametersFRX_Diff |
| --- | --- |
| Reason for change | For a UE supporting both FR1 and FR2, when the IE fr2-Add-UE-NR-Capabilities is included, then the lower level sub IE ss_SINR_Meas, csi_RSRP_AndRSRQ_MeasWithSSB, csi_RSRP_AndRSRQ_MeasWithoutSSB, handoverInterF may not be included, because they have been reported via measAndMobParameters-> measAndMobParametersFRX-Diff Based on Note 3 in Table 8.1.5.1.1.3.3-3 |
| Summary of change | Add ifpresent to allow some IEs to be omitted. |
| TTCN module | NR_AuxiliaryCapCheckFunctions |
| MCC160 Comment | Accepted |

Before Change

template (present) MeasAndMobParametersFRX_Diff cr_NR_MeasAndMobParametersFRX_Diff := { /* @status APPROVED (ENDC, NR5GC) */ ss_SINR_Meas := f_NR_Derive_ss_SINR_Meas(), // @sic R5-225384 sic@ csi_RSRP_AndRSRQ_MeasWithSSB := f_NR_Derive_MeasAndMobParameters_csi_RSRP_AndRSRQ_MeasWithSSB(), csi_RSRP_AndRSRQ_MeasWithoutSSB := f_NR_Derive_MeasAndMobParameters_csi_RSRP_AndRSRQ_MeasWithoutSSB(), // @sic R5-225384 sic@ csi_SINR_Meas := *, csi_RS_RLM := *, handoverInterF := f_NR_Derive_MeasAndMobParametersFRX_handoverInterF(), // @sic R5-225384 sic@ handoverLTE_EPC := *, /* @sic R5s190543 BASELINE MOVING 2019: handoverLTE -> handoverLTE_EPC sic@ */ handoverLTE_5GC := *, /* @sic R5s190543 BASELINE MOVING 2019: handover_eLTE -> handoverLTE_5GC sic@ */ maxNumberResource_CSI_RS_RLM := *, simultaneousRxDataSSB_DiffNumerology := *, /* @sic R5s190543 BASELINE MOVING 2019 sic@ */ nr_AutonomousGaps_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ nr_AutonomousGaps_ENDC_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ nr_AutonomousGaps_NEDC_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ nr_AutonomousGaps_NRDC_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ dummy := *, /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */ cli_RSSI_Meas_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ cli_SRS_RSRP_Meas_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ interFrequencyMeas_NoGap_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */ simultaneousRxDataSSB_DiffNumerology_Inter_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ idleInactiveNR_MeasReport_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ idleInactiveNR_MeasBeamReport_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ increasedNumberofCSIRSPerMO_r16 := * /* @sic R5s210444 BASELINE MOVING 2021 sic@ */ };

After Change

template (present) MeasAndMobParametersFRX_Diff cr_NR_MeasAndMobParametersFRX_Diff := { /* @status APPROVED (ENDC, NR5GC) */ ss_SINR_Meas := f_NR_Derive_ss_SINR_Meas() ifpresent, // @sic R5-225384 sic@ csi_RSRP_AndRSRQ_MeasWithSSB := f_NR_Derive_MeasAndMobParameters_csi_RSRP_AndRSRQ_MeasWithSSB() ifpresent, csi_RSRP_AndRSRQ_MeasWithoutSSB := f_NR_Derive_MeasAndMobParameters_csi_RSRP_AndRSRQ_MeasWithoutSSB() ifpresent, // @sic R5-225384 sic@ csi_SINR_Meas := *, csi_RS_RLM := *, handoverInterF := f_NR_Derive_MeasAndMobParametersFRX_handoverInterF() ifpresent, // @sic R5-225384 sic@ handoverLTE_EPC := *, /* @sic R5s190543 BASELINE MOVING 2019: handoverLTE -> handoverLTE_EPC sic@ */ handoverLTE_5GC := *, /* @sic R5s190543 BASELINE MOVING 2019: handover_eLTE -> handoverLTE_5GC sic@ */ maxNumberResource_CSI_RS_RLM := *, simultaneousRxDataSSB_DiffNumerology := *, /* @sic R5s190543 BASELINE MOVING 2019 sic@ */ nr_AutonomousGaps_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ nr_AutonomousGaps_ENDC_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ nr_AutonomousGaps_NEDC_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ nr_AutonomousGaps_NRDC_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ dummy := *, /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */ cli_RSSI_Meas_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ cli_SRS_RSRP_Meas_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ interFrequencyMeas_NoGap_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 - R5s210444 BASELINE MOVING 2021 sic@ */ simultaneousRxDataSSB_DiffNumerology_Inter_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ idleInactiveNR_MeasReport_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ idleInactiveNR_MeasBeamReport_r16 := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ increasedNumberofCSIRSPerMO_r16 := * /* @sic R5s210444 BASELINE MOVING 2021 sic@ */ };

## Change 2

| Function name | f_NR_Derive_AddXDD_UE_NR_Capabilities() |
| --- | --- |
| Reason for change | pc_intraAndInterF_MeasAndReport, pc_eventA_MeasAndReport, pc_handoverInterF are optional and independent capabilities and a UE may support none, any or all of them. |
| Summary of change | Change some condition checking |
| TTCN module | NR_AuxiliaryCapCheckFunctions |
| MCC160 Comment | Accepted |

Before Change

function f_NR_Derive_AddXDD_UE_NR_Capabilities() return template UE_NR_CapabilityAddXDD_Mode { // @sic R5s190471 R5s201302 sic@ var template (present) UE_NR_CapabilityAddXDD_Mode v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode; if (f_NR_DualModeCheck() and (not pc_intraAndInterF_MeasAndReport) and (not pc_eventA_MeasAndReport) and (not pc_handoverInterF) and (not (pc_SFTD_MeasNR_Neigh_FDD or pc_SFTD_MeasNR_Neigh_TDD))) { v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode_Support; } else if ((f_NR_DualModeCheck() and pc_intraAndInterF_MeasAndReport and pc_eventA_MeasAndReport and pc_handoverInterF and (pc_SFTD_MeasNR_Neigh_FDD or pc_SFTD_MeasNR_Neigh_TDD)) or (pc_nrFDD and (not pc_nrTDD)) or ((not pc_nrFDD) and pc_nrTDD)) { v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode_Omit; } else { f_SetVerdict(inconc,__FILE__, __LINE__, "PICS do not match condition for UE-NR-CapabilityAddXDD-Mode") } return v_UE_NR_CapabilityAddXDD_Mode ifpresent; };

After Change

function f_NR_Derive_AddXDD_UE_NR_Capabilities() return template UE_NR_CapabilityAddXDD_Mode { // @sic R5s190471 R5s201302 sic@ var template (present) UE_NR_CapabilityAddXDD_Mode v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode; if ((f_NR_DualModeCheck() and (not pc_intraAndInterF_MeasAndReport) and (not pc_eventA_MeasAndReport) and (not pc_handoverInterF) and (not (pc_SFTD_MeasNR_Neigh_FDD or pc_SFTD_MeasNR_Neigh_TDD))) or (not f_NR_DualModeCheck())) { v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode_Omit; } else if (f_NR_DualModeCheck() and (pc_intraAndInterF_MeasAndReport or pc_eventA_MeasAndReport or pc_handoverInterF or (pc_SFTD_MeasNR_Neigh_FDD or pc_SFTD_MeasNR_Neigh_TDD))) { v_UE_NR_CapabilityAddXDD_Mode := cr_UE_NR_CapabilityAddXDD_Mode; } else { f_SetVerdict(inconc,__FILE__, __LINE__, "PICS do not match condition for UE-NR-CapabilityAddXDD-Mode") } return v_UE_NR_CapabilityAddXDD_Mode ifpresent; };