3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221190

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0877 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for LTE IMS IRAT test case 19.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Specific message contents in 34.229-1 for step 5b4 of 19.3.2 indicate that PDP context status in ROUTING AREA UPDATE ACCEPT should be set to 0 (all PDP contexts inactive). Current TTCN implementation will set the PDP context status from the ROUTING AREA UPDATE REQUEST which may cause test case failure during postamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Send PDP context status as “inactive” in UTRAN RAU procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_IMS_CSFB_UTRAN_Body() |
| --- | --- |
| Reason for change | Specific message contents in 34.229-1 for step 5b4 of 19.3.2 indicate that PDP context status in ROUTING AREA UPDATE ACCEPT should be set to 0 (all PDP contexts inactive). Current TTCN implementation will set the PDP context status from the ROUTING AREA UPDATE REQUEST which may cause test case failure during postamble. |
| Summary of change | Pass PDP context status “inactive” to f_UTRAN_GMM_RAU() function (see change 2) |
| TTCN module | NR5GC_IMSProcedures |
| MCC160 Comment | Rejected Not supported by prose |

Before Change:

function f_IMS_CSFB_UTRAN_Body (EMERGENCY_CALL_Type p_CallType := normalEmergencyCall, template ServCategory p_ServCategory := cr_ServCategory ifpresent) runs on UTRAN_IRAT_PTC { // 19.1.3, 19.3.2 (and 19.3.2c) now all have the same procedure in UTRAN // 21.x as well //Initialise test case f_IMS_CSFB_UTRAN_Init (); // @sic R5s 160070 sic@ f_UTRAN_TestBody_Set(true); //@siclog "Step 9a1" siclog@ // UE performs cell reselection to a cell supporting CS domain (UTRAN or GERAN cell) and performs emergency call in CS domain together with MM/GMM registration f_UTRAN_EmgCallSetup_RAU(utran_Cell5, p_ServCategory); // @sic R5-184704, R5s140788, R5-145792 sic@ //@siclog "Step 9a2" siclog@ if (p_CallType == eCall) { f_Delay(5.0); } //The SS releases the UE connection & receives release complete message(s). f_UTRAN_CC_Disconnect(cs_TI_MO); // @sic R5s140788 sic@ @sic R5-145792 sic@ f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch, true); f_UTRAN_RRC_ConnEst(utran_Cell5); f_UTRAN_GMM_RAU(utran_Cell5, false); f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch); f_UTRAN_OptionalPDPEstabForIMSReRegistration(utran_Cell5); // @sic R5-182560, R5-183106 sic@ // Switch off/power off and release cells // f_UTRAN_IP_Handling_Change_Configuration(utran_CellDedicated, cell_DCH_64kPS_RAB_SRB); // @sic R5s140788, R5s150736 now done in postamble sic@ f_UTRAN_Postamble(utran_Cell5, U1_IDLE); f_UTRAN_TestBody_Set(false); // send coordination message for end of test case f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger); } }

After Change:

function f_IMS_CSFB_UTRAN_Body (EMERGENCY_CALL_Type p_CallType := normalEmergencyCall, template ServCategory p_ServCategory := cr_ServCategory ifpresent) runs on UTRAN_IRAT_PTC { // 19.1.3, 19.3.2 (and 19.3.2c) now all have the same procedure in UTRAN // 21.x as well //Initialise test case f_IMS_CSFB_UTRAN_Init (); // @sic R5s160070 sic@ f_UTRAN_TestBody_Set(true); //@siclog "Step 9a1" siclog@ // UE performs cell reselection to a cell supporting CS domain (UTRAN or GERAN cell) and performs emergency call in CS domain together with MM/GMM registration f_UTRAN_EmgCallSetup_RAU(utran_Cell5, p_ServCategory); // @sic R5-184704, R5s140788, R5-145792 sic@ //@siclog "Step 9a2" siclog@ if (p_CallType == eCall) { f_Delay(5.0); } //The SS releases the UE connection & receives release complete message(s). f_UTRAN_CC_Disconnect(cs_TI_MO); // @sic R5s140788 sic@ @sic R5-145792 sic@ f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch, true); f_UTRAN_RRC_ConnEst(utran_Cell5); f_UTRAN_GMM_RAU(utran_Cell5, false, -, cs_PDP_ContextStatusInactive); f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch); f_UTRAN_OptionalPDPEstabForIMSReRegistration(utran_Cell5); // @sic R5-182560, R5-183106 sic@ // Switch off/power off and release cells // f_UTRAN_IP_Handling_Change_Configuration(utran_CellDedicated, cell_DCH_64kPS_RAB_SRB); // @sic R5s140788, R5s150736 now done in postamble sic@ f_UTRAN_Postamble(utran_Cell5, U1_IDLE); f_UTRAN_TestBody_Set(false); // send coordination message for end of test case f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger); } }

## Change 2

| Function name | f_UTRAN_GMM_RAU() |
| --- | --- |
| Reason for change | Specific message contents in 34.229-1 for step 5b4 of 19.3.2 indicate that PDP context status in ROUTING AREA UPDATE ACCEPT should be set to 0 (all PDP contexts inactive). Current TTCN implementation will set the PDP context status from the ROUTING AREA UPDATE REQUEST which may cause test case failure during postamble. |
| Summary of change | New parameter to allow PDP context status to be passed into the function |
| TTCN module | UTRAN_IRAT_CommonFunctions.ttcn |
| MCC160 Comment | Rejected Not supported by prose |

Before Change:

function f_UTRAN_GMM_RAU(UTRAN_CellId_Type p_CellId, boolean p_DoAuth := true, boolean p_NoSecurityAfterHO := false) runs on UTRAN_PTC { var START_Value v_START_Value := '00000000000000000000'B; var B3_Type v_UpdateType := '000'B; var RRC_DATA_IND v_RRC_DataInd; var PDP_ContextStatus v_PDP_ContextStatus; var UTRAN_SecurityInfo_Type v_SecurityInfo := f_UTRAN_Security_Get(); var template (present) RRCDataIndType v_RAU := cr_U_RA_UpdReqAny(cr_GMM_UpdateType(?, (tsc_I_UpdateTypeRAOnly, tsc_I_UpdateTypeCombined, tsc_I_UpdateTypeCombinedWithIMSI))); // @sic R5s120776 sic@ alt { [] U_Dc.receive(car_PS_InitDirectTransfer(utran_CellDedicated, tsc_RB3, v_RAU)) // @sic R5s110216 sic@ -> value v_RRC_DataInd {} [] U_Dc.receive(car_PS_UplinkDirectTransfer(utran_CellDedicated, tsc_RB3, v_RAU)) // @sic R5s110216 sic@ -> value v_RRC_DataInd {} } v_SecurityInfo.authKeys.AuthPS.KeySeq := v_RRC_DataInd.msg.routingAreaUpdateRequest.gprsCiphKeySeqNo.keySeq; // @sic R5s140858 sic@ f_UTRAN_Security_Set(v_SecurityInfo); // @sic R5s140858 sic@ if (p_DoAuth) { // @sic R5s120141 sic@ v_START_Value := '00000000000000000000'B; } else if (ispresent(v_RRC_DataInd.ttcn_start)) { v_START_Value := v_RRC_DataInd.ttcn_start; } else { v_START_Value := v_SecurityInfo.start_PS; // current value. If not previously set, default value is all zeros } if (not p_NoSecurityAfterHO) { // @sic R5s110216 sic@ f_UTRAN_SS_SecurityDownloadStart(ps_domain, v_START_Value); } v_UpdateType := f_UG_RauReq_GetUpdateValueForRauAcc(v_RRC_DataInd.msg.routingAreaUpdateRequest); /* @sic R5s120019 Additional Changes sic@ */ v_PDP_ContextStatus := v_RRC_DataInd.msg.routingAreaUpdateRequest.pDP_ContextStatus; f_UTRAN_GMM_RAU_Without_RAUReq(p_CellId, v_START_Value, v_UpdateType, v_PDP_ContextStatus, p_DoAuth, p_NoSecurityAfterHO); // @sic R5-133584 sic@ }

After Change:

function f_UTRAN_GMM_RAU(UTRAN_CellId_Type p_CellId, boolean p_DoAuth := true, boolean p_NoSecurityAfterHO := false, template (omit) PDP_ContextStatus p_PdpContextStatus := omit) runs on UTRAN_PTC { var START_Value v_START_Value := '00000000000000000000'B; var B3_Type v_UpdateType := '000'B; var RRC_DATA_IND v_RRC_DataInd; var PDP_ContextStatus v_PDP_ContextStatus; var UTRAN_SecurityInfo_Type v_SecurityInfo := f_UTRAN_Security_Get(); var template (present) RRCDataIndType v_RAU := cr_U_RA_UpdReqAny(cr_GMM_UpdateType(?, (tsc_I_UpdateTypeRAOnly, tsc_I_UpdateTypeCombined, tsc_I_UpdateTypeCombinedWithIMSI))); // @sic R5s120776 sic@ alt { [] U_Dc.receive(car_PS_InitDirectTransfer(utran_CellDedicated, tsc_RB3, v_RAU)) // @sic R5s110216 sic@ -> value v_RRC_DataInd {} [] U_Dc.receive(car_PS_UplinkDirectTransfer(utran_CellDedicated, tsc_RB3, v_RAU)) // @sic R5s110216 sic@ -> value v_RRC_DataInd {} } v_SecurityInfo.authKeys.AuthPS.KeySeq := v_RRC_DataInd.msg.routingAreaUpdateRequest.gprsCiphKeySeqNo.keySeq; // @sic R5s140858 sic@ f_UTRAN_Security_Set(v_SecurityInfo); // @sic R5s140858 sic@ if (p_DoAuth) { // @sic R5s120141 sic@ v_START_Value := '00000000000000000000'B; } else if (ispresent(v_RRC_DataInd.ttcn_start)) { v_START_Value := v_RRC_DataInd.ttcn_start; } else { v_START_Value := v_SecurityInfo.start_PS; // current value. If not previously set, default value is all zeros } if (not p_NoSecurityAfterHO) { // @sic R5s110216 sic@ f_UTRAN_SS_SecurityDownloadStart(ps_domain, v_START_Value); } v_UpdateType := f_UG_RauReq_GetUpdateValueForRauAcc(v_RRC_DataInd.msg.routingAreaUpdateRequest); /* @sic R5s120019 Additional Changes sic@ */ if (isvalue (p_PdpContextStatus) { v_PDP_ContextStatus := p_PdpContextStatus; } else { v_PDP_ContextStatus := v_RRC_DataInd.msg.routingAreaUpdateRequest.pDP_ContextStatus; } f_UTRAN_GMM_RAU_Without_RAUReq(p_CellId, v_START_Value, v_UpdateType, v_PDP_ContextStatus, p_DoAuth, p_NoSecurityAfterHO); // @sic R5-133584 sic@ }