3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221219

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2824 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.5.10.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, theT346f is started at Step 6 after receiving the ueAssistanceInformation (Step 4-5). According to 38331 T346f start condition “Upon transmitting UEAssistanceInformation message with releasePreference.”. Thus, at UE side, this timer will have started as soon as UE RRC sent ueAssistanceInformation. But in TTCN, this “lead time” is not accounted for. Thus, the testcase fails in Step 9, when the SS side timer is still running, but the UE side timer has finished and the UE transmits urAssistanceInformaiton at Step 9. This needs to be addressed. It is proposed to call t_Wait_T346f.start(f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 30.0)); instead of t_Wait_T346f.start; at Step 6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Called t_Wait_T346f.start(f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 30.0)); instead of t_Wait_T346f.start; at Step 6 Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	4

2.1	Change 1	4

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

- Corrections required

  - Change 1

| Function name | function f_TC_8_1_5_10_1_TestBody () |
| --- | --- |
| Reason for change | According 38331 T346f start condition “Upon transmitting UEAssistanceInformation message with releasePreference.”. Currently T346f starts after step 5 received , which it will cause the timer started later then UE, when UE side T346f expiry and transmit UEAssistanceInfomation, TTCN T346 is running and cause failure. |
| Summary of change | Move the T346 before the step 5 UEAssistanceInformation received |
| TTCN module | NR5GC\8_1_5\RRC_Others_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 6" siclog@ //SS starts timer T346f = 30s. t_Wait_T346f.start; //@siclog "Steps 7-8" siclog@ //The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = 'inactive'. //The UE transmits an SET UAI RESPONSE message. f_NR5GC_SET_UAI(nr_Cell1, '01'B);//@sic R5s211586 sic@ //@siclog "Step 9" siclog@ //Check: Does the UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = 'inactive' within the ReleasePreference-r16 IE?. alt { [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_UEAssistanceInformation(crs_ReleasePreference_r16(inactive)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9"); } []t_Wait_T346f.timeout{}//SS waits for timer T346f expiry. } t_Wait_NoPreferredRRC_State_r16_connected.start(5.0); //@siclog "Steps 11-12" siclog@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 6" siclog@ //SS starts timer T346f = 30s. t_Wait_T346f.start(f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 30.0)); //WA#WI1011367_8.1.5.10.1 //@siclog "Steps 7-8" siclog@ //The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = 'inactive'. //The UE transmits an SET UAI RESPONSE message. f_NR5GC_SET_UAI(nr_Cell1, '01'B);//@sic R5s211586 sic@ //@siclog "Step 9" siclog@ //Check: Does the UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = 'inactive' within the ReleasePreference-r16 IE?. alt { [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_UEAssistanceInformation(crs_ReleasePreference_r16(inactive)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9"); } []t_Wait_T346f.timeout{}//SS waits for timer T346f expiry. } t_Wait_NoPreferredRRC_State_r16_connected.start(5.0); //@siclog "Steps 11-12" siclog@ <<SKIPPED CODE>>