3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221226

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4699 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT MAC testcase 22.3.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, NB_IOTenh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the MAC TC 22.3.1.8, there are two Paging carriers configured: Anchor with pagingWeight = w1 and Non-Anchor with pagingWeight = w1. The implementation by default chooses the Anchor carrier to send the Paging message at Step 1. However, this is not always the case. Pls. consider the example below: Default Paging Cycle (T): 128 UE_ID: 15935 nB: ONE_T pagingWeightAnchor (w[0]): 1 pagingWeight of the non-anchor(w[1]): 1 From TS 36.304, Section 7.1: N: min(T,nB) Ns: max(1,nB/T) W = W[0] + W[1] + ... W[Nn-1] UE_ID: If the UE supports E-UTRA connected to 5GC and NAS indicated to use 5GC for the selected cell: 5G-S-TMSI mod 1024, if P-RNTI is monitored on PDCCH. else IMSI mod 1024, if P-RNTI is monitored on PDCCH. IMSI mod 4096, if P-RNTI is monitored on NPDCCH. IMSI mod 16384, if P-RNTI is monitored on MPDCCH or if P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information W(i): Weight for NB-IoT paging carrier i In the example above, this becomes: N = min(T, T) where T = 128, so N = 128 Ns = max(1, 1), so Ns = 1 W = W[0] + W[1] = 1 + 1 = 2 UE_ID: IMSI mod 16384 = 15935 W(0) is the paging weight for the anchor carrier. W(0) + W(1) is the sum of the anchor weight + non-anchor weight. If P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information, then the paging carrier is determined by the paging carrier with smallest index n (0 ≤ n ≤ Nn-1) fulfilling the following equation: floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n) floor( 15935 / (128 * 1)) mod 2 = 0 < 1 -> Condition fulfilled for Anchor Carrier This would mean that, in this example, the choice would be the Anchor carrier. However, the calculation rests on the UE_ID and choice of anchor or non-anchor is determined by floor(UE_ID/128) being odd or even Hence, a function is required to determine the correct PagingCarrier ID to be used for sending the Paging message. It is also necessary to configure the Non-Anchor carrier related parameters in the local end so that SS stack can send the Paging on the Non-Anchor carrier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined a function to calculate the Paging Carrier ID as well as configured the Non-Anchor carrier related parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to function f_TC_22_3_1_8_NBIOT	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function f_TC_22_3_1_8_NBIOT

| Function name | f_TC_22_3_1_8_NBIOT |
| --- | --- |
| Reason for change | In the MAC TC 22.3.1.8, there are two Paging carriers configured: Anchor with pagingWeight = w1 and Non-Anchor with pagingWeight = w1. The implementation by default chooses the Anchor carrier to send the Paging message at Step 1. However, this is not always the case. Pls. consider the example below: Default Paging Cycle (T): 128 UE_ID: 15935 nB: ONE_T pagingWeightAnchor (w[0]): 1 pagingWeight of the non-anchor(w[1]): 1 From TS 36.304, Section 7.1: N: min(T,nB) Ns: max(1,nB/T) W = W[0] + W[1] + ... W[Nn-1] UE_ID: If the UE supports E-UTRA connected to 5GC and NAS indicated to use 5GC for the selected cell: 5G-S-TMSI mod 1024, if P-RNTI is monitored on PDCCH. else IMSI mod 1024, if P-RNTI is monitored on PDCCH. IMSI mod 4096, if P-RNTI is monitored on NPDCCH. IMSI mod 16384, if P-RNTI is monitored on MPDCCH or if P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information W(i): Weight for NB-IoT paging carrier i In the example above, this becomes: N = min(T, T) where T = 128, so N = 128 Ns = max(1, 1), so Ns = 1 W = W[0] + W[1] = 1 + 1 = 2 UE_ID: IMSI mod 16384 = 15935 W(0) is the paging weight for the anchor carrier. W(0) + W(1) is the sum of the anchor weight + non-anchor weight. If P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information, then the paging carrier is determined by the paging carrier with smallest index n (0 ≤ n ≤ Nn-1) fulfilling the following equation: floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n) floor( 15935 / (128 * 1)) mod 2 = 0 < 1 -> Condition fulfilled for Anchor Carrier This would mean that, in this example, the choice would be the Anchor carrier. However, the calculation rests on the UE_ID and choice of anchor or non-anchor is determined by floor(UE_ID/128) being odd or even Hence, a function is required to determine the correct PagingCarrier ID to be used for sending the Paging message. It is also necessary to configure the Non-Anchor carrier related parameters in the local end so that SS stack can send the Paging on the Non-Anchor carrier. |
| Summary of change | Defined a function to calculate the Paging Carrier ID as well as configured the Non-Anchor carrier related parameters. |
| TTCN module | NBIOT_MAC_Testcases.ttcn |
| MCC160 Comment | Accepted, but will be implemented using alternative function call, see proposal in clause 2.2 |

Before Change:

function f_TC_22_3_1_8_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var template (value) SystemInformationBlockType22_NB_r14 v_SIB22; var template (value) NPRACH_ParametersList_NB_r14 v_NPRACH_ParametersList_NB_r14; var template (value) UL_ConfigCommon_NB_r14 v_UL_ConfigCommon; var template (value) DL_ConfigCommon_NB_r14 v_DL_ConfigCommon; var template (value) NB_NonAnchorCarrierCommonUL_List_Type v_NB_NonAnchorCarrierCommonUL_List; var NB_CarrierFreq_Type v_NB_CarrierFreq; var CarrierFreq_NB_r13 v_CarrierFreqUL; var template (present) NAS_MSG_Indication_Type v_ExpectedNasMsg; var integer v_NonAnchorCarrierId := 1; <<SKIPPED CODE>> f_NBIOT_CellConfig_Def(v_CellId, CONTROL_PLANE); v_NB_NonAnchorCarrierCommonUL_List := { cs_NB_NonAnchorCarrierCommonUL(v_NonAnchorCarrierId, v_UL_ConfigCommon) }; f_NBIOT_SS_ConfigActiveCellInfo(v_CellId, cs_TimingInfo_Now, cds_NB_ActiveCellConfig_CarrierConfigCommonUL_Config(v_NB_NonAnchorCarrierCommonUL_List)); <<SKIPPED CODE>> //@siclog "Step 0B" siclog@ f_Delay(90.0); // @sic R5s190880 sic@ //@siclog "Step 1" siclog@ f_NBIOT_UE_Page(v_CellId); <<SKIPPED CODE>>

After Change:

import from NBIOT_RRC_Paging all; //WA# import from Parameters all; //WA# function f_TC_22_3_1_8_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var template (value) SystemInformationBlockType22_NB_r14 v_SIB22; var template (value) NPRACH_ParametersList_NB_r14 v_NPRACH_ParametersList_NB_r14; var template (value) UL_ConfigCommon_NB_r14 v_UL_ConfigCommon; var template (value) DL_ConfigCommon_NB_r14 v_DL_ConfigCommon; var template (value) NB_NonAnchorCarrierCommonUL_List_Type v_NB_NonAnchorCarrierCommonUL_List; var template (value) NB_NonAnchorCarrierCommonDL_List_Type v_NB_NonAnchorCarrierCommonDL_List; //WA# var NB_CarrierFreq_Type v_NB_CarrierFreq; var CarrierFreq_NB_r13 v_CarrierFreqUL; var template (present) NAS_MSG_Indication_Type v_ExpectedNasMsg; var integer v_NonAnchorCarrierId := 1; var integer v_PagingCarrierID; //WA# var hexstring v_Imsi := px_IMSI_Def; //WA# var PCCH_Config_NB_r13 v_SI_Pcch_Configuration; //WA# <<SKIPPED CODE>> f_NBIOT_CellConfig_Def(v_CellId, CONTROL_PLANE); v_NB_NonAnchorCarrierCommonDL_List := { cs_NB_NonAnchorCarrierCommonDL(v_NonAnchorCarrierId, v_DL_ConfigCommon) }; //WA# v_NB_NonAnchorCarrierCommonUL_List := { cs_NB_NonAnchorCarrierCommonUL(v_NonAnchorCarrierId, v_UL_ConfigCommon) }; f_NBIOT_SS_ConfigActiveCellInfo(v_CellId, cs_TimingInfo_Now, cds_NB_ActiveCellConfig_CarrierConfigCommon_Config(v_NB_NonAnchorCarrierCommonUL_List, v_NB_NonAnchorCarrierCommonDL_List)); //WA# <<SKIPPED CODE>> //@siclog "Step 0B" siclog@ f_Delay(90.0); // @sic R5s190880 sic@ //@siclog "Step 1" siclog@ //WA# v_SI_Pcch_Configuration := f_NBIOT_CellInfo_GetPCCH_ConfigInSYSINFO(v_CellId); v_PagingCarrierID := f_NBIOT_Calculate_PagingID(v_SI_Pcch_Configuration.defaultPagingCycle_r13, v_SI_Pcch_Configuration.nB_r13, v_Imsi, v_SIB22.pagingWeightAnchor_r14, v_DL_ConfigCommon.pcch_Config_r14.pagingWeight_r14); f_NBIOT_UE_Page(nbiot_Cell1, -, v_PagingCarrierID); //WA# Anchor carrier or Non-anchor carrier <<SKIPPED CODE>>

New Function:

| Function name | f_NBIOT_Calculate_PagingID |
| --- | --- |
| Reason for change | In the MAC TC 22.3.1.8, there are two Paging carriers configured: Anchor with pagingWeight = w1 and Non-Anchor with pagingWeight = w1. The implementation by default chooses the Anchor carrier to send the Paging message at Step 1. However, this is not always the case. Pls. consider the example below: Default Paging Cycle (T): 128 UE_ID: 15935 nB: ONE_T pagingWeightAnchor (w[0]): 1 pagingWeight of the non-anchor(w[1]): 1 From TS 36.304, Section 7.1: N: min(T,nB) Ns: max(1,nB/T) W = W[0] + W[1] + ... W[Nn-1] UE_ID: If the UE supports E-UTRA connected to 5GC and NAS indicated to use 5GC for the selected cell: 5G-S-TMSI mod 1024, if P-RNTI is monitored on PDCCH. else IMSI mod 1024, if P-RNTI is monitored on PDCCH. IMSI mod 4096, if P-RNTI is monitored on NPDCCH. IMSI mod 16384, if P-RNTI is monitored on MPDCCH or if P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information W(i): Weight for NB-IoT paging carrier i In the example above, this becomes: N = min(T, T) where T = 128, so N = 128 Ns = max(1, 1), so Ns = 1 W = W[0] + W[1] = 1 + 1 = 2 UE_ID: IMSI mod 16384 = 15935 W(0) is the paging weight for the anchor carrier. W(0) + W(1) is the sum of the anchor weight + non-anchor weight. If P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information, then the paging carrier is determined by the paging carrier with smallest index n (0 ≤ n ≤ Nn-1) fulfilling the following equation: floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n) floor( 15935 / (128 * 1)) mod 2 = 0 < 1 -> Condition fulfilled for Anchor Carrier This would mean that, in this example, the choice would be the Anchor carrier. However, the calculation rests on the UE_ID and choice of anchor or non-anchor is determined by floor(UE_ID/128) being odd or even Hence, a function is required to determine the correct PagingCarrier ID to be used for sending the Paging message. |
| Summary of change | Defined a function to calculate the Paging Carrier ID |
| TTCN module | NBIOT_MAC_Testcases.ttcn |
| MCC160 Comment | Accepted, but will be implemented alternatively as a more generic function, see cl 2.2 |

function f_NBIOT_Calculate_PagingID (NB_DefaultPagingCycle_Type p_T, PCCH_Config_NB_r13.nB_r13 p_Nb, hexstring p_Imsi, template (value) PagingWeight_NB_r14 pagingWeightAnchor_r14, template (value) PagingWeight_NB_r14 pagingWeightNonAnchor_r14) return integer { var integer v_T := 0; var integer v_Nb := 0; var integer v_N := 0;//min(T,nB) var integer v_Ns := 0;//max(1,nB/T) var charstring v_Imsi_String := hex2str(p_Imsi); var integer v_Imsi_Integer := str2int(v_Imsi_String); var integer v_Ue_Id := v_Imsi_Integer mod 16384; var integer v_W; var integer v_W0; var integer v_W1; var integer v_Paging; var integer v_PagingCarrierID; v_T := f_NBIOT_GetPagingCycleValue(p_T); select (p_Nb) { case (fourT) { v_Nb := 4*v_T; } case (twoT) { v_Nb := 2*v_T; } case (oneT) { v_Nb := v_T; } case (halfT) { v_Nb := v_T/2; } case (quarterT) { v_Nb := v_T/4;} case (one8thT) { v_Nb := v_T/8; } case (one16thT) { v_Nb := v_T/16; } case (one32ndT) { v_Nb := v_T/32; } case (one64thT) { v_Nb := v_T/64; } case (one128thT) { v_Nb := v_T/128; } case (one256thT) { v_Nb := v_T/256; } case (one512thT) { v_Nb := v_T/512; } case (one1024thT) { v_Nb := v_T/1024; } case else { FatalError(__FILE__, __LINE__, "Provided value for nB is unrecognizable"); } } select (pagingWeightAnchor_r14) { case (w1) {v_W0 := 1;} case (w2) {v_W0 := 2;} case (w3) {v_W0 := 3;} case (w4) {v_W0 := 4;} case (w5) {v_W0 := 5;} case (w6) {v_W0 := 6;} case (w7) {v_W0 := 7;} case (w8) {v_W0 := 8;} case (w9) {v_W0 := 9;} case (w10) {v_W0 := 10;} case (w11) {v_W0 := 11;} case (w12) {v_W0 := 12;} case (w13) {v_W0 := 13;} case (w14) {v_W0 := 14;} case (w15) {v_W0 := 15;} case (w16) {v_W0 := 16;} } select (pagingWeightNonAnchor_r14) { case (w1) {v_W1 := 1;} case (w2) {v_W1 := 2;} case (w3) {v_W1 := 3;} case (w4) {v_W1 := 4;} case (w5) {v_W1 := 5;} case (w6) {v_W1 := 6;} case (w7) {v_W1 := 7;} case (w8) {v_W1 := 8;} case (w9) {v_W1 := 9;} case (w10) {v_W1 := 10;} case (w11) {v_W1 := 11;} case (w12) {v_W1 := 12;} case (w13) {v_W1 := 13;} case (w14) {v_W1 := 14;} case (w15) {v_W1 := 15;} case (w16) {v_W1 := 16;} } v_W := v_W0+v_W1; if (v_T <= v_Nb) { v_N := v_T } else { v_N := v_Nb } if (1 >= ( v_Nb/v_T)) { v_Ns := 1 } else { v_Ns := v_Nb/v_T } v_Paging := (v_Ue_Id / (v_N * v_Ns)) mod v_W; if (v_Paging < v_W0) { v_PagingCarrierID := 0; } else if (v_Paging < (v_W0 + v_W1)) { v_PagingCarrierID := 1; } return v_PagingCarrierID; }

## MCC 160 alternative implementation

New Function f_NBIOT_Calculate_PagingID:

/* * @desc Generic function to calculate paging ID according to TS 36.304 cl.7.1 * @param p_T * @param p_Nb * @param p_SIB22 * @param p_Imsi * @return integer * @status */ function f_NBIOT_Calculate_PagingID (NB_DefaultPagingCycle_Type p_T, PCCH_Config_NB_r13.nB_r13 p_Nb, template (value) SystemInformationBlockType22_NB_r14 p_SIB22, hexstring p_Imsi := px_IMSI_Def ) return integer { var integer v_T := 0; var integer v_Nb := 0; var integer v_N := 0;//min(T,nB) var integer v_Ns := 0;//max(1,nB/T) var charstring v_Imsi_String := hex2str(p_Imsi); var integer v_Imsi_Integer := str2int(v_Imsi_String); var integer v_Ue_Id := v_Imsi_Integer mod 16384; //if P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information var integer v_W; var integer v_Paging; var integer v_PagingCarrierID; var integer v_CarrierSelectionCriteria := 0; var integer v_NumNonAnchorCarriers := lengthof(p_SIB22.dl_ConfigList_r14); var template (omit) PagingWeight_NB_r14 v_AnchorCarrierWeight; var integer v_ListOfCarrierWeights [maxNonAnchorCarriers_NB_r14]; var integer v_CarrierNum; var integer v_Cnt; if (ispresent(p_SIB22.pagingWeightAnchor_r14)) { v_AnchorCarrierWeight := p_SIB22.pagingWeightAnchor_r14; v_W:= f_NBIOT_Calculate_PagingWeight(v_AnchorCarrierWeight); v_ListOfCarrierWeights[0] := v_W; } else { v_ListOfCarrierWeights[0] := 0; } for (v_Cnt := 1; v_Cnt < v_NumNonAnchorCarriers; v_Cnt := v_Cnt +1) { if(ispresent(p_SIB22.dl_ConfigList_r14[v_Cnt].pcch_Config_r14.pagingWeight_r14)) { v_ListOfCarrierWeights[v_Cnt] := f_NBIOT_Calculate_PagingWeight(p_SIB22.dl_ConfigList_r14[v_Cnt].pcch_Config_r14.pagingWeight_r14); } else { v_ListOfCarrierWeights[v_Cnt] := 0; } v_W := v_W + v_ListOfCarrierWeights[v_Cnt]; } v_T := f_NBIOT_GetPagingCycleValue(p_T); select (p_Nb) { case (fourT) { v_Nb := 4*v_T; } case (twoT) { v_Nb := 2*v_T; } case (oneT) { v_Nb := v_T; } case (halfT) { v_Nb := v_T/2; } case (quarterT) { v_Nb := v_T/4;} case (one8thT) { v_Nb := v_T/8; } case (one16thT) { v_Nb := v_T/16; } case (one32ndT) { v_Nb := v_T/32; } case (one64thT) { v_Nb := v_T/64; } case (one128thT) { v_Nb := v_T/128; } case (one256thT) { v_Nb := v_T/256; } case (one512thT) { v_Nb := v_T/512; } case (one1024thT) { v_Nb := v_T/1024; } case else { FatalError(__FILE__, __LINE__, "Provided value for nB is unrecognizable"); } } if (v_T <= v_Nb) { v_N := v_T } else { v_N := v_Nb } if (1 >= ( v_Nb/v_T)) { v_Ns := 1 } else { v_Ns := v_Nb/v_T } v_Paging := (v_Ue_Id / (v_N * v_Ns)) mod v_W; for(v_CarrierNum := 0; v_CarrierNum < v_NumNonAnchorCarriers +1; v_CarrierNum := v_CarrierNum +1) { v_CarrierSelectionCriteria := v_CarrierSelectionCriteria + v_ListOfCarrierWeights[v_CarrierNum]; if (v_Paging < v_CarrierSelectionCriteria) { v_PagingCarrierID := v_CarrierNum; break; } } return v_PagingCarrierID; }

New Function f_NBIOT_Calculate_PagingWeight:

/* * @desc Generic function to convert given paging weight from enumerated to an inveter value * @param pagingWeight_r14 * @return integer * @status */ function f_NBIOT_Calculate_PagingWeight ( template (value) PagingWeight_NB_r14 p_PagingWeight_r14) return integer { var integer v_W; select (valueof(p_PagingWeight_r14)) { case (w1) {v_W := 1;} case (w2) {v_W := 2;} case (w3) {v_W := 3;} case (w4) {v_W := 4;} case (w5) {v_W := 5;} case (w6) {v_W := 6;} case (w7) {v_W := 7;} case (w8) {v_W := 8;} case (w9) {v_W := 9;} case (w10) {v_W := 10;} case (w11) {v_W := 11;} case (w12) {v_W := 12;} case (w13) {v_W := 13;} case (w14) {v_W := 14;} case (w15) {v_W := 15;} case (w16) {v_W := 16;} } return v_W }

Modified Function f_NBIOT_UE_Page:

/* * @desc Generic function to convert given paging weight from enumerated to an inveter value * @param pagingWeight_r14 * @return integer * @status */ function f_NBIOT_UE_Page(NBIOT_CellId_Type p_CellId, template (omit) PagingRecordList_NB_r13 p_PagingRecordList := omit, template (omit) integer p_PagingCarrierId := omit) runs on NBIOT_PTC { var PCCH_Config_NB_r13 v_SI_Pcch_Configuration; var NB_DefaultPagingCycle_Type v_PagingCycle; var SubFrameTiming_Type v_CurrentTiming; var SubFrameTimingList_Type v_PoList; var hexstring v_Imsi := px_IMSI_Def var GutiParameters_Type v_GutiParameters; var template (value) PagingRecordList_NB_r13 v_PagingRecordList; var template (omit) integer v_PagingCarrierID := p_PagingCarrierId; var NBIOT_CellInfo_Type v_CellInfo; if (isvalue(p_PagingRecordList)) { v_PagingRecordList := valueof(p_PagingRecordList); } else { v_GutiParameters := f_NBIOT_CellInfo_GetGuti(p_CellId); v_PagingRecordList := cs_NB_Paging_OneRecord(cs_PagingUE_Identity_S_TMSI(v_GutiParameters.MME_Code, v_GutiParameters.M_TMSI)); } f_Delay(tsc_NBIOT_DelayForCellSelection); //Derive DRX params from SYS Info v_SI_Pcch_Configuration := f_NBIOT_CellInfo_GetPCCH_ConfigInSYSINFO(p_CellId); //Set the Pagingcycle v_PagingCycle := v_SI_Pcch_Configuration.defaultPagingCycle_r13; //In case paging carrier ID is not given, check if it could be derrived based on SIB22 configuration if (not(ispresent(p_PagingCarrierId))) {//@sic R5s221226 sic@ v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); if(ispresent(v_CellInfo.Sysinfo.BCCH_Info.SIs[1].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[0].sib22_v1430)) { v_PagingCarrierID := f_NBIOT_Calculate_PagingID(v_PagingCycle, v_SI_Pcch_Configuration.nB_r13, f_NBIOT_CellInfo_GetSIB22(p_CellId), v_Imsi ); } } //Get current SFN and subframe v_CurrentTiming := f_NBIOT_GetCurrentTiming(p_CellId); //Calculate next Paging Ocassion v_PoList := f_NBIOT_Calculate_PF_PO(v_CurrentTiming.SFN.Number, v_CurrentTiming.HSFN.Number, v_PagingCycle, v_SI_Pcch_Configuration.nB_r13, v_Imsi); SYS.send(cas_NB_CellConfig_Paging_REQ(p_CellId, v_PoList[0], v_PagingRecordList, v_PagingCarrierID)); /* @sic R5w180306: Support of non-anchor carrier, R5s221226 sic@ */ }