3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221231

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2832 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC test cases with SNPN-only device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The current implementation of f_NR_CellInfo_GetNIDsForAllCells() tries to retrieve PLMN identity for each cell index. For some indices the cell information may never have been intialised in the execution of f_NR_CellArray_Init() due to the required frequency not being defined (e.g. f2 for nr_cell3). - Currently NID input is used for calculating XRES* in SNPN test cases, this should also be used for all test cases in case of SNPN-only UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A check is applied to see ifthe PLMN identity exists before accessing. - Updated the check to include the SNPN_Only case for the legacy 5G test cases to populate the NID value to be used for Authentication calculations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NR5GC test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR_CellInfo_GetNIDsForAllCells |
| --- | --- |
| Reason for change | The current implementation tries to retrieve PLMN identity for each cell index. For some indices the cell information may never have been intialised in the execution of f_NR_CellArray_Init() due to the required frequeny not being defined (e.g. f2 for nr_cell3). |
| Summary of change | A check is applied to see ifthe PLMN identity exists before accessing. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Accepted in principle |

Before Change:

function f_NR_CellInfo_GetNIDsForAllCells() runs on NR_BASE_PTC return SNPN_SubscriberDataList { var integer v_Index; var SNPN_SubscriberDataList v_SNPN_DataList; var integer v_ListIndex := 0; var PLMN_Identity v_ASN_PLMN; // !! Must update this if the number of cells increases for (v_Index :=0; v_Index <= 14; v_Index := v_Index +1) { if (isbound(vc_NR_Global.CellArray[v_Index])) { v_ASN_PLMN := vc_NR_Global.CellArray[v_Index].NAS_Parameters.PLMN_Identity; // NID is 0 for all cells v_SNPN_DataList[v_ListIndex] := {v_ASN_PLMN, 0}; v_ListIndex := v_ListIndex + 1; } } return v_SNPN_DataList; }

After Change:

function f_NR_CellInfo_GetNIDsForAllCells() runs on NR_BASE_PTC return SNPN_SubscriberDataList { var integer v_Index; var SNPN_SubscriberDataList v_SNPN_DataList := {}; var integer v_ListIndex := 0; var PLMN_Identity v_ASN_PLMN; // !! Must update this if the number of cells increases for (v_Index :=0; v_Index <= 14; v_Index := v_Index +1) { if (isbound(vc_NR_Global.CellArray[v_Index])) { if(ispresent(vc_NR_Global.CellArray[v_Index].NAS_Parameters.PLMN_Identity)) { v_ASN_PLMN := vc_NR_Global.CellArray[v_Index].NAS_Parameters.PLMN_Identity; // NID is 0 for all cells v_SNPN_DataList[v_ListIndex] := {v_ASN_PLMN, 0}; v_ListIndex := v_ListIndex + 1; } } } return v_SNPN_DataList; }

## Change 2

| Function name | f_NR_Authentication5G_AKA |
| --- | --- |
| Reason for change | Currently NID input is used for calculating XRES* in SNPN test cases, this should also be used for all test cases in case of SNPN-only UE. |
| Summary of change | Updated the check to include the SNPN_Only case for the legacy 5G test cases to populate the NID value to be used for Authentication calculations |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_Authentication5G_AKA(NR_CellId_Type p_NR_CellId, SecurityHeaderType p_SecurityStatusAuthRequest, template (present) SecurityHeaderType p_SecurityStatusAuthResponse, NAS_PlmnId p_PLMN, NR_RadioBearerId_Type p_SRBId := tsc_NR_RbId_SRB1, integer p_AssignmentMode := 1) runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var NR_SecurityParams_Type v_RRC_SecurityParams := f_NR_Security_Get(); var NID_r16 v_NID_ASN; var template (omit) hexstring v_NID := omit; // build AUTHENTICATION REQUEST msg, input PLMN+NID if SNPN test case if(f_GetTestcaseAttrib_NR_SNPN(testcasename())){ // @sic R5s220753 sic@ v_NID_ASN := f_NR_InitNDI(p_NR_CellId, p_AssignmentMode); // returns a bitstring v_NID := bit2hex(v_NID_ASN); } // send AUTHENTICATION REQUEST v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, p_PLMN, -, v_NID); // @sic R5s220753 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRBId, -, cs_NG_NAS_Request(p_SecurityStatusAuthRequest, v_MsgToSend))); // receive AUTHENTICATION RESPONSE / FAILURE SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRBId, cr_NG_NAS_Ind(p_SecurityStatusAuthResponse))) -> value v_ReceivedAsp; if (not f_Check_5GAKA_AuthenticationResponseFailure(v_NAS_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Authentication Failed"); } // Now NAS parameters updated for Authentication, store updated common values in RRC global variable v_RRC_SecurityParams.KAMF := v_NAS_SecurityParams.KAMF; v_RRC_SecurityParams.KSIamf := v_NAS_SecurityParams.KSIamf; // Now store both f_NR_Security_Set(v_RRC_SecurityParams); f_NR5GC_Security_Set(v_NAS_SecurityParams); }

After Change:

function f_NR_Authentication5G_AKA(NR_CellId_Type p_NR_CellId, SecurityHeaderType p_SecurityStatusAuthRequest, template (present) SecurityHeaderType p_SecurityStatusAuthResponse, NAS_PlmnId p_PLMN, NR_RadioBearerId_Type p_SRBId := tsc_NR_RbId_SRB1, integer p_AssignmentMode := 1) runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var NR_SecurityParams_Type v_RRC_SecurityParams := f_NR_Security_Get(); var NID_r16 v_NID_ASN; var template (omit) hexstring v_NID := omit; // build AUTHENTICATION REQUEST msg, input PLMN+NID if SNPN test case if(f_GetTestcaseAttrib_NR_SNPN(testcasename() or pc_SNPN_only){ // @sic R5s220753 sic@ v_NID_ASN := f_NR_InitNDI(p_NR_CellId, p_AssignmentMode); // returns a bitstring v_NID := bit2hex(v_NID_ASN); } // send AUTHENTICATION REQUEST v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, p_PLMN, -, v_NID); // @sic R5s220753 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRBId, -, cs_NG_NAS_Request(p_SecurityStatusAuthRequest, v_MsgToSend))); // receive AUTHENTICATION RESPONSE / FAILURE SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRBId, cr_NG_NAS_Ind(p_SecurityStatusAuthResponse))) -> value v_ReceivedAsp; if (not f_Check_5GAKA_AuthenticationResponseFailure(v_NAS_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Authentication Failed"); } // Now NAS parameters updated for Authentication, store updated common values in RRC global variable v_RRC_SecurityParams.KAMF := v_NAS_SecurityParams.KAMF; v_RRC_SecurityParams.KSIamf := v_NAS_SecurityParams.KSIamf; // Now store both f_NR_Security_Set(v_RRC_SecurityParams); f_NR5GC_Security_Set(v_NAS_SecurityParams); }