3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221235

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2835 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of 5GMM Rel-16 RACS test case 9.1.9.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 9.1.9.7 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.1.9.7 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	5

4.1	Samsung SM-G996B	5

5	References	5

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.1.9.7 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

  - Verification Test Summary

Test Case: 	9.1.9.7

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Samsung SM-G996B

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | function fl_TC_9_1_9_7_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | The three coordination messages sent during fl_TC_9_1_9_7_NR5GC_TestBody() are not handled here. |
| Summary of change | Add handling for received coordination triggers |
| TTCN module | RACS_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change

function fl_TC_9_1_9_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; // @siclog "Step 1" siclog@ //The SS configures:- NGC Cell A as "Serving cell"- E-UTRA Cell A as "Non-suitable "off" cell". f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); f_EUTRA38_SetCellPower(eutra_CellA, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); // @siclog "Step 2" siclog@ //Check: Does the UE perform test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1 on NGC Cell A? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_CellA); // @siclog "Step 3" siclog@ //The SS configures:- E-UTRA Cell A as "Serving cell"- NGC Cell A as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_CellA, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); // @siclog "Step 4" siclog@ //Check: Does the UE perform Test procedure for UE Tracking area updating for Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1, 'connected without release' on E-UTRA Cell A? v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_CellA,tsc_RRC_TI_Def); if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapNeeded, cr_UeRadioCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "TAU req failed"); } f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_CellA, v_NasInd, -, noRrcConnectionRelease); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4"); } }

After Change

function fl_TC_9_1_9_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; // @siclog "Step 1" siclog@ //The SS configures:- NGC Cell A as "Serving cell"- E-UTRA Cell A as "Non-suitable "off" cell". f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); f_EUTRA38_SetCellPower(eutra_CellA, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR); // @siclog "Step 2" siclog@ //Check: Does the UE perform test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1 on NGC Cell A? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_CellA); f_EUTRA_NR_WaitForCoOrd_Trigger(NR); // @siclog "Step 3" siclog@ //The SS configures:- E-UTRA Cell A as "Serving cell"- NGC Cell A as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_CellA, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR); // @siclog "Step 4" siclog@ //Check: Does the UE perform Test procedure for UE Tracking area updating for Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1, 'connected without release' on E-UTRA Cell A? v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_CellA,tsc_RRC_TI_Def); if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapNeeded, cr_UeRadioCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "TAU req failed"); } f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_CellA, v_NasInd, -, noRrcConnectionRelease); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4"); } }

- Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

- Execution Log Files

  - Samsung SM-G996B

The Samsung SM-G996B UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_1_9_7_LOG.html

- PICS/PIXIT parameter file:

TC_9_1_9_7_PIXIT.xml

- References

| [1] | R5s221236 : NR5GC FR1 : Supporting information for addition of 5GMM Rel-16 RACS test case 9.1.9.7 |
| --- | --- |