3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221238

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2837 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Rel-16 NR5GC test case 8.1.6.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step 9 Registration Req message is received as an ULinformation transfer message and is handled in the test body, but the message is received from the UE before TTCN function f_NR5GC_508RRC_InterRAT_HO_ToNR is exited, the last SS RACH configuration is expecting a CNF to be received from the system before exiting the function and causing the test case to fail. it is proposed to not wait for the CNF from the SS so that the Registration Req can be handled correctly in the test body |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced a new parameter p_RACHCnfFlag in function f_NR5GC_508RRC_InterRAT_HO_ToNR and passed it as false so that the Cnfflag is set to false. New parameter is also introduced in f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14 to pass the parameter into f_NR_SS_RachProcedureConfig_Def |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for test case 8.1.6.2.3	2

2.1	Correction to fl_TC_8_1_6_2_3_TestBody	2

2.2	Correction to f_NR5GC_508RRC_InterRAT_HO_ToNR	7

2.3	Correction to f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14	13

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

- Corrections required for test case 8.1.6.2.3

  - Correction to fl_TC_8_1_6_2_3_TestBody

| Function name | fl_TC_8_1_6_2_3_TestBody |
| --- | --- |
| Reason for change | At Step 9 Registration Req message is received as an ULinformation transfer message and is handled in the test body, but the message is received from the UE before TTCN function f_NR5GC_508RRC_InterRAT_HO_ToNR is exited, the last SS RACH configuration is expecting a CNF to be received from the system before exiting the function and causing the test case to fail. it is proposed to not wait for the CNF from the SS so that the Registration Req can be handled correctly in the test body |
| Summary of change | Introduced a new parameter p_RACHCnfFlag in function f_NR5GC_508RRC_InterRAT_HO_ToNR and passed it as false so that the Cnfflag is set to false. New parameter is also introduced in f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14 to pass the parameter into f_NR_SS_RachProcedureConfig_Def |
| TTCN module | RRC_MDT_InterRAT_NR5GC_NR |
| MCC160 Comment | Accepted as improvement - use tsc_NoCnfReq instead of false. |

Before Change:

function fl_TC_8_1_6_2_3_TestBody() runs on NR5GC_PTC { … … … //Power levels of NR cell 1: Table 8.1.6.2.3.3.2-1, 8.1.6.2.3.3.2-2 /* ¦ FR1 ¦ FR2 * ------------------------------- * T0 ¦ -90 ¦ FFS----//@sic R5-216499 R5-220052 sic@ * ------------------------------- * T1 ¦ off ¦ off * ------------------------------- * T2 ¦ -88 ¦ FFS //@sic R5-216499 R5-220052 sic@. * ------------------------------- */ v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId); v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth); v_C_RNTI_beforeRLF := f_NR_CellInfo_GetRNTI(nr_Cell1);// @sic R5s220831 sic@ f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT measurement and reporting for event B2 //The UE transmits an RRCReconfigrationComplete message v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)}; v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)}; v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA(v_B2_Threshold1, v_B2_Threshold2EUTRA, v_Hysteresis, v_ReportAmount, true, false))}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE(cs_38508_GapConfig), cs_38508_QuantityConfig_EUTRA); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@siclog "Step 3" siclog@ //The UE transmits a MeasurementReport message on NR Cell 1. f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellId_Cell1, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)})); //@siclog "Step 4" siclog@ //The SS changes NR Cell1 and E-UTRA Cell 1 parameters according to the row "T1" in table 8.1.6.2.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 5" siclog@ //Generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 is performed on E-UTRA Cell 1. f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, -, v_DrbIdList);// @sic R5s220831 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, -, omit);// @sic R5s220831 sic@ //@siclog "Step 6" siclog@ //The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.2.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ //The SS transmits a MobilityFromEUTRACommand message on E-UTRA Cell 1. v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (EUTRA);// @sic R5s220831 sic@ f_NR5GC_MappedContextFromS1_InitNAS (v_SecurityRx, true); // this also sends octetstring to EUTRA and waits for the Complete message //@siclog "Step 8" siclog@ //Check: Does the UE transmit a RRCReconfigurationComplete message containing rlf-InfoAvailable on NR Cell 1? v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); v_RRCConnReconfigComplete:=cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-,cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_RLF))); f_NR5GC_508RRC_InterRAT_HO_ToNR(nr_Cell1, v_PduSessionInfoList, v_SecurityRx,-,-,v_RRCConnReconfigComplete); //@siclog "Step 9" siclog@ //The UE transmits a ULInformationTransfer message and a REGISTRATION REQUEST message indicating "mobility registration updating"? is sent to update the registration of the actual tracking area. SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected))) -> value v_ReceivedAsp; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 10-11" siclog@

After Change:

function fl_TC_8_1_6_2_3_TestBody() runs on NR5GC_PTC { … … … //Power levels of NR cell 1: Table 8.1.6.2.3.3.2-1, 8.1.6.2.3.3.2-2 /* ¦ FR1 ¦ FR2 * ------------------------------- * T0 ¦ -90 ¦ FFS----//@sic R5-216499 R5-220052 sic@ * ------------------------------- * T1 ¦ off ¦ off * ------------------------------- * T2 ¦ -88 ¦ FFS //@sic R5-216499 R5-220052 sic@. * ------------------------------- */ v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId); v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth); v_C_RNTI_beforeRLF := f_NR_CellInfo_GetRNTI(nr_Cell1);// @sic R5s220831 sic@ f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT measurement and reporting for event B2 //The UE transmits an RRCReconfigrationComplete message v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)}; v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)}; v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA(v_B2_Threshold1, v_B2_Threshold2EUTRA, v_Hysteresis, v_ReportAmount, true, false))}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE(cs_38508_GapConfig), cs_38508_QuantityConfig_EUTRA); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@siclog "Step 3" siclog@ //The UE transmits a MeasurementReport message on NR Cell 1. f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellId_Cell1, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)})); //@siclog "Step 4" siclog@ //The SS changes NR Cell1 and E-UTRA Cell 1 parameters according to the row "T1" in table 8.1.6.2.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 5" siclog@ //Generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 is performed on E-UTRA Cell 1. f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Power Level"); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, -, v_DrbIdList);// @sic R5s220831 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, -, omit);// @sic R5s220831 sic@ //@siclog "Step 6" siclog@ //The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.2.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ //The SS transmits a MobilityFromEUTRACommand message on E-UTRA Cell 1. v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (EUTRA);// @sic R5s220831 sic@ f_NR5GC_MappedContextFromS1_InitNAS (v_SecurityRx, true); // this also sends octetstring to EUTRA and waits for the Complete message //@siclog "Step 8" siclog@ //Check: Does the UE transmit a RRCReconfigurationComplete message containing rlf-InfoAvailable on NR Cell 1? v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); v_RRCConnReconfigComplete:=cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-,cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_RLF))); f_NR5GC_508RRC_InterRAT_HO_ToNR(nr_Cell1, v_PduSessionInfoList, v_SecurityRx,-,-,v_RRCConnReconfigComplete,false); // added RACH config cnf flag //@siclog "Step 9" siclog@ //The UE transmits a ULInformationTransfer message and a REGISTRATION REQUEST message indicating "mobility registration updating"? is sent to update the registration of the actual tracking area. SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected))) -> value v_ReceivedAsp; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 10-11" siclog@

  - Correction to f_NR5GC_508RRC_InterRAT_HO_ToNR

| Function name | f_NR5GC_508RRC_InterRAT_HO_ToNR |
| --- | --- |
| Reason for change | At Step 9 Registration Req message is received as an ULinformation transfer message and is handled in the test body, but the message is received from the UE before TTCN function f_NR5GC_508RRC_InterRAT_HO_ToNR is exited, the last SS RACH configuration is expecting a CNF to be received from the system before exiting the function and causing the test case to fail. it is proposed to not wait for the CNF from the SS so that the Registration Req can be handled correctly in the test body |
| Summary of change | Introduced a new parameter p_RACHCnfFlag in function f_NR5GC_508RRC_InterRAT_HO_ToNR and passed it as false so that the Cnfflag is set to false. |
| TTCN module | NR5GC_IRAT_CommonFunctions |
| MCC160 Comment | Accepted as improvement |

Before Change:

function f_NR5GC_508RRC_InterRAT_HO_ToNR(NR_CellId_Type p_TargetCellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList, EUTRA_NR_CoOrd_Security_Type p_EUTRASecurityParams, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete ) runs on NR5GC_PTC { var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA var template (value) RRCReconfiguration v_RRCReconfiguration; var template (value) PDUSessionInfoList_Type v_SessionInfoList; var integer v_NoOfPduSessions := lengthof(p_PduSessionInfoList); var template (value) RadioBearerConfig v_RadioBearerConfig; var template(value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_RadioBearerList; // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here) var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var template (value) MasterKeyUpdate v_MasterKeyUpdate; var NextHopChainingCount v_NCC := tsc_NR_38508_NextHopChainingCount; var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var MessageAuthenticationCode v_CalculatedMac; var template (value) S1ToN1TransparentContainer v_NASContainer; var bitstring v_NASContainerString; var PDU_PDN_DNN_Type v_PDU_Type := Internet_DNN; // assume this for now @sic R5-206296 sic@ var integer i; if (isvalue(p_RACH_ConfigDedicated.uplink)){ //CFRA v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); v_RachProcedureConfig := cs_NR_RachProcedureConfig_CFRA(v_NR_PhysicalParameters); } v_NASContainer := cs_S1ToN1TransparentContainer (omit, v_SecurityParams.NAS_Ciphering.Algorithm, v_SecurityParams.NAS_Integrity.Algorithm, int2bit(p_EUTRASecurityParams.NCC, 3), // @sic R5s201078 sic@ tsc_NasKsi_MappedSecurityContext, p_EUTRASecurityParams.KSI); // @sic R5s201078 sic@ v_NASContainerString := encvalue(valueof(v_NASContainer)); v_CalculatedMac := fx_NG_NasIntegrityAlgorithm(bit2oct(v_NASContainerString), v_SecurityParams.NAS_Integrity.Algorithm, v_SecurityParams.NAS_Integrity.K_NAS, 'FFFFFFFF'O, // according to 33.501 cl. 8.4.2 '00001'B, tsc_DirectionDL); v_NASContainer.mac := v_CalculatedMac; v_NASContainerString := encvalue(v_NASContainer); v_MasterKeyUpdate := cs_MasterKeyUpdate(true, v_NCC, bit2oct(v_NASContainerString)); for (i := 0; i < v_NoOfPduSessions; i := i + 1) { if (p_PduSessionInfoList[i].IsIMS) { // @sic R5-206296 sic@ v_PDU_Type := IMS_DNN; } v_SessionInfoList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionInfoList[i].PDU_SessionId, 1), -, v_PDU_Type); f_NR5GC_MobileInfo_AddPDUSession(valueof(v_SessionInfoList[i])); } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_RadioBearerList, v_SDAP_ConfigurationList, p_TargetCellId, valueof(v_SessionInfoList), NONE); v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_InterRAT_HO(v_RadioBearerConfig, v_CellGroupConfig, -, v_MasterKeyUpdate); // Enable IP handling for each default DRB for (i := 0; i < v_NoOfPduSessions; i := i + 1) { f_IP_ChangeDrbMappingNR(IP, p_TargetCellId, cs_IP_QosFlowId(p_PduSessionInfoList[i].PDU_SessionId, p_PduSessionInfoList[i].QFI), p_PduSessionInfoList[i].PdnIndex); } f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14(p_TargetCellId, v_RadioBearerList, p_RNTI_Value, v_RRCReconfiguration, v_RachProcedureConfig, -, p_RRCConnReconfigComplete, // @sic R5-216287 sic@ v_NCC, p_RACH_ConfigDedicated); }

After Change:

function f_NR5GC_508RRC_InterRAT_HO_ToNR(NR_CellId_Type p_TargetCellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList, EUTRA_NR_CoOrd_Security_Type p_EUTRASecurityParams, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, template (omit) boolean p_RACHCnfFlag := omit // added RACH config cnf flag ) runs on NR5GC_PTC { var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA var template (value) RRCReconfiguration v_RRCReconfiguration; var template (value) PDUSessionInfoList_Type v_SessionInfoList; var integer v_NoOfPduSessions := lengthof(p_PduSessionInfoList); var template (value) RadioBearerConfig v_RadioBearerConfig; var template(value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_RadioBearerList; // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here) var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var template (value) MasterKeyUpdate v_MasterKeyUpdate; var NextHopChainingCount v_NCC := tsc_NR_38508_NextHopChainingCount; var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var MessageAuthenticationCode v_CalculatedMac; var template (value) S1ToN1TransparentContainer v_NASContainer; var bitstring v_NASContainerString; var PDU_PDN_DNN_Type v_PDU_Type := Internet_DNN; // assume this for now @sic R5-206296 sic@ var integer i; if (isvalue(p_RACH_ConfigDedicated.uplink)){ //CFRA v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); v_RachProcedureConfig := cs_NR_RachProcedureConfig_CFRA(v_NR_PhysicalParameters); } v_NASContainer := cs_S1ToN1TransparentContainer (omit, v_SecurityParams.NAS_Ciphering.Algorithm, v_SecurityParams.NAS_Integrity.Algorithm, int2bit(p_EUTRASecurityParams.NCC, 3), // @sic R5s201078 sic@ tsc_NasKsi_MappedSecurityContext, p_EUTRASecurityParams.KSI); // @sic R5s201078 sic@ v_NASContainerString := encvalue(valueof(v_NASContainer)); v_CalculatedMac := fx_NG_NasIntegrityAlgorithm(bit2oct(v_NASContainerString), v_SecurityParams.NAS_Integrity.Algorithm, v_SecurityParams.NAS_Integrity.K_NAS, 'FFFFFFFF'O, // according to 33.501 cl. 8.4.2 '00001'B, tsc_DirectionDL); v_NASContainer.mac := v_CalculatedMac; v_NASContainerString := encvalue(v_NASContainer); v_MasterKeyUpdate := cs_MasterKeyUpdate(true, v_NCC, bit2oct(v_NASContainerString)); for (i := 0; i < v_NoOfPduSessions; i := i + 1) { if (p_PduSessionInfoList[i].IsIMS) { // @sic R5-206296 sic@ v_PDU_Type := IMS_DNN; } v_SessionInfoList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionInfoList[i].PDU_SessionId, 1), -, v_PDU_Type); f_NR5GC_MobileInfo_AddPDUSession(valueof(v_SessionInfoList[i])); } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_RadioBearerList, v_SDAP_ConfigurationList, p_TargetCellId, valueof(v_SessionInfoList), NONE); v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_InterRAT_HO(v_RadioBearerConfig, v_CellGroupConfig, -, v_MasterKeyUpdate); // Enable IP handling for each default DRB for (i := 0; i < v_NoOfPduSessions; i := i + 1) { f_IP_ChangeDrbMappingNR(IP, p_TargetCellId, cs_IP_QosFlowId(p_PduSessionInfoList[i].PDU_SessionId, p_PduSessionInfoList[i].QFI), p_PduSessionInfoList[i].PdnIndex); } f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14(p_TargetCellId, v_RadioBearerList, p_RNTI_Value, v_RRCReconfiguration, v_RachProcedureConfig, -, p_RRCConnReconfigComplete, // @sic R5-216287 sic@ v_NCC, p_RACH_ConfigDedicated, p_RACHCnfFlag); // added RACH config cnf flag }

  - Correction to f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14

| Function name | f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14 |
| --- | --- |
| Reason for change | At Step 9 Registration Req message is received as an ULinformation transfer message and is handled in the test body, but the message is received from the UE before TTCN function f_NR5GC_508RRC_InterRAT_HO_ToNR is exited, the last SS RACH configuration is expecting a CNF to be received from the system before exiting the function and causing the test case to fail. it is proposed to not wait for the CNF from the SS so that the Registration Req can be handled correctly in the test body |
| Summary of change | New parameter is also introduced in f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14 to pass the parameter into f_NR_SS_RachProcedureConfig_Def |
| TTCN module | NR5GC_IRAT_CommonFunctions |
| MCC160 Comment | Accepted as improvement |

Before Change:

function f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14(NR_CellId_Type p_TargetCellId, template( value) NR_RadioBearerList_Type p_RadioBearerList, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (value) RRCReconfiguration p_RRCReconfiguration, template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig, template (value) UL_GrantConfig_Type p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, NextHopChainingCount p_NCC := tsc_NR_38508_NextHopChainingCount, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit ) runs on NR_BASE_PTC { var octetstring v_EncodedRRCReconfig; // Step 1: Target Cell: Configuration of DRBs f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, p_RadioBearerList); // Step 2: void // Don't need this step as 1st time on NR, so all counts start from 0 //f_NR_SS_PdcpCount_Handover(p_SourceCellId, p_TargetCellId, p_RadioBearerList, p_RBConfig_KeyChange); // Step 3: void //f_NR_SS_PdcpHandoverCtrl(p_TargetCellId, cas_NR_PdcpHandoverInit_REQ(p_SourceCellId, p_TargetCellId)); // Step 4: Target Cell: Configure RACH procedure either dedicated or C-RNTI based. // Save new C-RNTI in cell configuration for p_TargetCellId f_NR_CellInfo_SetRNTI(p_TargetCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_TargetCellId, p_RNTI_Value); f_NR_SS_RachProcedureConfigHO(p_TargetCellId, p_RachProcedureConfig, -, p_RACH_ConfigDedicated); //Step 5: Target Cell: Activate security. f_NR_SS_AS_ActivateSecurity_Current(p_TargetCellId, p_NCC); // Step 6: Void // Step 7: void // Step 8: Target Cell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_Now, -, -, p_UL_GrantConfig_TargetCell); // Step 9: Send RRCReconfiguration to EUTRA v_EncodedRRCReconfig := bit2oct(encvalue(p_RRCReconfiguration)); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (v_EncodedRRCReconfig, "HO Cmd")); // Step 10: Target Cell: Receive RRCReconfigurationComplete SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, p_RRCConnReconfigComplete)); // Step 11: Target Cell: Start periodic TA f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -); // Step 12: void // Step 13: Target Cell: Re-configure RACH procedure as for initial access. f_NR_SS_RachProcedureConfig_Def(p_TargetCellId);

After Change:

function f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14(NR_CellId_Type p_TargetCellId, template( value) NR_RadioBearerList_Type p_RadioBearerList, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (value) RRCReconfiguration p_RRCReconfiguration, template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig, template (value) UL_GrantConfig_Type p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, NextHopChainingCount p_NCC := tsc_NR_38508_NextHopChainingCount, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (omit) boolean p_RACHCnfFlag := omit // added RACH config cnf flag ) runs on NR_BASE_PTC { var octetstring v_EncodedRRCReconfig; // Step 1: Target Cell: Configuration of DRBs f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, p_RadioBearerList); // Step 2: void // Don't need this step as 1st time on NR, so all counts start from 0 //f_NR_SS_PdcpCount_Handover(p_SourceCellId, p_TargetCellId, p_RadioBearerList, p_RBConfig_KeyChange); // Step 3: void //f_NR_SS_PdcpHandoverCtrl(p_TargetCellId, cas_NR_PdcpHandoverInit_REQ(p_SourceCellId, p_TargetCellId)); // Step 4: Target Cell: Configure RACH procedure either dedicated or C-RNTI based. // Save new C-RNTI in cell configuration for p_TargetCellId f_NR_CellInfo_SetRNTI(p_TargetCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_TargetCellId, p_RNTI_Value); f_NR_SS_RachProcedureConfigHO(p_TargetCellId, p_RachProcedureConfig, -, p_RACH_ConfigDedicated); //Step 5: Target Cell: Activate security. f_NR_SS_AS_ActivateSecurity_Current(p_TargetCellId, p_NCC); // Step 6: Void // Step 7: void // Step 8: Target Cell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_Now, -, -, p_UL_GrantConfig_TargetCell); // Step 9: Send RRCReconfiguration to EUTRA v_EncodedRRCReconfig := bit2oct(encvalue(p_RRCReconfiguration)); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (v_EncodedRRCReconfig, "HO Cmd")); // Step 10: Target Cell: Receive RRCReconfigurationComplete SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, p_RRCConnReconfigComplete)); // Step 11: Target Cell: Start periodic TA f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -); // Step 12: void // Step 13: Target Cell: Re-configure RACH procedure as for initial access. f_NR_SS_RachProcedureConfig_Def(p_TargetCellId,-,p_RACHCnfFlag); // added RACH config cnf flag