3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221244

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2840 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC eNS test case 9.1.10.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When pc_USIM_Removal = TRUE, as specified in 38.523-1, Table 9.1.10.6.3.2-1, test will undergo Steps 28a1 to 28a27. - For Step 28a23, UE is to complete Steps 1a2 to 1a5 of generic procedure for Switch off in RRC_IDLE specified in TS 38.508-1[4], subclause 4.9.6.1. This is accomplished by calling the function f_NR_UE_DeregisterOnSwitchoff(). In the current TTCN implementation, after executing f_NR_UE_DeregisterOnSwitchoff(), the same procedures are then called again outside of the function (redundancy issue). Leading to a timeout failure after UE is switched to IDLE mode after successful completion of f_NR_UE_DeregisterOnSwitchoff(), - As UE is shifted to IDLE_MODE on Step28a23a1, after insertion of USIM and Power_On for Cell A, Switch_On command is required to trigger attach procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Step 28a23 redundant steps are removed. - Step 28a26 Switch_On at command is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_9_1_10_6_TestBody() |
| --- | --- |
| Reason for change | When pc_USIM_Removal = TRUE, as specified in 38.523-1, Table 9.1.10.6.3.2-1, test will undergo Steps 28a1 to 28a27. - For Step 28a23, UE is to complete Steps 1a2 to 1a5 of generic procedure for Switch off in RRC_IDLE specified in TS 38.508-1[4], subclause 4.9.6.1. This is accomplished by calling the function f_NR_UE_DeregisterOnSwitchoff(). In the current TTCN implementation, after executing f_NR_UE_DeregisterOnSwitchoff(), the same procedures are then called again outside of the function (redundancy issue). Leading to a timeout failure after UE is switched to IDLE mode after successful completion of f_NR_UE_DeregisterOnSwitchoff(), - As UE is shifted to IDLE_MODE on Step28a23a1, after insertion of USIM and Power_On for Cell A, Switch_On command is required to trigger attach procedure. |
| Summary of change | - Step 28a23 redundant steps are removed. - Step 28a26 Switch_On at command is added. |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment | - Accepted - Rejected: The UE is not Switched off earlier, only the USIM is removed. Thus just USIM insertion should be fine. |

Before Change:

function fl_TC_9_1_10_6_TestBody() runs on NR5GC_PTC { . . . //@siclog "Step 27A" siclog@ //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //EXCEPTION: Steps 28a1 to 28a27 take place if pc_USIM_Removal = TRUE if(pc_USIM_Removal) { //@siclog "Step 28a1 - 28a21" siclog@ fl_TC_9_1_10_6_Steps2To20(); /*Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause 'S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization' associated with current PLMN */ v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s220222 sic@ if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { // @sic R5s220222 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } //@siclog "Step 28a21" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=2 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); // @sic R5s220222 sic@ //@siclog "Step 28a22" siclog@ f_UT_USIM_Remove (UT, NO_CNF_REQUIRED); // @sic R5-222007 sic@ //@siclog "Step 28a23a1 - 28a23a4" siclog@ //steps 1a2 to 1a5 of Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed. // @sic R5-222007 sic@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A, -, true); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); v_ReceivedMsg := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(v_NGC_NASCellA, v_ReceivedMsg); // @sic R5-204389 sic@ f_NR_RRCRelease(v_NGC_NASCellA); if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedMsg.Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } //@siclog "Step 28a24" siclog@ //The SS configures NGC Cell A as the "Non-suitable cell" f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); //@siclog "Step 28a25" siclog@ //The USIM is inserted into the UE. f_UT_USIM_Insert(UT, omit); // @sic R5-222007 sic@ //@siclog "Step 28a26" siclog@ /*verify that S-NSSAI=2 is not in the Rejected NSSAI list associated with current PLMN */ v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28a26"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28a26"); //@siclog "Step 28a27" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); } . . } }

After Change:

function fl_TC_9_1_10_6_TestBody() runs on NR5GC_PTC { . . . //@siclog "Step 27A" siclog@ //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //EXCEPTION: Steps 28a1 to 28a27 take place if pc_USIM_Removal = TRUE if(pc_USIM_Removal) { //@siclog "Step 28a1 - 28a21" siclog@ fl_TC_9_1_10_6_Steps2To20(); /*Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause 'S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization' associated with current PLMN */ v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s220222 sic@ if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { // @sic R5s220222 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } //@siclog "Step 28a21" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=2 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); // @sic R5s220222 sic@ //@siclog "Step 28a22" siclog@ f_UT_USIM_Remove (UT, NO_CNF_REQUIRED); // @sic R5-222007 sic@ //@siclog "Step 28a23a1 - 28a23a4" siclog@ //steps 1a2 to 1a5 of Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed. // @sic R5-222007 sic@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A, -, true); //@siclog "Step 28a24" siclog@ //The SS configures NGC Cell A as the "Non-suitable cell" f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); //@siclog "Step 28a25" siclog@ //The USIM is inserted into the UE. f_UT_USIM_Insert(UT, omit); // @sic R5-222007 sic@ //@siclog "Step 28a26" siclog@ /*verify that S-NSSAI=2 is not in the Rejected NSSAI list associated with current PLMN */ v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28a26"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28a26"); //@siclog "Step 28a27" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_SwitchOnAndResetIMS(); } . . . } }