3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221255

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4702 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UDP and ICMPv6 Bind functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the raise of R5s220117 an implementation was included to handle the attempt of closing multiple TCP sockets. TCP close was handled without blocking the processing of expected messages. Some legacy test cases are facing a similar situation (for which R5s220117 was raised). TCP closing attemps are being sent blocking the processing of required messages to advance the test cases, making them fail unfairly. This can be solved applying R5s220117 principles to different functions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added TCP close message handling to not block expected message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.5.1, 11.2.7, 9.2.3.1.10, 9.2.3.1.11, 9.2.3.1.12, 9.3.1.4, 9.3.1.5, 9.3.1.6, 8.2.1.5, 9.2.1.1.2, 9.2.2.2.1, 9.2.2.2.14, 9.2.3.1.13, 9.2.3.1.14, 9.2.3.1.16, 9.2.3.1.18, 9.2.3.2.10, 9.2.3.2.12, 9.3.1.7, 9.3.1.7a, 9.3.1.17, 9.2.3.1.15a, 9.2.3.1.18a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Correction to function fl_ICMPv6_Bind	3

2.2	Correction to function fl_UDP_Bind	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function fl_ICMPv6_Bind

| Function name | fl_ICMPv6_Bind |
| --- | --- |
| Reason for change | TCP closing attemps are being sent blocking the processing of required messages to advance the test cases, making them fail unfairly. |
| Summary of change | Added TCP close message handling to not block expected message. |
| TTCN module | \Common\IP_PTC\IP_PTC_Main.ttcn |
| MCC160 Comment | Accepted in principle as improvement even though the proposed change does not affect TCP closing |

Before Change:

<<SKIPPED CODE>> function fl_ICMPv6_Bind(template (value) IP_AddrInfo_Type p_LocalIpAddr, boolean p_AllowBroadcast) runs on IP_PTC { /* @sic R5-113734 change 8 sic@ */ IP_SOCK_CTRL.send(cs_ICMPv6_SOCKET_REQ(cs_IP_Socket(p_LocalIpAddr, omit), p_AllowBroadcast)); // LocalPort=omit -> ICMP does not have a port number IP_SOCK_CTRL.receive(cr_ICMPv6_SOCKET_CNF(cr_IP_Socket(p_LocalIpAddr, omit))); // LocalPort=omit -> ICMP does not have a port number } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_ICMPv6_Bind(template (value) IP_AddrInfo_Type p_LocalIpAddr, boolean p_AllowBroadcast) runs on IP_PTC { /* @sic R5-113734 change 8 sic@ */ var IP_SOCKET_IND v_IP_SOCKET_IND; IP_SOCK_CTRL.send(cs_ICMPv6_SOCKET_REQ(cs_IP_Socket(p_LocalIpAddr, omit), p_AllowBroadcast)); // LocalPort=omit -> ICMP does not have a port number alt{ []IP_SOCK_CTRL.receive(cr_ICMPv6_SOCKET_CNF(cr_IP_Socket(p_LocalIpAddr, omit))){ // LocalPort=omit -> ICMP does not have a port number IP_SOCK_CTRL_QUEUE.send(crs_IP_FLUSH_IND); } []IP_SOCK_CTRL.receive(cr_IP_SOCKET_IND_AnyCtrl) -> value v_IP_SOCKET_IND{ IP_SOCK_CTRL_QUEUE.send(v_IP_SOCKET_IND); repeat; } }; } <<SKIPPED CODE>>

## Correction to function fl_UDP_Bind

| Function name | fl_UDP_Bind |
| --- | --- |
| Reason for change | TCP closing attemps are being sent blocking the processing of required messages to advance the test cases, making them fail unfairly. |
| Summary of change | Added TCP close message handling to not block expected message. |
| TTCN module | \Common\IP_PTC \UDP_Functions.ttcn |
| MCC160 Comment | Accepted in principle as improvement even though the proposed change does not affect TCP closing |

Before Change:

<<SKIPPED CODE>> function fl_UDP_Bind(template (value) IP_AddrInfo_Type p_LocalIpAddr, template (omit) PortNumber_Type p_LocalPort, boolean p_AllowBroadcast) runs on IP_PTC return PortNumber_Type { /* @sic R5-113734 change 8 sic@ */ var IP_SOCKET_IND v_IP_SOCKET_IND; IP_SOCK_CTRL.send(cs_UDP_SOCKET_REQ(cs_IP_Socket(p_LocalIpAddr, p_LocalPort), p_AllowBroadcast)); IP_SOCK_CTRL.receive(cr_UDP_SOCKET_CNF(cr_IP_Socket(p_LocalIpAddr, ?))) -> value v_IP_SOCKET_IND; return v_IP_SOCKET_IND.CTRL.ConnectionId.Local.Port; /* in case of p_LocalPort=omit we need to get back the ephemeral port */ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_UDP_Bind(template (value) IP_AddrInfo_Type p_LocalIpAddr, template (omit) PortNumber_Type p_LocalPort, boolean p_AllowBroadcast) runs on IP_PTC return PortNumber_Type { /* @sic R5-113734 change 8 sic@ */ var IP_SOCKET_IND v_IP_SOCKET_IND; IP_SOCK_CTRL.send(cs_UDP_SOCKET_REQ(cs_IP_Socket(p_LocalIpAddr, p_LocalPort), p_AllowBroadcast)); alt{ []IP_SOCK_CTRL.receive(cr_UDP_SOCKET_CNF(cr_IP_Socket(p_LocalIpAddr, ?))) -> value v_IP_SOCKET_IND{ IP_SOCK_CTRL_QUEUE.send(crs_IP_FLUSH_IND); } []IP_SOCK_CTRL.receive(cr_IP_SOCKET_IND_AnyCtrl) -> value v_IP_SOCKET_IND{ IP_SOCK_CTRL_QUEUE.send(v_IP_SOCKET_IND); repeat; } }; return v_IP_SOCKET_IND.CTRL.ConnectionId.Local.Port; /* in case of p_LocalPort=omit we need to get back the ephemeral port */ } <<SKIPPED CODE>>

## MCC160 Implementation

altstep a_IP_PTC_ReceiveCNF_NonBlocking(template (present) IP_SOCKET_IND p_ExpectedCNF, out IP_SOCKET_IND p_ReceivedCNF) runs on IP_PTC { var IP_SOCKET_IND v_IP_SOCKET_IND; [] IP_SOCK_CTRL.receive(p_ExpectedCNF) -> value v_IP_SOCKET_IND { IP_SOCK_CTRL_QUEUE.send(crs_IP_FLUSH_IND); p_ReceivedCNF := v_IP_SOCKET_IND; } [] IP_SOCK_CTRL.receive(cr_IP_SOCKET_IND_AnyCtrl) -> value v_IP_SOCKET_IND { IP_SOCK_CTRL_QUEUE.send(v_IP_SOCKET_IND); repeat; } }

function f_IP_PTC_SendREQ_ReceiveCNF_NonBlocking(template (value) IP_SOCKET_REQ p_IP_SOCKET_REQ, template (present) IP_SOCKET_IND p_IP_SOCKET_CNF) runs on IP_PTC return IP_SOCKET_IND { var IP_SOCKET_IND v_IP_SOCKET_IND; IP_SOCK_CTRL.send(p_IP_SOCKET_REQ); a_IP_PTC_ReceiveCNF_NonBlocking(p_IP_SOCKET_CNF, v_IP_SOCKET_IND); return v_IP_SOCKET_IND; }

function fl_TCP_Close(template (value) IP_Connection_Type p_IP_Connection, boolean p_WaitForCNF) runs on IP_PTC { var template (present) IP_Connection_Type v_IP_Connection := p_IP_Connection; var IP_SOCKET_IND v_IP_SOCKET_IND; if (not isvalue(p_IP_Connection.Remote)) { v_IP_Connection.Remote := ?; } IP_SOCK_CTRL.send(cs_TCP_CLOSE_REQ(p_IP_Connection)); if (p_WaitForCNF) { alt { [] a_IP_PTC_ReceiveCNF_NonBlocking(cr_TCP_CLOSE_CNF(p_IP_Connection), v_Dummy) {} [] IP_SOCK_CTRL.receive(cr_TCP_CLOSE_IND(v_IP_Connection)) { repeat; } } } }

function fl_UDP_Bind(template (value) IP_AddrInfo_Type p_LocalIpAddr, template (omit) PortNumber_Type p_LocalPort, boolean p_AllowBroadcast) runs on IP_PTC return PortNumber_Type { var IP_SOCKET_IND v_IP_SOCKET_IND; v_IP_SOCKET_IND := f_IP_PTC_SendREQ_ReceiveCNF_NonBlocking(cs_UDP_SOCKET_REQ(cs_IP_Socket(p_LocalIpAddr, p_LocalPort), p_AllowBroadcast), cr_UDP_SOCKET_CNF(cr_IP_Socket(p_LocalIpAddr, ?))); return v_IP_SOCKET_IND.CTRL.ConnectionId.Local.Port; /* in case of p_LocalPort=omit we need to get back the ephemeral port */ }

function fl_ICMPv6_Bind(template (value) IP_AddrInfo_Type p_LocalIpAddr, boolean p_AllowBroadcast) runs on IP_PTC { f_IP_PTC_SendREQ_ReceiveCNF_NonBlocking(cs_ICMPv6_SOCKET_REQ(cs_IP_Socket(p_LocalIpAddr, omit), p_AllowBroadcast), // LocalPort=omit -> ICMP does not have a port number cr_ICMPv6_SOCKET_CNF(cr_IP_Socket(p_LocalIpAddr, omit))); // LocalPort=omit -> ICMP does not have a port number }