3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221322

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2857 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current TTCN uses the constant tsc_LCID_SCG_Bearer = ‘000101’B (=5) when creating MAC PDUs. If the UE has created only an IMS PDU session during preamble, the DRB ID passed into the function will be 1 which will be associated with LCID 4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Derive LCID to use based on DRB ID parameter (LCID = DRB ID + 3) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.9.NR5GC, 7.1.1.3.9.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_9_NR_TestBody() |
| --- | --- |
| Reason for change | Current TTCN uses the constant tsc_LCID_SCG_Bearer = ‘000101’B (=5) when creating MAC PDUs. If the UE has created only an IMS PDU session during preamble, the DRB ID passed into the function will be 1 which will be associated with LCID 4. |
| Summary of change | Derive LCID to use based on DRB ID parameter (LCID = DRB ID + 3) |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_MAC_SDU_Type v_EncodedRlcPdu; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var boolean v_FDD := f_NR_CellInfo_GetIsFDD(nr_Cell1); var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); var default v_IgnoreNRSR := null; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; /*Skipping the code */ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header=44B =352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); //@sic R5s220332 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu) }; /*Skipping the code */ //@siclog "Step 5" siclog@ //SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 4. f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 1, p_NR_DRB_Id); }

After Change:

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var NR_MAC_SDU_Type v_EncodedRlcPdu; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var boolean v_FDD := f_NR_CellInfo_GetIsFDD(nr_Cell1); var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); var default v_IgnoreNRSR := null; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//KS@22xxxx /*Skipping the code */ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header=44B =352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); //@sic R5s220332 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu) }; /*Skipping the code */ //@siclog "Step 5" siclog@ //SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 4. f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id); }