3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230001

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2871 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-16 power saving test case 7.1.1.12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 7.1.1.12.3 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.12.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	17

2.3	Change 3	22

2.4	Change 4	24

2.5	Change 5	30

2.6	Change 6	33

2.7	Change 7	35

2.8	Change 8	39

2.9	Change 9	40

2.10	Change 10	41

2.11	Change 11	42

2.12	Change 12	42

2.13	Change 13	42

2.14	Change 14	46

2.15	Change 15	46

2.16	Change 16	47

2.17	Change 17	48

2.18	Change 18	50

2.19	Change 19	50

2.20	Change 20	51

3	Branches executed	53

4	Execution Log Files	53

4.1	MediaTek MT6879	53

5	References	53

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.1.12.3 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.1.1.12.3

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	MediaTek MT6879

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_12_3_NR5GC |
| --- | --- |
| Reason for change | - At step 1-2 and 12-13, v_PhysicalCellGroupConfig is changed to config dcp_Config_r16, but is not saved for future usage. - at steps 1-2 and 12-13 v_MAC_CellGroupConfig is changed but not applied to SS configuration - According to 38.331: • DCI_2_6 should be sent via common search space; and • DCI_2_6 should be configured via searchSpaceToAddModListExt; and • if the network includes searchSpaceToAddModListExt, it includes the same number of entries, and listed in the same order, as in searchSpacesToAddModList in each of them. However in Table 7.1.1.12.3.3.3-4, DCI_2_6 is configured via searchSpacesToAddModList with UE Specific search space - drx-LongCycle for step 3-6 is 10240ms, not 1024 - MeasGapConfig should be included in the RRCReconfiguration message at step 20. - Due to the inclusion of MeasGapConfig, the sending of RRCReconfiguration at step 20 should be timed to alighn with MeasGap configuration at the SS. But the existing function f_NR_SendRRCReconfiguration() can not be used as this does not consider DRX configuration. - According to current DRX/DCP configuration of step 20, the DCP occasion would be * ¦ * ¦ 4 ¦ *, but according to tdd-UL-DL-ConfigurationCommon (7, 6, 2,4), subframe 4 is a UL subframe, so for TDD, the DCI_2_6 at step22A can not be sent out. - v_MAC_CellGroupConfig is changed before step 22-24, but is not applied to SS - The time returned from f_NR_GetNextSendOccasion_7_1_1_5_X is the start point of the active time of DRX, it is not always a valid PDCCH occasion for PDSCH(C_RNTI) for sending RRCRelease, see also change 6. |
| Summary of change | - Call f_NR_CellInfo_SetPhysicalCellGroupConfig to save v_PhysicalCellGroupConfig - apply the changes on v_MAC_CellGroupConfig to SS while calling f_NR_SS_ServingCellConfig - modify PDCCH-Config in rrcReconfiguration and config SS to enable DCI_2_6 searchspace set v_SpCellConfig.spCellConfigDedicated.tag_Id configure SS to create common searchspace for DCI_2_6 using searchSpaceToAddModListExt Note : An associated prose CR on 38.523-1 will be raised at RAN5#98 - Change v_LongDRXCycle and v_DrxStartOffset according to actual LongCycleStartOffset configuration - Include MeasGapConfig in the RRCReconfiguration message at step 20 - Send RRCReconfiguration at specific TTI, including MeasGapConfig: - include the measGapConfig and change ps-offset-r16 to 32 - Sent DCI_2_6 to wake-up UE before sent rrcReconfigure message - Sent rrcReconfiguration at T - Configure the Measurement Gap at T + 5ms - Configure the new DRX/DCP at SS at T + 5ms - Save the new Physical Cell Group - Waiting for RRCReconfigurationComplete - Change ps-offset-r16 to 32 so that DCI 2_6 is sent in a valid DL slot Note : An associated prose CR on 38.523-1 will be raised at RAN5#98 - call f_NR_SS_ServingCellConfig to apply the changes on v_MAC_CellGroupConfig to SS before step 22-24 - Use function f_NR_GetWakeUpSignalTiming() to get valid occasion for rrcRelease, see also change 6 |
| TTCN module | MAC_PowerSaving_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication var DRB_Identity v_DRBId; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template(value) ControlResourceSet v_ControlResourceSet; var UInt_Type v_LongDRXCycle; var UInt_Type v_DrxStartOffset; var template (value) DRX_Config.drx_onDurationTimer v_NR_DRX_OnDurationTimer; var template (value) DRX_Config.drx_LongCycleStartOffset v_NR_DRX_LongCycleStartOffset; var SubFrameTiming_Type v_Timing; var RSRP_Range v_RsrpThres; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (omit) MeasGapConfig v_MeasGapUE ; //Init Cell parameters f_NR5GC_Init(NR_1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId), -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); //@siclog Steps 1-2 siclog@ //SS transmits RRCReconfiguration to configure specific DCP parameters. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) //Table 7.1.1.12.3.3.3-2 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.dcp_Config_r16.setup := cs_DCP_Config_r16_Def; f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); //Table 7.1.1.12.3.3.3-4 v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := 1; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet} v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_Dci_Format2_6} v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, -, -, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); v_LongDRXCycle := 1024; v_DrxStartOffset := 0; //@siclog Steps 3-6 siclog@ f_TC_7_1_1_12_3_NR_Steps3_6(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); //@siclog Steps 7-8 siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := true_; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog Steps 9-11 siclog@ f_TC_7_1_1_12_3_NR_Steps9_11(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); //@siclog Steps 12-13 siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_NR_DRX_OnDurationTimer := { milliSeconds := ms10 }; v_NR_DRX_LongCycleStartOffset := {ms20 := 0}; v_MAC_CellGroupConfig.drx_Config.setup := cs_NR_DRX_Config_Def( v_NR_DRX_OnDurationTimer, ms6, 56, 56, sl320, sl320, v_NR_DRX_LongCycleStartOffset, -, 0); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_CellGroupConfig.mac_CellGroupConfig := v_MAC_CellGroupConfig; v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := omit; v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_Offset_r16 := 40; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog Steps 14-19 siclog@ f_TC_7_1_1_12_3_NR_Steps14_19(v_LongDRXCycle, v_DrxStartOffset, v_DRBId, valueof(v_CellGroupConfig.mac_CellGroupConfig.drx_Config.setup.drx_HARQ_RTT_TimerDL)); //@siclog Steps 20-24 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_RsrpThres := 2; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, omit, omit), cs_MeasObjectId2(nr_Cell3, omit, omit)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasGapUE := cs_38508_MeasGapConfig_UE(cs_NR_GapConfig(34, ms6, ms40, ms0)); f_NR_SS_MeasGapCtrlConfig(nr_Cell1, cs_NR_MeasGapCtrl_Config(v_MeasGapUE)); v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_RsrpThres, infinity_, -, -, cs_NR_MeasReportQuantity_Rsrp)}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); //@sic R5-192854, R5-199097 sic@ if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } v_MAC_CellGroupConfig.drx_Config.setup.drx_onDurationTimer := v_NR_DRX_OnDurationTimer; v_MAC_CellGroupConfig.drx_Config.setup.drx_InactivityTimer := ms5; v_NR_DRX_LongCycleStartOffset := {ms40 := 39}; v_MAC_CellGroupConfig.drx_Config.setup.drx_LongCycleStartOffset := v_NR_DRX_LongCycleStartOffset; v_MAC_CellGroupConfig.drx_Config.setup.drx_SlotOffset := 0; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_CellGroupConfig.mac_CellGroupConfig := v_MAC_CellGroupConfig; // DPC config remains unchanged v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext, -, v_MeasConfig); f_TC_7_1_1_12_3_NR_Steps22_24(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // to configure SS to stop report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); // leaving UE test state 3 v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, v_DrxStartOffset); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def), cs_TimingInfo_NR(v_Timing)); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); };

After Change

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication var DRB_Identity v_DRBId; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template(value) ControlResourceSet v_ControlResourceSet; var UInt_Type v_LongDRXCycle; var UInt_Type v_DrxStartOffset; var template (value) DRX_Config.drx_onDurationTimer v_NR_DRX_OnDurationTimer; var template (value) DRX_Config.drx_LongCycleStartOffset v_NR_DRX_LongCycleStartOffset; var SubFrameTiming_Type v_Timing; var RSRP_Range v_RsrpThres; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (omit) MeasGapConfig v_MeasGapUE ; var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_NextOccasion; var SubFrameTiming_Type v_DrxActiveTiming; var template(value) SearchSpace v_SearchSpace; var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates; //Init Cell parameters f_NR5GC_Init(NR_1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId), -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); //@siclog "Steps 1-2" siclog@ //SS transmits RRCReconfiguration to configure specific DCP parameters. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) //Table 7.1.1.12.3.3.3-2 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.dcp_Config_r16.setup := cs_DCP_Config_r16_Def; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig))); //Table 7.1.1.12.3.3.3-4 v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := 1; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet} v_SearchSpace := f_NR_CellInfo_GetSearchSpaceConfigAtUE(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_SearchSpace.searchSpaceId := 6; v_SearchSpace.controlResourceSetId := 1; v_SearchSpace.searchSpaceType.common := { dci_Format0_0_AndFormat1_0 := omit, dci_Format2_0 := omit, dci_Format2_1 := omit, dci_Format2_2 := omit, dci_Format2_3 := omit } v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModList := {v_SearchSpace} v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_Dci_Format2_6} v_SpCellConfig.spCellConfigDedicated.tag_Id := 0; v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_DownlinkBWP.Pdcch.SearchSpaceExtArray := { {SearchSpaceExt := {R16 := cs_SearchSpaceExt_r16_Dci_Format2_6}}}; f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_NrofCandidates := cs_38508_NrofCandidates_Def; } else { v_NrofCandidates := cds_38508_NrofCandidates_FR2 } f_NR_SS_SearchSpaceExtConfig(nr_Cell1, -, v_NrofCandidates); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, -, -, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); v_LongDRXCycle := 10240; v_DrxStartOffset := 0; //@siclog "Steps 3-6" siclog@ f_TC_7_1_1_12_3_NR_Steps3_6(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); //@siclog "Steps 7-8" siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := true_; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); change_711123 f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog "Steps 9-11" siclog@ f_TC_7_1_1_12_3_NR_Steps9_11(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); //@siclog "Steps 12-13" siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_NR_DRX_OnDurationTimer := { milliSeconds := ms10 }; v_NR_DRX_LongCycleStartOffset := {ms20 := 0}; v_MAC_CellGroupConfig.drx_Config.setup := cs_NR_DRX_Config_Def( v_NR_DRX_OnDurationTimer, ms6, 56, 56, sl320, sl320, v_NR_DRX_LongCycleStartOffset, -, 0); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_CellGroupConfig.mac_CellGroupConfig := v_MAC_CellGroupConfig; v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := omit; v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_Offset_r16 := 40; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); v_LongDRXCycle := 20; //@siclog "Steps 14-19" siclog@ f_TC_7_1_1_12_3_NR_Steps14_19(v_LongDRXCycle, v_DrxStartOffset, v_DRBId, valueof(v_CellGroupConfig.mac_CellGroupConfig.drx_Config.setup.drx_HARQ_RTT_TimerDL)); //@siclog "Steps 20-24" siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_RsrpThres := 2; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, omit, omit), cs_MeasObjectId2(nr_Cell3, omit, omit)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasGapUE := cs_38508_MeasGapConfig_UE(cs_NR_GapConfig(34, ms6, ms40, ms0)); //MOVED f_NR_SS_MeasGapCtrlConfig(nr_Cell1, cs_NR_MeasGapCtrl_Config(v_MeasGapUE)); v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_RsrpThres, infinity_, -, -, cs_NR_MeasReportQuantity_Rsrp)}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE); //@sic R5-192854, R5-199097 sic@ if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } v_MAC_CellGroupConfig.drx_Config.setup.drx_onDurationTimer := v_NR_DRX_OnDurationTimer; v_MAC_CellGroupConfig.drx_Config.setup.drx_InactivityTimer := ms5; v_NR_DRX_LongCycleStartOffset := {ms40 := 39}; v_MAC_CellGroupConfig.drx_Config.setup.drx_LongCycleStartOffset := v_NR_DRX_LongCycleStartOffset; v_MAC_CellGroupConfig.drx_Config.setup.drx_SlotOffset := 0; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_CellGroupConfig.mac_CellGroupConfig := v_MAC_CellGroupConfig; // DPC config remains unchanged v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_Offset_r16 := 32; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //MOVED f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext, -, v_MeasConfig); v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_DrxActiveTiming, v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7);//KS R5sxxx change_711123 f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),{cs_NR_DciFormat_2_6_Block_Type('1'B)}));//KS R5sxxx change_711123 v_Timing := f_SubFrameTiming_AddMilliSeconds(v_NextOccasion, 5); // add 5ms SRB.send(cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_NextOccasion), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, v_MeasConfig, v_V1530Ext)), -, -, -, cs_RlcBearerRouting_NR(nr_Cell1))); f_NR_SS_MeasGapCtrlConfig(nr_Cell1, cs_NR_MeasGapCtrl_Config(v_MeasGapUE), cs_TimingInfo_NR(v_Timing)); f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig))); //KS R5sxxx change_711123 f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); //@sic R5s190109 change 10 sic@ v_LongDRXCycle := 40; v_DrxStartOffset:= 39; f_TC_7_1_1_12_3_NR_Steps22_24(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // to configure SS to stop report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); // leaving UE test state 3 v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_DrxActiveTiming, v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def), cs_TimingInfo_NR(v_NextOccasion)); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); };

## Change 2

| Function name | f_TC_7_1_1_12_3_NR_Steps3_6 |
| --- | --- |
| Reason for change | - TTCN may receive unexpected HARQ ACK indication caused by MAC PDU TA during the test steps. - Since the drx_LongCycle is 10240ms, a 5s watchdog is too short - WakeUp signal and PDSCH may be sent at the wrong time, additional parameters needed for the calculation, see change 6 - TTCN will receive unexpected HARQ ACK indication during the follow-up rrcReconfiguration pocedure because indications are still enabled - for the test purpose, DCI_2_6 with one block is enough. It is not needed to configure 6 blocks |
| Summary of change | - Stop TA transmission using cs_NR_UplinkTimeAlignment_Stop - Get duration for watchdog depend on the actual PDCCH occasion - Additional parameters for WUS timing calculation, see change 6 - enable/disable UL HARQ indications in each sub function - replace cs_NR_DciFormat_2_6_Block_List_6Items with {cs_NR_DciFormat_2_6_Block_Type |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_12_3_NR_Steps3_6(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 5.0; // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@siclog Step 3 siclog@ //Wait 1280ms to ensure UE is out DRX active time f_Delay(1.28); //@siclog Steps 3A siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. v_WakeUpTiming := f_NR_GetWakeUpSingalTiming (nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming), cs_NR_DciFormat_2_6_Block_List_6Items)); //off //@siclog Steps 3B siclog@ //In a PDCCH occasion the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(0, p_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_WakeUpTiming), v_MAC_PDUListDL)); t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3B"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } //@siclog Steps 4 siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. v_WakeUpTiming := f_NR_GetWakeUpSingalTiming (nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming), cs_NR_DciFormat_2_6_Block_List_6Items('1'B))); //@siclog Steps 5 siclog@ // In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_WakeUpTiming), v_MAC_PDUListDL)); //@siclog Steps 6 siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }

After Change

function f_TC_7_1_1_12_3_NR_Steps3_6(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 5.0; var SubFrameTiming_Type v_NextOccasion; var SubFrameTiming_Type v_DrxActiveTiming; var integer v_Duration; f_NR_ULGrantConfiguration_Start(nr_Cell1,-, -, -, cs_NR_UplinkTimeAlignment_Stop); // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@siclog "Step 3" siclog@ //Wait 1280ms to ensure UE is out DRX active time f_Delay(1.28); //@siclog "Steps 3A" siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_DrxActiveTiming, v_NextOccasion, nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); v_Duration := f_SubFrameTiming_Duration(f_NR_GetCurrentTiming(nr_Cell1), v_NextOccasion); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming), {cs_NR_DciFormat_2_6_Block_Type})); //off //@siclog "Steps 3B" siclog@ //In a PDCCH occasion the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(0, p_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_NextOccasion), v_MAC_PDUListDL)); t_Watchdog.start(0.001 * int2float(v_Duration) + 1.0); alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3B"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } //@siclog "Steps 4" siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_DrxActiveTiming, v_NextOccasion, nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),{cs_NR_DciFormat_2_6_Block_Type('1'B)})); //@siclog "Steps 5" siclog@ // In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_NextOccasion), v_MAC_PDUListDL)); //@siclog "Steps 6" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }

## Change 3

| Function name | f_TC_7_1_1_12_3_NR_Steps9_11 |
| --- | --- |
| Reason for change | - WakeUp signal and PDSCH may be sent at the wrong time, additional parameters needed for the calculation, see change 6 - TTCN will receive unexpected HARQ ACK indication during the follow-up rrcReconfiguration pocedure because indications are still enabled |
| Summary of change | - Additional parameters for WUS timing calculation, see change 6 - enable/disable UL HARQ indications in each sub function |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_12_3_NR_Steps9_11(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_Timing; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; //@siclog Step 9 siclog@ //Wait 1280ms to ensure UE is out DRX active time f_Delay(1.28); //@siclog Steps 10 siclog@ //In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(2, p_DRBId); v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL)); //@siclog Steps 11 siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); }

After Change

function f_TC_7_1_1_12_3_NR_Steps9_11(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_Timing; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var NR_SchedulingParameters_Type v_SchedulingParameters := fl_NR_SchedulingParameters_DL(); //to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@siclog "Step 9" siclog@ //Wait 1280ms to ensure UE is out DRX active time f_Delay(1.28); //@siclog "Steps 10" siclog@ //In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(2, p_DRBId); v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); v_Timing := fl_NR_NextPdcchOcassion(nr_Cell1, v_Timing.HSFN, v_Timing.SFN, v_Timing.Subframe.Number, v_SchedulingParameters); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL)); //@siclog "Steps 11" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }

## Change 4

| Function name | f_TC_7_1_1_12_3_NR_Steps14_19() |
| --- | --- |
| Reason for change | - TTCN will receive unexpected HARQ ACK indication during the follow-up rrcReconfiguration pocedure because indications are still enabled - for the test purpose, DCI_2_6 with one block is enough. It is not needed to configure 6 blocks - The parameter p_DRX_HARQ_RTT_TimerDL will be measured in symbols (not slots) - WakeUp signal and PDSCH may be sent at the wrong time, additional parameters needed for the calculation, see change 6 - According to the test purpose, below conditions should be satisfied - Step 15 and step 16 should happen in the same cycle - Step 17A and step 18 should happen in the same cycle - Duration between step 15 and step17A = N * p_LongDRXCycle - Duration between step 16 and step 17A > v_NR_SlotOffsetK1 + p_DRX_HARQ_RTT_TimerDL - v_NR_SlotOffsetK1 + p_DRX_HARQ_RTT_TimerDL <= duration between step 16 and step 18 <= v_NR_SlotOffsetK1 + p_DRX_HARQ_RTT_TimerDL + drx-RetransmissionTimerDL The current implemation does not meet these requirements - The default timing advance of 2 cycles ahead is too short for all the SS reconfigurations |
| Summary of change | - enable/disable UL HARQ indications in each sub function - replace cs_NR_DciFormat_2_6_Block_List_6Items with {cs_NR_DciFormat_2_6_Block_Type} - Convert p_DRX_HARQ_RTT_TimerDL from symbols to slots - Additional parameters for WUS timing calculation, see change 6 - Modify the timing for each step - Add additional time advance |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_12_3_NR_Steps14_19(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId, DRX_Config.drx_HARQ_RTT_TimerDL p_DRX_HARQ_RTT_TimerDL) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_Timing; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var integer v_SlotsInSubframe; var integer v_NR_SlotOffsetK1; var integer v_DRC_onDurationTimer; timer t_Watchdog := 5.0; //@siclog Step 14 siclog@ //Wait 20ms to ensure UE is out DRX active time. f_Delay(0.02); //@siclog Step 15 siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. v_WakeUpTiming := f_NR_GetWakeUpSingalTiming (nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items('1'B))); //@siclog Step 16 siclog@ //In the last PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of an invalid DL MAC PDU on the PDCCH. f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Erroneous)); v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); // Last PDCCH occasion while On Duration Timer is running. v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1) ; v_DRC_onDurationTimer := 10; //Table 7.1.1.12.3.3.3-6 v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing, (v_DRC_onDurationTimer * v_SlotsInSubframe) - 1); // - the last slot during onDurationTimer v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing); v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(3, p_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); //@siclog "Step 17A" siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. v_WakeUpTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, p_LongDRXCycle); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items)); //@siclog "Step 18" siclog@ //In the PDCCH occasion when the next Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_NR_SlotOffsetK1 := f_NR_GetSlotOffsetK1(nr_Cell1); v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing ,v_NR_SlotOffsetK1 + p_DRX_HARQ_RTT_TimerDL - 1); v_Timing := f_NR_NextDL_Slot(nr_Cell1,v_Timing); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_Timing))); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); t_Watchdog.start; //@siclog "Step 17" siclog@ // The UE transmit a HARQ NACK for the DL MAC PDU in Step 16. SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)); //@siclog "Step 19" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 18? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); t_Watchdog.stop; }

After Change

function f_TC_7_1_1_12_3_NR_Steps14_19(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId, DRX_Config.drx_HARQ_RTT_TimerDL p_DRX_HARQ_RTT_TimerDL) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_Timing; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var integer v_SlotsInSubframe; var integer v_NR_SlotOffsetK1; var integer v_DRC_onDurationTimer; var SubFrameTiming_Type v_NextOccasion; var SubFrameTiming_Type v_DrxActiveTiming; timer t_Watchdog := 5.0; var integer v_AddSlots := 0; var integer v_NuOfLongDRXCycle := 2; v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1) ; v_NR_SlotOffsetK1 := f_NR_GetSlotOffsetK1(nr_Cell1); v_AddSlots := v_NR_SlotOffsetK1 + p_DRX_HARQ_RTT_TimerDL/14; if(v_AddSlots < v_NuOfLongDRXCycle * p_LongDRXCycle * v_SlotsInSubframe) { v_AddSlots := v_NuOfLongDRXCycle * p_LongDRXCycle * v_SlotsInSubframe; } else { v_NuOfLongDRXCycle := v_AddSlots / (p_LongDRXCycle * v_SlotsInSubframe); if(v_AddSlots > v_NuOfLongDRXCycle * p_LongDRXCycle * v_SlotsInSubframe){ v_NuOfLongDRXCycle := v_NuOfLongDRXCycle + 1; } v_AddSlots := v_NuOfLongDRXCycle * p_LongDRXCycle * v_SlotsInSubframe; } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@siclog "Step 14" siclog@ //Wait 20ms to ensure UE is out DRX active time. f_Delay(0.02); //@siclog "Step 15" siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_DrxActiveTiming, v_NextOccasion, nr_Cell1, p_LongDRXCycle, p_DrxStartOffset, 13); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming), {cs_NR_DciFormat_2_6_Block_Type('1'B)})); //@siclog "Step 16" siclog@ //In the last PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of an invalid DL MAC PDU on the PDCCH. f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Erroneous)); v_Timing := v_DrxActiveTiming; // Last PDCCH occasion while On Duration Timer is running. v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1) ; v_DRC_onDurationTimer := 10; //Table 7.1.1.12.3.3.3-6 v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing, (v_DRC_onDurationTimer * v_SlotsInSubframe) - 1); // - the last slot during onDurationTimer v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing); v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(3, p_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); //@siclog "Step 17A" siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. v_WakeUpTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, (p_LongDRXCycle * v_NuOfLongDRXCycle)); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),{cs_NR_DciFormat_2_6_Block_Type})); //@siclog "Step 18" siclog@ //In the PDCCH occasion when the next Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing ,v_AddSlots-1); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_WakeUpTiming))); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR( v_WakeUpTiming), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); t_Watchdog.start; //@siclog "Step 17" siclog@ // The UE transmit a HARQ NACK for the DL MAC PDU in Step 16. SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)); //@siclog "Step 19" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 18? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); t_Watchdog.stop; f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }

## Change 5

| Function name | f_TC_7_1_1_12_3_NR_Steps22_24 |
| --- | --- |
| Reason for change | - TTCN will receive unexpected HARQ ACK indication during the follow-up rrcReconfiguration pocedure because indications are still enabled - for the test purpose, DCI_2_6 with one block is enough. It is not needed to configure 6 blocks - WakeUp signal and PDSCH may be sent at the wrong time, additional parameters needed for the calculation, see change 6 - For these steps, the p_LongDRXCycle is 20ms, the default 2 cycles ahead is too short for the SS overhead. |
| Summary of change | - Enable/disable UL HARQ indications in each sub function - Replace cs_NR_DciFormat_2_6_Block_List_6Items with {cs_NR_DciFormat_2_6_Block_Type} - Additional parameters for WUS timing calculation, see change 6 - Pass parameter to f_NR_GetWakeUpSignalTiming for additional 3 cycles of advance time |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_12_3_NR_Steps22_24(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_Timing; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; //@siclog Step 22 siclog@ //Wait 10ms to ensure UE is out DRX active time. f_Delay(0.01); //@siclog Step 22A siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. v_WakeUpTiming := f_NR_GetWakeUpSingalTiming (nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items)); //@siclog Step 23 siclog@ //In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(4, p_DRBId); v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, p_LongDRXCycle, p_DrxStartOffset); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL)); //@siclog Step 24 siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 23? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24"); }

After Change

function f_TC_7_1_1_12_3_NR_Steps22_24(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_NextOccasion; var SubFrameTiming_Type v_DrxActiveTiming; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@siclog "Step 22" siclog@ //Wait 10ms to ensure UE is out DRX active time. f_Delay(0.01); //@siclog "Step 22A" siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_DrxActiveTiming, v_NextOccasion, nr_Cell1, p_LongDRXCycle, p_DrxStartOffset, 3); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming), {cs_NR_DciFormat_2_6_Block_Type('0'B)})); //@siclog "Step 23" siclog@ //In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(4, p_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_NextOccasion), v_MAC_PDUListDL)); //@siclog "Step 24" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 23? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }

## Change 6

| Function name | f_NR_GetWakeUpSingalTiming() |
| --- | --- |
| Reason for change | - Based on the test purpose of 7.1.1.12.3, the PDCCH DCI_2_6 and the following MAC PDU should be sent in the same cycle. The current implementation does not guarantee this requirement. PDCCH DCI_2_6 and MAC PDU should be sent at different TTI, PDCCH DCI_2_6 is sent before the start of the active time. MAC PDU should be sent during the active time of DRX The time returned from f_NR_GetNextSendOccasion_7_1_1_5_X() is the start point of the active time of DRX, but it is not always a valid PDCCH occasion for PDSCH(C_RNTI），the later one also depends on the scheduling parameters - For short DRX cycle, the default p_DrxCycleOffset=0 for f_NR_GetNextSendOccasion_7_1_1_5_X is too little for the SS overhead - The 2nd parameter of f_SubFrameTiming_SubtractMilliSeconds() is in unit ms, but the current implementation assumes the unit is slot, also v_PS_Offset_Slots is calculated incorrectly |
| Summary of change | - Change the function to return 3 timings: v_WakeUpTiming time used to sent DCI_2_6 p_DrxActiveTiming start point the active time of DRX cycle (starting of drx- onDurationTimer) p_NextOccasion the 1st PDCCH occasion for MAC PDU - add one input parameter for the cycle ahead to give SS enough time - change the 2nd parameter in call to f_SubFrameTiming_SubtractMilliSeconds() Note : Function name also corrected |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_NR_GetWakeUpSingalTiming(NR_CellId_Type p_NR_CellId, UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset) runs on NR_BASE_PTC return SubFrameTiming_Type { var SubFrameTiming_Type v_Timing; var PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var float v_PS_Offset_ms; var integer v_PS_Offset_Slots; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_NR_CellId); v_PS_Offset_ms := int2float(v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_Offset_r16) * 0.125; select (v_SCS){ case (kHz15) {v_PS_Offset_Slots := float2int (v_PS_Offset_ms);} case (kHz30) {v_PS_Offset_Slots := float2int (v_PS_Offset_ms/2.0);} case (kHz60) {v_PS_Offset_Slots := float2int (v_PS_Offset_ms/4.0);} case (kHz120){v_PS_Offset_Slots := float2int (v_PS_Offset_ms/8.0);} case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSpacing");} } v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(p_NR_CellId, p_LongDRXCycle, p_DrxStartOffset); v_Timing := f_SubFrameTiming_SubstractMilliSeconds(v_Timing, v_PS_Offset_Slots); return v_Timing; }

After Change

function f_NR_GetWakeUpSignalTiming(inout SubFrameTiming_Type p_DrxActiveTiming, inout SubFrameTiming_Type p_NextOccasion, NR_CellId_Type p_NR_CellId, UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, integer p_CycleAhead := 0) runs on NR_BASE_PTC return SubFrameTiming_Type { var SubFrameTiming_Type v_WakeUpTiming; var PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var float v_PS_Offset_ms; var integer v_PS_Offset_Slots; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var NR_SchedulingParameters_Type v_SchedulingParameters := fl_NR_SchedulingParameters_DL; v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_NR_CellId); v_PS_Offset_ms := int2float(v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_Offset_r16) * 0.125; /* REMOVED select (v_SCS){ case (kHz15) {v_PS_Offset_Slots := float2int (v_PS_Offset_ms);} case (kHz30) {v_PS_Offset_Slots := float2int (v_PS_Offset_ms/2.0);} case (kHz60) {v_PS_Offset_Slots := float2int (v_PS_Offset_ms/4.0);} case (kHz120){v_PS_Offset_Slots := float2int (v_PS_Offset_ms/8.0);} case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSpacing");} }*/ p_DrxActiveTiming := f_NR_GetNextSendOccasion_7_1_1_5_X(p_NR_CellId, p_LongDRXCycle, p_DrxStartOffset, -, p_CycleAhead); v_WakeUpTiming := f_SubFrameTiming_SubtractMilliSeconds(p_DrxActiveTiming, float2int(v_PS_Offset_ms)); p_NextOccasion := fl_NR_NextPdcchOcassion(p_NR_CellId, p_DrxActiveTiming.HSFN, p_DrxActiveTiming.SFN, p_DrxActiveTiming.Subframe.Number, v_SchedulingParameters); return v_WakeUpTiming; }

## Change 7

| Function name | f_SubFrameTiming_SubstractMilliSeconds() |
| --- | --- |
| Reason for change | Incorrect timing returned |
| Summary of change | Rewrite this function Note : Function name also corrected |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_SubFrameTiming_SubstractMilliSeconds(SubFrameTiming_Type p_TimingInfo, UInt_Type p_MilliSeconds) return SubFrameTiming_Type { var SubFrameTiming_Type v_TimingInfo; var UInt_Type v_Diff; if (not (ischosen(p_TimingInfo.Subframe.Number) and ischosen(p_TimingInfo.SFN.Number) and ischosen(p_TimingInfo.HSFN.Number))) { FatalError(__FILE__, __LINE__, "Invalid SubFrameTiming"); } v_Diff := p_TimingInfo.Subframe.Number - p_MilliSeconds; if (v_Diff > 0) { v_TimingInfo.Subframe.Number := p_TimingInfo.Subframe.Number - p_MilliSeconds; } else { //in previous SFN v_TimingInfo.Subframe.Number := 9 - v_Diff; v_Diff := v_TimingInfo.SFN.Number - 1; if (v_Diff > 0) { v_TimingInfo.SFN.Number := p_TimingInfo.SFN.Number -1; } else {//in previous HSFN v_TimingInfo.SFN.Number := 1023; v_Diff := v_TimingInfo.HSFN.Number - 1; if (v_Diff > 0) { v_TimingInfo.HSFN.Number := v_TimingInfo.HSFN.Number -1; } else { v_TimingInfo.HSFN.Number := 1023; } } } v_TimingInfo.Slot := p_TimingInfo.Slot; return v_TimingInfo; }

After Change

function f_SubFrameTiming_SubtractMilliSeconds(SubFrameTiming_Type p_TimingInfo, UInt_Type p_MilliSeconds) return SubFrameTiming_Type { var SubFrameTiming_Type v_TimingInfo; var UInt_Type v_Diff; var integer v_SFN_Previous; var integer v_Add_MilliSeconds; if (not (ischosen(p_TimingInfo.Subframe.Number) and ischosen(p_TimingInfo.SFN.Number) and ischosen(p_TimingInfo.HSFN.Number))) { FatalError(__FILE__, __LINE__, "Invalid SubFrameTiming"); } v_Diff := p_TimingInfo.Subframe.Number - p_MilliSeconds; if (v_Diff >= 0) //in same SFN { v_TimingInfo.Subframe.Number := p_TimingInfo.Subframe.Number - p_MilliSeconds; v_TimingInfo.SFN.Number := p_TimingInfo.SFN.Number; v_TimingInfo.HSFN.Number := p_TimingInfo.HSFN.Number; } else { //in previous n SFN v_SFN_Previous := (p_MilliSeconds + 9) / 10; v_Add_MilliSeconds := v_SFN_Previous * 10 - p_MilliSeconds; //decrease v_SFN_Previous whole frame firstly, which is v_Add_MilliSeconds more than p_MilliSeconds v_TimingInfo.Subframe.Number := p_TimingInfo.Subframe.Number v_Diff := p_TimingInfo.SFN.Number - v_SFN_Previous; if (v_Diff >= 0) //in same HSFN { v_TimingInfo.SFN.Number := p_TimingInfo.SFN.Number - v_SFN_Previous; v_TimingInfo.HSFN.Number := p_TimingInfo.HSFN.Number; } else { //in previous HSFN v_TimingInfo.SFN.Number := (p_TimingInfo.SFN.Number + 1023 - v_SFN_Previous) mod 1024; v_Diff := p_TimingInfo.HSFN.Number - 1; if (v_Diff >= 0) { v_TimingInfo.HSFN.Number := p_TimingInfo.HSFN.Number -1; } else { v_TimingInfo.HSFN.Number := 1023; } } //then add v_Add_MilliSeconds if( v_Add_MilliSeconds > 0) { f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, v_Add_MilliSeconds); } } v_TimingInfo.Slot := p_TimingInfo.Slot; return v_TimingInfo; }

## Change 8

| Function name | f_NR_CellInfo_SetPhysicalCellGroupConfig() |
| --- | --- |
| Reason for change | PhysicalCellGroupConfig is not saved in current implementation |
| Summary of change | Save PhysicalCellGroupConfig |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_NR_CellInfo_SetPhysicalCellGroupConfig(NR_CellId_Type p_NR_CellId, template (value) PhysicalCellGroupConfig p_PhysicalCellGroupConfig) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (ischosen(v_NR_CellInfo.ServingCellConfig.SpCell)) { v_NR_CellInfo.ServingCellConfig.SpCell.CellGroupConfig.PhysicalCellGroupConfig.R15 := valueof(p_PhysicalCellGroupConfig) ; } else { FatalError(__FILE__, __LINE__, "No PhysicalCellGroupConfig not found"); } };

After Change

function f_NR_CellInfo_SetPhysicalCellGroupConfig(NR_CellId_Type p_NR_CellId, template (value) PhysicalCellGroupConfig p_PhysicalCellGroupConfig) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (ischosen(v_NR_CellInfo.ServingCellConfig.SpCell)) { v_NR_CellInfo.ServingCellConfig.SpCell.CellGroupConfig.PhysicalCellGroupConfig.R15 := valueof(p_PhysicalCellGroupConfig) ; } else { FatalError(__FILE__, __LINE__, "No PhysicalCellGroupConfig not found"); } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); };

## Change 9

| Function name | NR_BWP_PDCCH_Configuration_Type |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | Addition of SearchSpaceExtArray under NR_BWP_PDCCH_Configuration_Type, for R16 search space configuration |
| TTCN module | NR_ASP_TypeDefs |
| MCC160 Comment |  |

Before Change

type record NR_BWP_PDCCH_Configuration_Type { /* PDCCH configuration at the SS for specific BWP; NOTE: There are no fields for PDCCH-Config's "downlinkPreemption", "slotFormatIndicator", "tpc-PUSCH" and "tpc-PUCCH": This information is related to triggering DCI formats 2_X an shall be configured there (NR_DCI_Trigger_Type) according to test case requirements */ NR_BWP_SearchSpaceList_Type SearchSpaceArray optional, NR_BWP_CoresetList_Type CoresetArray optional, NR_PDCCH_EPREs_Type RelativeTxPower optional, integer Coreset0_OffsetRBs optional /* Number of Offset RBs as specified in 38.213 clause 13. Applied to CORESET#0 only */ };

After Change

type record NR_BWP_PDCCH_Configuration_Type { /* PDCCH configuration at the SS for specific BWP; NOTE: There are no fields for PDCCH-Config's "downlinkPreemption", "slotFormatIndicator", "tpc-PUSCH" and "tpc-PUCCH": This information is related to triggering DCI formats 2_X an shall be configured there (NR_DCI_Trigger_Type) according to test case requirements */ NR_BWP_SearchSpaceList_Type SearchSpaceArray optional, NR_BWP_CoresetList_Type CoresetArray optional, NR_PDCCH_EPREs_Type RelativeTxPower optional, integer Coreset0_OffsetRBs optional, /* Number of Offset RBs as specified in 38.213 clause 13. Applied to CORESET#0 only */ NR_BWP_SearchSpaceExtList_Type SearchSpaceExtArray optional };

## Change 10

| Function name | New type NR_ASN1_SearchSpaceExt_Type |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | New wrapper type |
| TTCN module |  |
| MCC160 Comment |  |

New type

type union NR_ASN1_SearchSpaceExt_Type { SearchSpaceExt_r16 R16 };

## Change 11

| Function name | New type NR_BWP_SearchSpaceExtList_Type |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | New wrapper type |
| TTCN module |  |
| MCC160 Comment |  |

New type

type record of NR_BWP_SearchSpaceExt_Type NR_BWP_SearchSpaceExtList_Type;

## Change 12

| Function name | New type NR_BWP_SearchSpaceExt_Type |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | New wrapper type |
| TTCN module |  |
| MCC160 Comment |  |

New type

type record NR_BWP_SearchSpaceExt_Type { NR_ASN1_SearchSpaceExt_Type SearchSpaceExt };

## Change 13

| Function name | cs_38508_DownlinkBWP_Initial |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | Initialise SearchSpaceExtArray, see change 9 |
| TTCN module | NR_ASP_Templates_BWP |
| MCC160 Comment |  |

Before Change

template (value) NR_DownlinkBWP_Type cs_38508_DownlinkBWP_Initial(SubcarrierSpacing p_SubcarrierSpacing, PhysCellId p_PhysCellId, BWP.locationAndBandwidth p_LocationAndBandwidth, template (value) DMRS_DownlinkConfig p_DMRS_DownlinkConfig, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates, template (value) PDSCH_TimeDomainResourceAllocationList p_PDSCH_TimeDomainResourceAllocationList, template (omit) PDSCH_Config.mcs_Table p_Mcs_Table := omit, template (omit) integer p_Coreset0_OffsetRBs, SSB_Index p_SSB_Index) := { // Acc to 38.508-1 Table 4.6.3-9: BWP-Downlink // @sic R5-190670: p_Mcs_Table:=omit per default sic@ /* @sic R5-190786: p_PhysCellId sic@ */ /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ Id := tsc_NR_BWP_Id, // Initial BWP BWP := { R15 := cs_38508_BWP (p_SubcarrierSpacing, p_LocationAndBandwidth) }, Pdcch := { SearchSpaceArray := { cs_NR_BWP_SearchSpaceConfig_SearchSpaceZero(p_SubcarrierSpacing), cs_NR_BWP_SearchSpaceConfig_CSS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_USS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_SISS(p_NrofCandidates) // @sic R5-191187 sic@ }, CoresetArray := { cs_NR_ASN1_ControlResourceSet(cs_NR_ControlResourceSetZeroAtSS(p_PhysCellId)), // CORESET#0 according to 38.211 clause 7.3.2.2 with shift index being the physical layer cell id @sic R5-190786, RP-182523 sic@ */ cs_NR_ASN1_ControlResourceSet(cs_38508_ControlResourceSet()) // default CORESET as according to 38.508-1 Table 4.6.3-17 }, RelativeTxPower := cs_NR_PDCCH_EPREs_Def, Coreset0_OffsetRBs := p_Coreset0_OffsetRBs //@sic R5-201020 sic@ }, Pdsch := { ConfigCommon := { R15 := cs_38508_PDSCH_ConfigCommon(p_PDSCH_TimeDomainResourceAllocationList) }, ConfigDedicated := { R15 := cs_38508_PDSCH_Config(p_DMRS_DownlinkConfig, p_Mcs_Table, p_SSB_Index) }, RelativeTxPower := cs_NR_PDSCH_EPREs_Def }, Sps := omit }； }

After Change

template (value) NR_DownlinkBWP_Type cs_38508_DownlinkBWP_Initial(SubcarrierSpacing p_SubcarrierSpacing, PhysCellId p_PhysCellId, BWP.locationAndBandwidth p_LocationAndBandwidth, template (value) DMRS_DownlinkConfig p_DMRS_DownlinkConfig, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates, template (value) PDSCH_TimeDomainResourceAllocationList p_PDSCH_TimeDomainResourceAllocationList, template (omit) PDSCH_Config.mcs_Table p_Mcs_Table := omit, template (omit) integer p_Coreset0_OffsetRBs, SSB_Index p_SSB_Index) := { // Acc to 38.508-1 Table 4.6.3-9: BWP-Downlink // @sic R5-190670: p_Mcs_Table:=omit per default sic@ /* @sic R5-190786: p_PhysCellId sic@ */ /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ Id := tsc_NR_BWP_Id, // Initial BWP BWP := { R15 := cs_38508_BWP (p_SubcarrierSpacing, p_LocationAndBandwidth) }, Pdcch := { SearchSpaceArray := { cs_NR_BWP_SearchSpaceConfig_SearchSpaceZero(p_SubcarrierSpacing), cs_NR_BWP_SearchSpaceConfig_CSS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_USS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_SISS(p_NrofCandidates) // @sic R5-191187 sic@ }, CoresetArray := { cs_NR_ASN1_ControlResourceSet(cs_NR_ControlResourceSetZeroAtSS(p_PhysCellId)), // CORESET#0 according to 38.211 clause 7.3.2.2 with shift index being the physical layer cell id @sic R5-190786, RP-182523 sic@ */ cs_NR_ASN1_ControlResourceSet(cs_38508_ControlResourceSet()) // default CORESET as according to 38.508-1 Table 4.6.3-17 }, RelativeTxPower := cs_NR_PDCCH_EPREs_Def, Coreset0_OffsetRBs := p_Coreset0_OffsetRBs, //@sic R5-201020 sic@ SearchSpaceExtArray := omit }, Pdsch := { ConfigCommon := { R15 := cs_38508_PDSCH_ConfigCommon(p_PDSCH_TimeDomainResourceAllocationList) }, ConfigDedicated := { R15 := cs_38508_PDSCH_Config(p_DMRS_DownlinkConfig, p_Mcs_Table, p_SSB_Index) }, RelativeTxPower := cs_NR_PDSCH_EPREs_Def }, Sps := omit }; }

## Change 14

| Function name | New template cs_NR_SearchSpaceTypeAndPriorityList_CSS_DCI_2_6 |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | New wrapper template |
| TTCN module | NR_ASP_Templates_BWP |
| MCC160 Comment |  |

New template

template (value) NR_SearchSpaceTypeAndPriorityList_Type cs_NR_SearchSpaceTypeAndPriorityList_CSS_DCI_2_6 := { tsc_NR_SearchSpaceTypeAndPriority_CssType3 };

## Change 15

| Function name | New template cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6 |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | New template for DCI 2_6 confriguration |
| TTCN module | NR_ASP_Templates_BWP |
| MCC160 Comment |  |

New template

template (value) NR_BWP_SearchSpaceConfig_Type cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates, SearchSpaceId p_SearchSpaceId := 6, ControlResourceSetId p_ControlResourceSetId := tsc_NR_ControlResourceSetId) := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ //@sic R5-202677 R5-202561 sic@ TypeAndPriorityList := cs_NR_SearchSpaceTypeAndPriorityList_CSS_DCI_2_6, AggregationLevel := AggregationLevel4, // according to 38.523-3 clause 7.1.2.1.2 SearchSpaceConfigAtUE := { R15 := cs_38508_SearchSpace(p_SearchSpaceId, p_ControlResourceSetId, cs_NR_SearchSpace_SearchSpaceType_CSS_DCI_2_6, p_NrofCandidates) } };

## Change 16

| Function name | New template cs_NR_BWP_SearchSpaceExtConfig_DCI_2_6 |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | New template for DCI 2_6 configuration |
| TTCN module | NR_ASP_Templates_BWP |
| MCC160 Comment |  |

New template

template (value) NR_BWP_SearchSpaceExt_Type cs_NR_BWP_SearchSpaceExtConfig_DCI_2_6 := { SearchSpaceExt := { R16 := { controlResourceSetId_r16 := 1, searchSpaceType_r16 := { common_r16 := { dci_Format2_4_r16 := omit, dci_Format2_5_r16 := omit, dci_Format2_6_r16 := {} } }, searchSpaceGroupIdList_r16 := omit, freqMonitorLocations_r16 := omit } } };

## Change 17

| Function name | New template cs_NR_BWP_SearchSpaceExtConfig_DCI_2_6 |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | New template for SS configuration |
| TTCN module | NR_ASP_Templates_BWP |
| MCC160 Comment |  |

New template

template (value) NR_SYSTEM_CTRL_REQ cads_NR_SSExtConfig_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates ) modifies cas_NR_CellConfigAllOmit_REQ := { Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo), Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := omit, BWPs := omit, BWPs := { ActiveBWP := omit, BwpArray := { { Id := omit, BWP := omit, Pdcch := { SearchSpaceArray := {cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(p_NrofCandidates)}, CoresetArray := omit, RelativeTxPower := omit, Coreset0_OffsetRBs := omit, SearchSpaceExtArray := {cs_NR_BWP_SearchSpaceExtConfig_DCI_2_6} }, Pdsch := omit, Sps := omit } } }, CsiConfig := omit }, Uplink := omit } } } } };

## Change 18

| Function name | New function f_NR_SS_SearchSpaceExtConfig |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | New function for SS configuration |
| TTCN module | NR_ASP_Templates_BWP |
| MCC160 Comment |  |

New function in NR_ConfigurationSteps

function f_NR_SS_SearchSpaceExtConfig(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates) runs on NR_BASE_PTC { f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_SSExtConfig_REQ(p_NR_CellId, p_TimingInfo, p_NrofCandidates)); }

## Change 19

| Function name | New template cs_NR_SearchSpace_SearchSpaceType_CSS_DCI_2_6 |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | New function for SS configuration |
| TTCN module | NR_ASP_Templates_BWP |
| MCC160 Comment |  |

template (value) NR_SearchSpace_SearchSpaceType_Type cs_NR_SearchSpace_SearchSpaceType_CSS_DCI_2_6 := { common := { dci_Format0_0_AndFormat1_0 := omit, dci_Format2_0 := omit, dci_Format2_1 := omit, dci_Format2_2 := omit, dci_Format2_3 := omit } };

## Change 20

| Function name | cs_NR_DownlinkBWP |
| --- | --- |
| Reason for change | Current ASP definitions do not support SS configuration of DCI 2_6 through SearchSpaceListExt-r16 IE. |
| Summary of change | Initialise SearchSpaceExtArray, see change 9 |
| TTCN module | MAC_BWP_Common_NR |
| MCC160 Comment |  |

Before Change

template (value) NR_DownlinkBWP_Type cs_NR_DownlinkBWP(template (value) BWP_Id p_Id, template (value) BWP p_BWP, template (value) NR_BWP_SearchSpaceConfig_Type p_SearchSpaceConfig, template (value) ControlResourceSet p_ControlResourceSet, template (value) PDSCH_Config p_PDSCH_Config) := { Id := p_Id, BWP := { R15 := p_BWP }, Pdcch := { SearchSpaceArray := { p_SearchSpaceConfig }, CoresetArray := { cs_NR_ASN1_ControlResourceSet(p_ControlResourceSet) }, RelativeTxPower := cs_NR_PDCCH_EPREs_Def, Coreset0_OffsetRBs := omit // CORESET#0 only }, Pdsch := { ConfigCommon := omit, ConfigDedicated := { R15 := p_PDSCH_Config }, RelativeTxPower := cs_NR_PDSCH_EPREs_Def }, Sps := omit };

After Change

template (value) NR_DownlinkBWP_Type cs_NR_DownlinkBWP(template (value) BWP_Id p_Id, template (value) BWP p_BWP, template (value) NR_BWP_SearchSpaceConfig_Type p_SearchSpaceConfig, template (value) ControlResourceSet p_ControlResourceSet, template (value) PDSCH_Config p_PDSCH_Config) := { Id := p_Id, BWP := { R15 := p_BWP }, Pdcch := { SearchSpaceArray := { p_SearchSpaceConfig }, CoresetArray := { cs_NR_ASN1_ControlResourceSet(p_ControlResourceSet) }, RelativeTxPower := cs_NR_PDCCH_EPREs_Def, Coreset0_OffsetRBs := omit, // CORESET#0 only SearchSpaceExtArray := omit }, Pdsch := { ConfigCommon := omit, ConfigDedicated := { R15 := p_PDSCH_Config }, RelativeTxPower := cs_NR_PDSCH_EPREs_Def }, Sps := omit };

Change

# Branches executed

The test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## MediaTek MT6879

The MediaTek MT6879UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_12_3_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_1_12_3_PIXIT.xml

# References

| [1] | R5s230002 : NR5GC FR1 : Supporting information for addition of Rel-16 power saving test case 7.1.1.12.3 |
| --- | --- |