3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230006

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0888 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS WLAN test case G.17.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test, MTSI_WLAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The direction-tag for the current-status local segment shall be "sendrecv" according to Annex. G.17.2.4, INVITE (Step 1). In the current TTCN implementation, WLAN RAN type is not considered, and this would cause the testcase to fail since UE would not get the invite/prompt to add video to the ongoing voice call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If-else condition is used to control the precondition attributes for WLAN and non-WLAN RAN types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_IMS_BuildSDP_InitialOfferAddVideo_TX |
| --- | --- |
| Reason for change | The direction-tag for the current-status local segment shall be "sendrecv" according to Annex. G.17.2.4, INVITE (Step 1). In the current TTCN implementation, WLAN RAN type is not considered, and this would cause the testcase to fail since UE would not get the invite/prompt to add video to the ongoing voice call. |
| Summary of change | If-else condition is used to control the precondition attributes for WLAN and non-WLAN RAN types. |
| TTCN module | IMS_SDP_MessagesVideo.ttcn |
| MCC160 Comment |  |

Before change:

function f_IMS_BuildSDP_InitialOfferAddVideo_TX(template (value) IMS_SDP_MediaDescriptionRtpList_Type p_MediaDescriptionRtpList_Audio,

IMS_SDP_VideoCodecList_Type p_VideoCodecList := tsc_IMS_SDP_VideoCodecList_H264_only,

boolean p_WithPreconditions := true) runs on IMS_PTC return template (value) SDP_Message

{ /* @sic R5-213524, R5-213538, R5-213539, R5-213541: new common function replacing f_IMS_BuildSDP_MTCallAudioVideo_TX sic@ */

/* @sic R5s220939, R5-225423: p_PrevSdpFromUE removed sic@ */

/* @sic R5s221127: p_MediaDescriptionRtpList_Audio instead of p_AudioCodec sic@ */

var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpList_Video;

var template (omit) SDP_attribute_list v_PreconditionAttributes_Audio := omit;

var template (omit) SDP_attribute_list v_PreconditionAttributes_Video := omit;

if (p_WithPreconditions) {

v_PreconditionAttributes_Audio := cs_SDP_PreconditionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_optional);

v_PreconditionAttributes_Video := cs_SDP_PreconditionAttributes(c_none, c_none, c_mandatory, c_optional);

}

// ***** Media description video *****

v_MediaDescriptionRtpList_Video := fl_SDP_MediaAttributesVideoCodecTX(p_VideoCodecList[0]);   /* @sic R5-213524, R5-213538, R5-213539, R5-213541: fl_SDP_MediaAttributesVideoCodecTX sic@

so far there is only one video codec used in MT video call test cases */

return fl_IMS_BuildSDP_CommonOfferAudioVideo_TX(AddVideo,

p_MediaDescriptionRtpList_Audio,

v_MediaDescriptionRtpList_Video,

v_PreconditionAttributes_Audio,

v_PreconditionAttributes_Video);

}

After change:

function f_IMS_BuildSDP_InitialOfferAddVideo_TX(template (value) IMS_SDP_MediaDescriptionRtpList_Type p_MediaDescriptionRtpList_Audio,

IMS_SDP_VideoCodecList_Type p_VideoCodecList := tsc_IMS_SDP_VideoCodecList_H264_only,

boolean p_WithPreconditions := true) runs on IMS_PTC return template (value) SDP_Message

{ /* @sic R5-213524, R5-213538, R5-213539, R5-213541: new common function replacing f_IMS_BuildSDP_MTCallAudioVideo_TX sic@ */

/* @sic R5s220939, R5-225423: p_PrevSdpFromUE removed sic@ */

/* @sic R5s221127: p_MediaDescriptionRtpList_Audio instead of p_AudioCodec sic@ */

var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpList_Video;

var template (omit) SDP_attribute_list v_PreconditionAttributes_Audio := omit;

var template (omit) SDP_attribute_list v_PreconditionAttributes_Video := omit;

var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN();

if (p_WithPreconditions) {

if (v_IsWLAN) {               /* G.17.2.4 INVITE (Step 1) */

v_PreconditionAttributes_Video := cs_SDP_PreconditionAttributes(c_sendrecv, c_none, c_mandatory, c_optional);

} else {               		/* 17.2.4 INVITE (Step 1) */

v_PreconditionAttributes_Video := cs_SDP_PreconditionAttributes(c_none, c_none, c_mandatory, c_optional);

}

v_PreconditionAttributes_Audio := cs_SDP_PreconditionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_optional);

}

// ***** Media description video *****

v_MediaDescriptionRtpList_Video := fl_SDP_MediaAttributesVideoCodecTX(p_VideoCodecList[0]);   /* @sic R5-213524, R5-213538, R5-213539, R5-213541: fl_SDP_MediaAttributesVideoCodecTX sic@

so far there is only one video codec used in MT video call test cases */

return fl_IMS_BuildSDP_CommonOfferAudioVideo_TX(AddVideo,

p_MediaDescriptionRtpList_Audio,

v_MediaDescriptionRtpList_Video,

v_PreconditionAttributes_Audio,

v_PreconditionAttributes_Video);

}