3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230022

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0890 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS testcase 10.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TTCN implementation of this testcase (22wk49), after Step 28 -29, the SS sends BYE and the UE responds with 200OK. However, the PDU session established for emergency is not released after this, and the UE is then switched off in postamble by calling function f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); This results in the testcase failure, when some UEs send two PDU Session Release REQs, one for the Emergency PDU Session and second for the IMS PDU Session. TTCN only expects one PDU Session Release REQ in postamble, and this causes testcase failure. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService);, called f_IMS_CC_Postamble(IPCAN_EmergencyCall_OptionalPDNRelease); This releases the Emergency PDU Session prior to the Switch OFF and avoids the above described situation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function f_TC_10_6_NR5GC_IMS2	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_10_6_NR5GC_IMS2

| Function name | f_TC_10_6_NR5GC_IMS2 |
| --- | --- |
| Reason for change | In current TTCN implementation of this testcase (22wk49), after Step 28 -29, the SS sends BYE and the UE responds with 200OK. However, the PDU session established for emergency is not released after this, and the UE is then switched off in postamble by calling function f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); This results in the testcase failure, when some UEs send two PDU Session Release REQs, one for the Emergency PDU Session and second for the IMS PDU Session. TTCN only expects one PDU Session Release REQ in postamble, and this causes testcase failure. This needs to be addressed. |
| Summary of change | Instead of calling f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService);, called f_IMS_CC_Postamble(IPCAN_EmergencyCall_OptionalPDNRelease); This releases the Emergency PDU Session prior to the Switch OFF and avoids the above described situation. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); // @sic R5s220789 sic@ }

After Change:

<<SKIPPED CODE>> f_IMS_TestBody_Set(false); //WA#10_6 COMMENTED OUT f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); // @sic R5s220789 sic@ f_IMS_CC_Postamble(IPCAN_EmergencyCall_OptionalPDNRelease); //WA#10_6 }