3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230044

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2894 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of NR5GC UAC test case 11.3.5 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1 According to TS38.523-1,Step 10Fa1 – 10Fa8 for 11.3.5 refers to TS 38.508-1Table 4.5.2.2-4,the specific message contents for SERVICE REQUEST in Table 4.5.2.3-2 shows that the service type should be ‘0000’B. 2 The procedure in 38.508-1 cl. 4.5.4 already sends a Service Accept, so we don't need to send another one after calling f_NR5GC_RRC_ConnectedState3N_Steps5_8 . |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Correct the service type. 2 Remove the duplicate code. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1  Overview	2

2 Corrections required	2

2.1 Correction to the Function fl_TC_11_3_5_TestBody	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Ting Gao

gaoting@starpointcomm.com

- Corrections required

  - Correction to the Function fl_TC_11_3_5_TestBody

| Function name | fl_TC_11_3_5_TestBody |
| --- | --- |
| Reason for change | 1 According to TS38.523-1,Step 10Fa1 – 10Fa8 for 11.3.5 refers to TS 38.508-1Table 4.5.2.2-4,the specific message contents for SERVICE REQUEST in Table 4.5.2.3-2 shows that the service type should be ‘0000’B. 2 The procedure in 38.508-1 cl. 4.5.4 already sends a Service Accept, so we don't need to send another one after calling f_NR5GC_RRC_ConnectedState3N_Steps5_8 . |
| Summary of change | 1 Correct the service type. 2 Remove the duplicate code |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { ... //@siclog "Steps 10Fa1-10Fa8" siclog@ // @sic R5-223436 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @sic R5s221172 sic@ SRB.send (cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg()))); f_NR5GC_PDUSessionEstablishment (nr_Cell12,-,tsc_NR_RbId_SRB1); ... }

After Change:

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { ... //@siclog "Steps 10Fa1-10Fa8" siclog@ // @sic R5-223436 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @sic R5s221172 sic@ f_NR5GC_PDUSessionEstablishment (nr_Cell12,-,tsc_NR_RbId_SRB1); ... }