3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230062

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2903 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.10.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the current TTCN implementation, at Step 12 in the Registration Accept message, S-NSSAI with SST=3 is included in the Allowed NSSAI list, and the S-NSSAIs with SST=2 is included in the Configured NSSAI list. In case the UE performs the PDU Session Establishment at Step 14a, according to current TTCN implementation the S-NSSAI with SST=1 is included in the PDU Session Establishment Accept. This causes an anomaly at the UE side : the S-NSSAI present in Allowed list is NOT what is included in PDU Session establishment ACCEPT, and the UE releases the PDU Session causing testcase failure. This needs to be addressed. - In current TTCN implementation, if the Step 14a is executed then at Step 18 in the Configuration Update Command mesaage the S-NSSAI with SST=3 also needs to be included along with S-NSSAI with SST=1. If this is not done, then the UE would release the PDU Session established earlier with S-NSSAI (SST=3). This also needs to be addressed. - In current TTCN implementation if the Step 14a is included, then at Step 22, the UE would include S-NSSAIs with SST=1 and SST=3 in the Registration REQ message. This needs to be accommodated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - At Step 14a, it is proposed to include S-NSSAI with SST=3 in the PDU Session Establishment Accept message. A Prose CR will be raised for this. - At Step 18, if pc_noOf_PDUsSameConnection >0 then the Allowed NSSAI is updated to include S-NSSAIs with SST=1 and SST=3. A Prose CR will be raised for this. - At Step 22, if pc_noOf_PDUsSameConnection >0 then the Requested NSSAI is updated to include S-NSSAIs with SST=1 and SST=3. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function fl_TC_9_1_10_1_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_TC_9_1_10_1_TestBody

| Function name | fl_TC_9_1_10_1_TestBody |
| --- | --- |
| Reason for change | - In the current TTCN implementation, at Step 12 in the Registration Accept message, S-NSSAI with SST=3 is included in the Allowed NSSAI list, and the S-NSSAIs with SST=2 is included in the Configured NSSAI list. In case the UE performs the PDU Session Establishment at Step 14a, according to current TTCN implementation the S-NSSAI with SST=1 is included in the PDU Session Establishment Accept. This causes an anomaly at the UE side : the S-NSSAI present in Allowed list is NOT what is included in PDU Session establishment ACCEPT, and the UE releases the PDU Session causing testcase failure. This needs to be addressed. - In current TTCN implementation, if the Step 14a is executed then at Step 18 in the Configuration Update Command mesaage the S-NSSAI with SST=3 also needs to be included along with S-NSSAI with SST=1. If this is not done, then the UE would release the PDU Session established earlier with S-NSSAI (SST=3). This also needs to be addressed. - In current TTCN implementation if the Step 14a is included, then at Step 22, the UE would include S-NSSAIs with SST=1 and SST=3 in the Registration REQ message. This needs to be accommodated. |
| Summary of change | - At Step 14a, it is proposed to include S-NSSAI with SST=3 in the PDU Session Establishment Accept message. A Prose CR will be raised for this. - At Step 18, if pc_noOf_PDUsSameConnection >0 then the Allowed NSSAI is updated to include S-NSSAIs with SST=1 and SST=3. A Prose CR will be raised for this. - At Step 22, if pc_noOf_PDUsSameConnection >0 then the Requested NSSAI is updated to include S-NSSAIs with SST=1 and SST=3. A Prose CR will be raised for this. |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, v_SRB); v_SRB := tsc_NR_RbId_SRB2; // SRB2 is established during PDU Session Establishment, hence further NAS signalling will happen on SRB2 } v_SecurityParams := f_NR5GC_Security_Get(); v_SecurityParams := f_NG_EAP_AuthenticationInit(v_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)); // @sic R5s211584 sic@ v_EAP := cs_EAP_Message (f_NG_EAP_ChallengeReq (v_SecurityParams, v_PLMN), omit); //@siclog "Step 15" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMMAND(cs_S_NSSAI('01'O, '01'O, omit),v_EAP)))); //@siclog "Step 16" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMPLETE(cs_S_NSSAI('01'O, '01'O, omit),?)))) -> value v_ReceivedAsp; if (not(f_NG_EAP_SuccessfulAuthenticationResponse(v_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.network_Slice_Specific_Authentication_Complete.eapMessage.eapMsg))){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "EAP Authentication Failed"); } //@siclog "Step 17" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT(cs_S_NSSAI('01'O, '01'O, omit),cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success)), omit))))); //@siclog "Step 18" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('1'B, '1'B), -, -, v_AllowedNSSAI)))); // @sic R5-217939 sic@ // @sic R5s211584 sic@ //@siclog "Step 19" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); //SS releases the RRC connection. //@siclog "Step 20" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 21" siclog@ //Void //@siclog "Step 22" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message with NSSAA set to '1'B ? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility,-,-,-,-,cdr_NG_GMM_Cap_NSSAA,-,v_RequestedNSSAI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ) { //WA#9_1_10_1 COMMENTED OUT f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, v_SRB); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, v_SRB, -, -, -, -, -, -, -, cs_S_NSSAI_WithIEI('22'O, cs_S_NSSAI('01'O, '03'O, omit))); //WA#9_1_10_1 v_SRB := tsc_NR_RbId_SRB2; // SRB2 is established during PDU Session Establishment, hence further NAS signalling will happen on SRB2 } v_SecurityParams := f_NR5GC_Security_Get(); v_SecurityParams := f_NG_EAP_AuthenticationInit(v_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)); // @sic R5s211584 sic@ v_EAP := cs_EAP_Message (f_NG_EAP_ChallengeReq (v_SecurityParams, v_PLMN), omit); //@siclog "Step 15" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMMAND(cs_S_NSSAI('01'O, '01'O, omit),v_EAP)))); //@siclog "Step 16" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMPLETE(cs_S_NSSAI('01'O, '01'O, omit),?)))) -> value v_ReceivedAsp; if (not(f_NG_EAP_SuccessfulAuthenticationResponse(v_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.network_Slice_Specific_Authentication_Complete.eapMessage.eapMsg))){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "EAP Authentication Failed"); } //@siclog "Step 17" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT(cs_S_NSSAI('01'O, '01'O, omit),cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success)), omit))))); //@siclog "Step 18" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ v_AllowedNSSAI := { //WA#9_1_10_1 iei := '15'O, lvPart := { iel := '04'O, s_nssai := {cs_S_NSSAI('01'O, '03'O, omit), cs_S_NSSAI('01'O, '01'O, omit)} }}; } SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('1'B, '1'B), -, -, v_AllowedNSSAI)))); // @sic R5-217939 sic@ // @sic R5s211584 sic@ //@siclog "Step 19" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); //SS releases the RRC connection. //@siclog "Step 20" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 21" siclog@ //Void //@siclog "Step 22" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message with NSSAA set to '1'B ? if (pc_noOf_PDUsSameConnection > 0 ) { v_RequestedNSSAI := { //WA#9_1_10_1 iei := '2F'O, lvPart := { iel := '04'O, s_nssai := {cs_S_NSSAI('01'O, '03'O, omit), cs_S_NSSAI('01'O, '01'O, omit)} }}; } v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility,-,-,-,-,cdr_NG_GMM_Cap_NSSAA,-,v_RequestedNSSAI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); } <<SKIPPED CODE>>