3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230063

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3669 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TTCN CR “ R5s210502” was used for resolve the issue that RRCConnectionSetupComplete_r3_add_ext_IEs uninitialized in TTCN , however few instances where this varibale is used are missed. Similar changes need done in funciton fl_UTRAN34_CheckCapabilityContainerExtension. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialize v_UE_CapabilityContainer_Add_Ext as OMIT when passed to function fl_UTRAN34_CheckCapabilityContainerExtension |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 change 1	2

2.2 change 2	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 change 1

| Object name | fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete |
| --- | --- |
| Reason for change | TTCN CR “ R5s210502” was used for resolve the issue that RRCConnectionSetupComplete_r3_add_ext_IEs uninitialized in TTCN , however few instances where this varibale is used are missed. Similar changes need done in funciton fl_UTRAN34_CheckCapabilityContainerExtension. |
| Summary of change | Initialize v_UE_CapabilityContainer_Add_Ext as OMIT when passed to function fl_UTRAN34_CheckCapabilityContainerExtension |
| Module | UTRAN34_UE_CapabilityInfo.ttcn |
| Label |  |

Before:

function fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete(UTRAN_CellId_Type p_CellId, RRCConnectionSetupComplete p_RRCConnectionSetupComplete) runs on UTRAN_PTC { var UE_CapabilityContainer_IEs v_UE_CapabilityContainer; var bitstring v_UeCapabilityContainerBitstring; var bitstring v_UeCapabilityContainerAddExtBitstring; var RRCConnectionSetupComplete_r3_add_ext_IEs v_UE_CapabilityContainer_Add_Ext; var template(omit) RRCConnectionSetupComplete_r3_add_ext_IEs v_UE_CapabilityContainer_Add_ExtContent := omit; //@sic R5s210502 sic@ // Decode the UE-CapabilityContainer if (ispresent(p_RRCConnectionSetupComplete.v370NonCriticalExtensions.v380NonCriticalExtensions.v3a0NonCriticalExtensions.laterNonCriticalExtensions.v3g0NonCriticalExtensions.v4b0NonCriticalExtensions.v590NonCriticalExtensions.v5c0NonCriticalExtensions.v690NonCriticalExtensions.rrcConnectionSetupComplete_v690ext.ueCapabilityContainer)) { v_UeCapabilityContainerBitstring := p_RRCConnectionSetupComplete.v370NonCriticalExtensions.v380NonCriticalExtensions.v3a0NonCriticalExtensions.laterNonCriticalExtensions.v3g0NonCriticalExtensions.v4b0NonCriticalExtensions.v590NonCriticalExtensions.v5c0NonCriticalExtensions.v690NonCriticalExtensions.rrcConnectionSetupComplete_v690ext.ueCapabilityContainer; if (decvalue(v_UeCapabilityContainerBitstring, v_UE_CapabilityContainer) != 0) { f_SetVerdict(fail, __FILE__, __LINE__, "UE capability container can not be decoded"); } } // decode UE-CapabilityContainer-AddExt if (ispresent(p_RRCConnectionSetupComplete.v370NonCriticalExtensions.v380NonCriticalExtensions.v3a0NonCriticalExtensions.laterNonCriticalExtensions.rrcConnectionSetupComplete_r3_add_ext)) { v_UeCapabilityContainerAddExtBitstring := p_RRCConnectionSetupComplete.v370NonCriticalExtensions.v380NonCriticalExtensions.v3a0NonCriticalExtensions.laterNonCriticalExtensions.rrcConnectionSetupComplete_r3_add_ext; if (decvalue(v_UeCapabilityContainerAddExtBitstring, v_UE_CapabilityContainer_Add_Ext) != 0) { f_SetVerdict(fail, __FILE__, __LINE__, "UE capability container can not be decoded"); } else { //@sic R5s210502 sic@ v_UE_CapabilityContainer_Add_ExtContent := v_UE_CapabilityContainer_Add_Ext; } } // Check RLC capabilities extensions fl_UTRAN34_CheckRLC_CapabilityExtension(p_CellId, p_RRCConnectionSetupComplete, v_UE_CapabilityContainer); // Check UE radio access capability content //----------------------------------------- fl_UTRAN34_CheckCapabilitiesRRCConnectionSetupComplete(p_RRCConnectionSetupComplete); // Check UE radio access capability content in the capability containier fl_UTRAN34_CheckCapabilityContainer(v_UE_CapabilityContainer); // Check UE radio access capability content in the capability containier Add Ext fl_UTRAN34_CheckCapabilityContainerAddExt(v_UE_CapabilityContainer_Add_ExtContent); //@sic R5s210502 sic@ // Check UE radio access capability extension fl_UTRAN34_CheckCapabilityContainerExtension(p_RRCConnectionSetupComplete, v_UE_CapabilityContainer_Add_Ext); <<SKIPPED CODE>>

After:

function fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete(UTRAN_CellId_Type p_CellId, RRCConnectionSetupComplete p_RRCConnectionSetupComplete) runs on UTRAN_PTC { var UE_CapabilityContainer_IEs v_UE_CapabilityContainer; var bitstring v_UeCapabilityContainerBitstring; var bitstring v_UeCapabilityContainerAddExtBitstring; var RRCConnectionSetupComplete_r3_add_ext_IEs v_UE_CapabilityContainer_Add_Ext; var template(omit) RRCConnectionSetupComplete_r3_add_ext_IEs v_UE_CapabilityContainer_Add_ExtContent := omit; //@sic R5s210502 sic@ // Decode the UE-CapabilityContainer if (ispresent(p_RRCConnectionSetupComplete.v370NonCriticalExtensions.v380NonCriticalExtensions.v3a0NonCriticalExtensions.laterNonCriticalExtensions.v3g0NonCriticalExtensions.v4b0NonCriticalExtensions.v590NonCriticalExtensions.v5c0NonCriticalExtensions.v690NonCriticalExtensions.rrcConnectionSetupComplete_v690ext.ueCapabilityContainer)) { v_UeCapabilityContainerBitstring := p_RRCConnectionSetupComplete.v370NonCriticalExtensions.v380NonCriticalExtensions.v3a0NonCriticalExtensions.laterNonCriticalExtensions.v3g0NonCriticalExtensions.v4b0NonCriticalExtensions.v590NonCriticalExtensions.v5c0NonCriticalExtensions.v690NonCriticalExtensions.rrcConnectionSetupComplete_v690ext.ueCapabilityContainer; if (decvalue(v_UeCapabilityContainerBitstring, v_UE_CapabilityContainer) != 0) { f_SetVerdict(fail, __FILE__, __LINE__, "UE capability container can not be decoded"); } } // decode UE-CapabilityContainer-AddExt if (ispresent(p_RRCConnectionSetupComplete.v370NonCriticalExtensions.v380NonCriticalExtensions.v3a0NonCriticalExtensions.laterNonCriticalExtensions.rrcConnectionSetupComplete_r3_add_ext)) { v_UeCapabilityContainerAddExtBitstring := p_RRCConnectionSetupComplete.v370NonCriticalExtensions.v380NonCriticalExtensions.v3a0NonCriticalExtensions.laterNonCriticalExtensions.rrcConnectionSetupComplete_r3_add_ext; if (decvalue(v_UeCapabilityContainerAddExtBitstring, v_UE_CapabilityContainer_Add_Ext) != 0) { f_SetVerdict(fail, __FILE__, __LINE__, "UE capability container can not be decoded"); } else { //@sic R5s210502 sic@ v_UE_CapabilityContainer_Add_ExtContent := v_UE_CapabilityContainer_Add_Ext; } } // Check RLC capabilities extensions fl_UTRAN34_CheckRLC_CapabilityExtension(p_CellId, p_RRCConnectionSetupComplete, v_UE_CapabilityContainer); // Check UE radio access capability content //----------------------------------------- fl_UTRAN34_CheckCapabilitiesRRCConnectionSetupComplete(p_RRCConnectionSetupComplete); // Check UE radio access capability content in the capability containier fl_UTRAN34_CheckCapabilityContainer(v_UE_CapabilityContainer); // Check UE radio access capability content in the capability containier Add Ext fl_UTRAN34_CheckCapabilityContainerAddExt(v_UE_CapabilityContainer_Add_ExtContent); //@sic R5s210502 sic@ // Check UE radio access capability extension fl_UTRAN34_CheckCapabilityContainerExtension(p_RRCConnectionSetupComplete, v_UE_CapabilityContainer_Add_ExtContent ); <<SKIPPED CODE>>

## 2.2 change 2

| Object name | fl_UTRAN34_CheckCapabilityContainerExtension |
| --- | --- |
| Reason for change | As mentioned above the same variable is passed to funtion fl_UTRAN34_CheckCapabilityContainerExtension as well, so it needs to be assinged an OMIT value otherwise it causes a runtime error. |
| Summary of change | Initialize v_UE_CapabilityContainer_Add_Ext in function fl_UTRAN34_CheckCapabilityContainerExtension |
| Module | UTRAN34_UE_CapabilityInfo.ttcn |
| Label |  |

Before:

function fl_UTRAN34_CheckCapabilityContainerExtension(RRCConnectionSetupComplete p_RRCConnectionSetupComplete, RRCConnectionSetupComplete_r3_add_ext_IEs p_UE_CapabilityContainer_r3_add_ext) { // Check band 1 only // If UE supports only FDD band 1 and UE does not support GSM and UE does not support E-UTRA // Check the band capabilities in RRCConnectionSetupComplete.ue-RadioAccessCapability //@sic R5-151965 sic@ if (pc_Band1_Supp and not (pc_Band2_Supp or pc_Band3_Supp or pc_Band4_Supp or pc_Band5_Supp or pc_Band7_Supp or pc_Band8_Supp or pc_Band9_Supp or pc_Band10_Supp or pc_Band11_Supp or pc_Band12_Supp or pc_Band13_Supp or pc_Band14_Supp or pc_Band19_Supp or pc_Band21_Supp or pc_Band22_Supp or pc_Band25_Supp or pc_Band26_Supp or pc_Band32_Supp or pc_GSM_450_BAND or pc_GSM_480_BAND or pc_GSM_850_BAND or pc_P_GSM_900_BAND or pc_E_GSM_900_BAND or pc_DCS_1800_BAND or pc_PCS_1900_BAND or pc_eFDD or pc_eTDD)) { <<SKIPPED CODE>> <<SKIPPED CODE>> fl_UTRAN34_CheckBands8_To22(p_UE_CapabilityContainer_r3_add_ext.rrcConnectionSetupComplete_v650ext.ue_RadioAccessCapability_v650ext.ue_RadioAccessCapabBandFDDList2); <<SKIPPED CODE>> <<SKIPPED CODE>> fl_UTRAN34_CheckBands23_AndAbove(p_UE_CapabilityContainer_r3_add_ext.v680NonCriticalExtensions.v7e0NonCriticalExtensions.v7f0NonCriticalExtensions.va40NonCriticalExtensions.rrcConnectionSetupCompleteBand_va40ext.ue_RadioAccessCapabBand.ue_RadioAccessCapabBandFDDList5);

After:

function fl_UTRAN34_CheckCapabilityContainerExtension(RRCConnectionSetupComplete p_RRCConnectionSetupComplete, template(omit) RRCConnectionSetupComplete_r3_add_ext_IEs p_UE_CapabilityContainer_r3_add_ext) { // Check band 1 only // If UE supports only FDD band 1 and UE does not support GSM and UE does not support E-UTRA // Check the band capabilities in RRCConnectionSetupComplete.ue-RadioAccessCapability //@sic R5-151965 sic@ if (pc_Band1_Supp and not (pc_Band2_Supp or pc_Band3_Supp or pc_Band4_Supp or pc_Band5_Supp or pc_Band7_Supp or pc_Band8_Supp or pc_Band9_Supp or pc_Band10_Supp or pc_Band11_Supp or pc_Band12_Supp or pc_Band13_Supp or pc_Band14_Supp or pc_Band19_Supp or pc_Band21_Supp or pc_Band22_Supp or pc_Band25_Supp or pc_Band26_Supp or pc_Band32_Supp or pc_GSM_450_BAND or pc_GSM_480_BAND or pc_GSM_850_BAND or pc_P_GSM_900_BAND or pc_E_GSM_900_BAND or pc_DCS_1800_BAND or pc_PCS_1900_BAND or pc_eFDD or pc_eTDD)) { <<SKIPPED CODE>> <<SKIPPED CODE>> fl_UTRAN34_CheckBands8_To22( valueof(p_UE_CapabilityContainer_r3_add_ext.rrcConnectionSetupComplete_v650ext.ue_RadioAccessCapability_v650ext.ue_RadioAccessCapabBandFDDList2)); <<SKIPPED CODE>> <<SKIPPED CODE>> fl_UTRAN34_CheckBands23_AndAbove( valueof(p_UE_CapabilityContainer_r3_add_ext.v680NonCriticalExtensions.v7e0NonCriticalExtensions.v7f0NonCriticalExtensions.va40NonCriticalExtensions.rrcConnectionSetupCompleteBand_va40ext.ue_RadioAccessCapabBand.ue_RadioAccessCapabBandFDDList5));