3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230079

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2917 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NCorrection for NR5GC UAC test case 11.3.1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step 1, the SS is configured to only send once short message every defaultPagingCycle in RRC_CONNECTED; the UE may not receive the Short message and hence fail the testcase; SS shall send short message at every PF(N times in one cycle) in RRC_CONNECTED |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Calling f_NR_ModifySysinfo function with RRC_CONNECTED |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.1a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_11_3_1a_TestBody() |
| --- | --- |
| Reason for change | At Step 1, the SS is configured to only send once short message every defaultPagingCycle in RRC_CONNECTED; the UE may not receive the Short message and hence fail the testcase; SS shall send short message at every PF(N times in one cycle) in RRC_CONNECTED |
| Summary of change | Updated Calling f_NR_ModifySysinfo function with RRC_CONNECTED |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_3_1a_TestBody() runs on NR5GC_PTC { var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var octetstring v_EncodedRrcSetup; var template(value) DL_CCCH_Message v_RrcSetup; var float v_TimerValue := 6.0; var float v_TimerValue2 := 30.0; timer t_WaitTime; //@siclog "Step 1" siclog@ //SS changes SIB1 according to Table 11.3.1a.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(7,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 2" siclog@ ... }

After Change

function fl_TC_11_3_1a_TestBody() runs on NR5GC_PTC { var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var octetstring v_EncodedRrcSetup; var template(value) DL_CCCH_Message v_RrcSetup; var float v_TimerValue := 6.0; var float v_TimerValue2 := 30.0; timer t_WaitTime; //@siclog "Step 1" siclog@ //SS changes SIB1 according to Table 11.3.1a.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(7,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1,-,RRC_CONNECTED); //Modify Sysinfo in SS f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 2" siclog@ ... }