3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230080

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2918 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.7.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. - At step 15, the UE may receive 2 UL grant when p_UL_CA = TRUE, as the first grant for p_SPCell is not stopped and SS is configuring the second grant for p_SCell and both are SR based so as soon as UE send SR both the grants are sent from SS side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Instead constant LCID, added conditional LCID value as per DRB id. - A check is made to stop the UL grant for the SPCell when p_UL_CA = TRUE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.7.1.1.NR5GC, 7.1.1.7.1.1.ENDC, 7.1.1.7.1.2.NR5GC, 7.1.1.7.1.2.ENDC, 7.1.1.7.1.3.NR5GC, 7.1.1.7.1.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_1_7_1_NR_TestBody () |
| --- | --- |
| Reason for change | - LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. - At step 15, the UE may receive 2 UL grant when p_UL_CA = TRUE, as the first grant for p_SPCell is not stopped and SS is configuring the second grant for p_SCell and both are SR based so as soon as UE send SR both the grants are sent from SS side. |
| Summary of change | - Instead constant LCID, added conditional LCID value as per DRB id. - A check is made to stop the UL grant for the SPCell when p_UL_CA = TRUE |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer ... timer t_Watchdog := 5.0; // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep); //@sic R5s210727 sic@ if(p_UL_CA) { f_NR_ULGrantConfiguration_Stop(p_SCell, -,cs_NR_UplinkTimeAlignment_Keep); }; // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; //@sic R5s210893 sic@ v_EncodedRlcPdu4 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListUL1:= { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer,v_EncodedRlcPdu4) }; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL1) } v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) }; v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) }; //@sic R5s210893 sic@ v_EncodedRlcPdu5 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListUL2:= { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer,v_EncodedRlcPdu5) }; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL2) } v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) }; v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) }; v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit, cs_NR_MAC_Padding_SubPDU('00000000'O)) }; // 1 byte sub headers +4byte Padding header= 40 bits v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2); v_CellID_UL := p_SCell; } t_Watchdog.start ; //@siclog "Steps 3" siclog@ ... //@siclog "Steps 15" siclog@ //@sic R5s210865 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(v_CellID_UL, v_TimingDL, 0); // 40 ms later f_NR_ULGrantTransmissionOnSR(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2); //@sic R5s211654r2 sic@ //@siclog "Steps 17" siclog@ ... }

After Change

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex , DRB_Identity p_NR_DRB_Id, boolean p_UL_CA) runs on NR_BASE_PTC { //Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer ... timer t_Watchdog := 5.0; // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep); //@sic R5s210727 sic@ if(p_UL_CA) { f_NR_ULGrantConfiguration_Stop(p_SCell, -,cs_NR_UplinkTimeAlignment_Keep); }; // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) }; //@sic R5s210893 sic@ v_EncodedRlcPdu4 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListUL1:= { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID,v_EncodedRlcPdu4) }; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL1) } v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu2) }; v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) }; //@sic R5s210893 sic@ v_EncodedRlcPdu5 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListUL2:= { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID,v_EncodedRlcPdu5) }; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL2) } v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_36B)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu3) }; v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) }; v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit, cs_NR_MAC_Padding_SubPDU('00000000'O)) }; // 1 byte sub headers +4byte Padding header= 40 bits v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2); v_CellID_UL := p_SCell; } t_Watchdog.start ; //@siclog "Steps 3" siclog@ ... //@siclog "Steps 15" siclog@ if(p_UL_CA) //KS#22xxxx { f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep); } //@sic R5s210865 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(v_CellID_UL, v_TimingDL, 0); // 40 ms later f_NR_ULGrantTransmissionOnSR(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2); //@sic R5s211654r2 sic@ //@siclog "Steps 17" siclog@ ... }