3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230082

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2920 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC EPS fallback test case 11.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 1 of Table 11.1.6.3.2-2, PDN CONNECTIVITY REQUEST is received on its own and not piggybacked to ATTACH REQUEST message. So as per TS 24.301, clause 8.3.20.3, IE may inlcude APN in the PDN CONNECTIVITY REQUEST message. This is not allowed in the current test specification which will lead to test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated TTCN for 11.1.6 to include APN in PDN connectivity request. Similar change was done before in R5-226605 for 11.1.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_11_1_6_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | At step 1 of Table 11.1.6.3.2-2, PDN CONNECTIVITY REQUEST is received on its own and not piggybacked to ATTACH REQUEST message. So as per TS 24.301, clause 8.3.20.3, IE may inlcude APN in the PDN CONNECTIVITY REQUEST message. This is not allowed in the current test specification which will lead to test case failure. |
| Summary of change | Updated TTCN for 11.1.6 to include APN in PDN connectivity. Similar change was done before in R5-226605 for 11.1.5 Note: prose CR is required |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_1_6_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { ................ // @siclog "Step 5A" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); f_TAUReject_AttachWithoutN26 (eutra_Cell1); t_Wait.start; alt { // @sic R5s211708 sic@ // @sic R5-221454 sic@ [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?)))) -> value v_ReceivedAsp { t_Wait.stop; v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_Wait.timeout { } } ................ }

After Change

function fl_TC_11_1_6_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { ................ // @siclog "Step 5A" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); f_TAUReject_AttachWithoutN26 (eutra_Cell1); t_Wait.start; alt { // @sic R5s211708 sic@ // @sic R5-221454 sic@ []SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?, -, -, *)))) -> value v_ReceivedAsp { t_Wait.stop; v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_Wait.timeout { } } ................ }