3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230089

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2926 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC CAG testcase 6.5.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TTCN implementation of the testcase 6.5.2.1, the IE manualCAGselectionAllowed_r16 needs to be set to TRUE for nr_Cell 2 and nr_Cell 4. This is not being done at present. Thus the TTCN is not in-line with Prose. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the implementation of function f_NR_GetPNI_NPN_IdentityInfoList to checkwhether the testcase 6.5.2.1 is being run and based on that initialized the variable v_NPN_IdentityInfoList. In case 6.5.2.1 is run, the variable v_NPN_IdentityInfoList is initialized using a newly created template cs_CAG_IdentityInfo_ManualCAGSelection which initializes the IE manualCAGselectionAllowed_r16 to true. For other sitautions, the earlier used template cs_CAG_IdentityInfo is used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_NR_GetPNI_NPN_IdentityInfoList	2

2.2	template cs_CAG_IdentityInfo_ManualCAGSelection	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_NR_GetPNI_NPN_IdentityInfoList

| Function name | f_NR_GetPNI_NPN_IdentityInfoList |
| --- | --- |
| Reason for change | In current TTCN implementation of the testcase 6.5.2.1, the IE manualCAGselectionAllowed_r16 needs to be set to TRUE for nr_Cell 2 and nr_Cell 4. This is not being done at present. Thus the TTCN is not in-line with Prose. This needs to be addressed. |
| Summary of change | Updated the implementation of function f_NR_GetPNI_NPN_IdentityInfoList to checkwhether the testcase 6.5.2.1 is being run and based on that initialized the variable v_NPN_IdentityInfoList. In case 6.5.2.1 is run, the variable v_NPN_IdentityInfoList is initialized using a newly created template cs_CAG_IdentityInfo_ManualCAGSelection which initializes the IE manualCAGselectionAllowed_r16 to true. For other sitautions, the earlier used template cs_CAG_IdentityInfo is used. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_GetPNI_NPN_IdentityInfoList( template(value) NR_NAS_Parameters_Type p_NAS_Parameters, integer p_Cag_Identity := 1) return template(omit) NPN_IdentityInfoList_r16 { var template(omit) NPN_IdentityInfoList_r16 v_NPN_IdentityInfoList := omit; v_NPN_IdentityInfoList := {cs_NPN_IdentityInfo_Def(cs_NPN_IdentityCAG ( valueof(p_NAS_Parameters.PLMN_Identity), valueof(cs_CAG_IdentityInfo(int2bit(p_Cag_Identity,32)))), p_NAS_Parameters.TrackingAreaCode, p_NAS_Parameters.Ranac, p_NAS_Parameters.CellIdentity) } return v_NPN_IdentityInfoList; }

After Change:

function f_NR_GetPNI_NPN_IdentityInfoList( template(value) NR_NAS_Parameters_Type p_NAS_Parameters, integer p_Cag_Identity := 1) return template(omit) NPN_IdentityInfoList_r16 { var template(omit) NPN_IdentityInfoList_r16 v_NPN_IdentityInfoList := omit; if(testcasename() == "TC_6_5_2_1_NR5GC") { //WA#6_5_2_1 v_NPN_IdentityInfoList := {cs_NPN_IdentityInfo_Def(cs_NPN_IdentityCAG ( valueof(p_NAS_Parameters.PLMN_Identity), valueof(cs_CAG_IdentityInfo_ManualCAGSelection(int2bit(p_Cag_Identity,32)))), p_NAS_Parameters.TrackingAreaCode, p_NAS_Parameters.Ranac, p_NAS_Parameters.CellIdentity) } }else{ v_NPN_IdentityInfoList := {cs_NPN_IdentityInfo_Def(cs_NPN_IdentityCAG ( valueof(p_NAS_Parameters.PLMN_Identity), valueof(cs_CAG_IdentityInfo(int2bit(p_Cag_Identity,32)))), p_NAS_Parameters.TrackingAreaCode, p_NAS_Parameters.Ranac, p_NAS_Parameters.CellIdentity) } } return v_NPN_IdentityInfoList; }

## template cs_CAG_IdentityInfo_ManualCAGSelection

| Function name | cs_CAG_IdentityInfo_ManualCAGSelection |
| --- | --- |
| Reason for change | Pleasesee change 2.1 |
| Summary of change | New template |
| TTCN module | NR_SysInfo_Templates.ttcn |
| MCC160 Comment |  |

template(value) CAG_IdentityInfo_r16 cs_CAG_IdentityInfo_ManualCAGSelection(B32_Type p_CAG_Identity) := //WA#6_5_2_1 { /* @status APPROVED (NR5GC) */ cag_Identity_r16 := p_CAG_Identity, manualCAGselectionAllowed_r16 := true_ };