3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230092

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2929 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC SNPN test case 9.1.11.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per Table 9.1.11.2.3.1–1, the Assignment mode in NID shall be 0, while the TTCN implementation has value of 1 assigned to the cell by default, which would cause a mismatch in the Authentication Response sent by the UE and the res calculated by the SS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new parameter p_AssignmentMode in all subsequent functions to make this value (0) assigned as per the TC requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.11.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

2.4	Change 4	8

2.5	Change 5	9

2.6	Change 6	11

2.7	Change 7	12

2.8	Change 8	14

2.9	Change 9	16

2.10	Change 10	17

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR_InitNDI |
| --- | --- |
| Reason for change | As per Table 9.1.11.2.3.1–1, the Assignment mode in NID shall be 0, while the TTCN implementation has value of 1 assigned to the cell by default, which would cause a mismatch in the Authentication Response sent by the UE and the res calculated by the SS. |
| Summary of change | Added new parameter p_AssignmentMode in all subsequent functions to make this value (0) assigned as per the TC requirement. |
| TTCN module | NR_CellInfoInit |
| MCC160 Comment |  |

Before Change

function f_NR_InitNDI( NR_CellId_Type p_NR_CellId ) return NID_r16 { var NID_r16 v_NID_r16; if (pc_SNPN_only) { v_NID_r16 := int2bit(1,4) & int2bit(0, 40) } else { select (p_NR_CellId) { case(nr_Cell1) { v_NID_r16 := int2bit(1,4) & int2bit(1, 40) } case(nr_Cell2) { v_NID_r16 := int2bit(1,4) & int2bit(2, 40) } case(nr_Cell3) { v_NID_r16 := int2bit(1,4) & int2bit(3, 40) } case(nr_Cell4) { v_NID_r16 := int2bit(1,4) & int2bit(4, 40) } case(nr_Cell6) { v_NID_r16 := int2bit(1,4) & int2bit(6, 40) } case(nr_Cell10) { v_NID_r16 := int2bit(1,4) & int2bit(10, 40) } case(nr_Cell11) { v_NID_r16 := int2bit(1,4) & int2bit(11, 40) } case(nr_Cell12) { v_NID_r16 := int2bit(1,4) & int2bit(12, 40) } case(nr_Cell13) { v_NID_r16 := int2bit(1,4) & int2bit(13, 40) } case(nr_Cell14) { v_NID_r16 := int2bit(1,4) & int2bit(14, 40) } case(nr_Cell23) { v_NID_r16 := int2bit(1,4) & int2bit(23, 40) } case(nr_Cell28) { v_NID_r16 := int2bit(1,4) & int2bit(28, 40) } case(nr_Cell29) { v_NID_r16 := int2bit(1,4) & int2bit(29, 40) } case(nr_Cell30) { v_NID_r16 := int2bit(1,4) & int2bit(30, 40) } case(nr_Cell31) { v_NID_r16 := int2bit(1,4) & int2bit(32, 40) } case else {FatalError(__FILE__, __LINE__, "Invalid NR CellId");} } } return v_NID_r16; }

After Change

function f_NR_InitNDI( NR_CellId_Type p_NR_CellId, integer p_AssignmentMode := 1 ) return NID_r16 { var NID_r16 v_NID_r16; if (pc_SNPN_only) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(0, 40) } else { select (p_NR_CellId) { case(nr_Cell1) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(1, 40) } case(nr_Cell2) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(2, 40) } case(nr_Cell3) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(3, 40) } case(nr_Cell4) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(4, 40) } case(nr_Cell6) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(6, 40) } case(nr_Cell10) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(10, 40) } case(nr_Cell11) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(11, 40) } case(nr_Cell12) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(12, 40) } case(nr_Cell13) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(13, 40) } case(nr_Cell14) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(14, 40) } case(nr_Cell23) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(23, 40) } case(nr_Cell28) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(28, 40) } case(nr_Cell29) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(29, 40) } case(nr_Cell30) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(30, 40) } case(nr_Cell31) { v_NID_r16 := int2bit(p_AssignmentMode,4) & int2bit(32, 40) } case else {FatalError(__FILE__, __LINE__, "Invalid NR CellId");} } }

## Change 2

| Function name | f_NR5GC_Preamble |
| --- | --- |
| Reason for change | Same as above Correction |
| Summary of change | Same as above Correction |
| TTCN module | NR_ NR5GC_NASSteps |
| MCC160 Comment |  |

Before Change

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId, NG_STATE_Type p_State, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, boolean p_ActivateSecurityInactiveState := true, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //@sic R5s190505: added p_ActivateSecurityInactiveState sic@ //@sic R5-199036: added p_IpPduDelayTime sic@ var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@ if (p_State == STATE_INACTIVE_2A) { v_ReleaseConnection := noRrcConnectionRelease; } // send UT functions to UE before cell switched on f_NR5GC_ProvideSNPNData(); // @sic R5-225300 sic@ // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); // Go to RRC_IDLE f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime, p_ServiceType); // @sic R5s210189 sic@ } select (p_State) { case (STATE_OFF_0B) { // Now turn off again f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A); } case (STATE_INACTIVE_2A) { f_NR_RRC_InactiveDef(p_NR_CellId, -, p_ActivateSecurityInactiveState); } case (STATE_CONNECTED_3A) { // Go to RRC_CONNECTED f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode); } } }

After Change

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId, NG_STATE_Type p_State, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, boolean p_ActivateSecurityInactiveState := true, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B, integer p_AssignmentMode := 1) runs on NR5GC_PTC { //@sic R5s190505: added p_ActivateSecurityInactiveState sic@ //@sic R5-199036: added p_IpPduDelayTime sic@ var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@ if (p_State == STATE_INACTIVE_2A) { v_ReleaseConnection := noRrcConnectionRelease; } // send UT functions to UE before cell switched on f_NR5GC_ProvideSNPNData(); // @sic R5-225300 sic@ // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); // Go to RRC_IDLE f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime, p_ServiceType, p_AssignmentMode); // @sic R5s210189 sic@ } select (p_State) { case (STATE_OFF_0B) { // Now turn off again f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A); } case (STATE_INACTIVE_2A) { f_NR_RRC_InactiveDef(p_NR_CellId, -, p_ActivateSecurityInactiveState); } case (STATE_CONNECTED_3A) { // Go to RRC_CONNECTED f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode); } } }

## Change 3

| Function name | f_NR5GC_RRC_IdleState1N_Def |
| --- | --- |
| Reason for change | Same as above Correction |
| Summary of change | Same as above Correction |
| TTCN module | NR_ NR5GC_NASSteps |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRC_IdleState1N_Def(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //@sic R5-199036: added p_IpPduDelayTime sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, p_RegType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Steps 5 - 20 f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, p_TestMode, p_ReleaseConnection, p_RegType, -, p_IpPduDelayTime, -, p_ServiceType); // @sic R5s210189 sic@ }

After Change

function f_NR5GC_RRC_IdleState1N_Def(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B, integer p_AssignmentMode := 1) runs on NR5GC_PTC { //@sic R5-199036: added p_IpPduDelayTime sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, p_RegType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Steps 5 - 20 f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, p_TestMode, p_ReleaseConnection, p_RegType, -, p_IpPduDelayTime, -, p_ServiceType, p_AssignmentMode); // @sic R5s210189 sic@ }

## Change 4

| Function name | f_NR5GC_RRC_Idle_Steps5_20 |
| --- | --- |
| Reason for change | Same as above Correction |
| Summary of change | Same as above Correction |
| TTCN module | NR_ NR5GC_NASSteps |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRC_Idle_Steps5_20(GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); // Steps 5-15 f_NR5GC_RRC_Idle_Steps5_15(p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, v_PLMN, p_TestMode, p_RegType, -, -, p_IwkN26); // @sic R5-202564 sic@ //Steps 16 - 20 f_NR5GC_RRC_Idle_Steps16_20 (p_NR_CellId, -, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime, // @sic R5s200751 sic@ -, -, -, -, -, -, -, -, -, p_ServiceType); // @sic R5s210189 sic@ }

After Change

function f_NR5GC_RRC_Idle_Steps5_20(GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, template (present) B4_Type p_ServiceType := '0000'B, integer p_AssignmentMode := 1) runs on NR5GC_PTC { var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); // Steps 5-15 f_NR5GC_RRC_Idle_Steps5_15(p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, v_PLMN, p_TestMode, p_RegType, -, -, p_IwkN26, p_AssignmentMode); // @sic R5-202564 sic@ //Steps 16 - 20 f_NR5GC_RRC_Idle_Steps16_20 (p_NR_CellId, -, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime, // @sic R5s200751 sic@ -, -, -, -, -, -, -, -, -, p_ServiceType); // @sic R5s210189 sic@ }

## Change 5

| Function name | f_NR5GC_RRC_Idle_Steps5_15 |
| --- | --- |
| Reason for change | Same as above Correction |
| Summary of change | Same as above Correction |
| TTCN module | NR_ NR5GC_NASSteps |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRC_Idle_Steps5_15 (GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, NR_TESTMODE_STATE_Type p_TestMode, TypeOfRegistration_Type p_RegType := Initial_Secure, template (omit) PLMN_List p_EPLMNs := omit, template (omit) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (p_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(p_PLMN, {bit2oct(v_TAC)}); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; // No SRB2 so must be on SRB1 // Steps 5 - 13 f_NR5GC_RRC_Idle_Steps5_13 (p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, p_PLMN, p_TestMode, p_RegType, p_IwkN26); // @sic R5-202564 sic@ // Step 14 ................................................. }

After Change

function f_NR5GC_RRC_Idle_Steps5_15 (GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, NR_TESTMODE_STATE_Type p_TestMode, TypeOfRegistration_Type p_RegType := Initial_Secure, template (omit) PLMN_List p_EPLMNs := omit, template (omit) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, integer p_AssignmentMode := 1) runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (p_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(p_PLMN, {bit2oct(v_TAC)}); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; // No SRB2 so must be on SRB1 // Steps 5 - 13 f_NR5GC_RRC_Idle_Steps5_13 (p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, p_PLMN, p_TestMode, p_RegType, p_IwkN26, p_AssignmentMode); // @sic R5-202564 sic@ // Step 14 // Registration Accept ................................................. }

## Change 6

| Function name | f_NR5GC_RRC_Idle_Steps5_13 |
| --- | --- |
| Reason for change | Same as above Correction |
| Summary of change | Same as above Correction |
| TTCN module | NR_ NR5GC_NASSteps |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRC_Idle_Steps5_13 (inout GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, // @sic R5s211357 sic@ NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, NR_TESTMODE_STATE_Type p_TestMode, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC { var NasCount_Type v_NasCount; // Steps 5 - 6 // Do Authentication v_NasCount := f_NR5GC_RRC_Idle_Steps5_9_AKA (p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, p_PLMN, p_RegType, p_IwkN26); // @sic R5-202564 sic@ // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(p_GMM_MobilityInfo_Type); // @sic R5-195328, R5s190388 sic@ ................................................. }

After Change

function f_NR5GC_RRC_Idle_Steps5_13 (inout GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, // @sic R5s211357 sic@ NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, NR_TESTMODE_STATE_Type p_TestMode, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, integer p_AssignmentMode := 1) runs on NR5GC_PTC { var NasCount_Type v_NasCount; // Steps 5 - 6 // Do Authentication v_NasCount := f_NR5GC_RRC_Idle_Steps5_9_AKA (p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, p_PLMN, p_RegType, p_IwkN26, p_AssignmentMode); // @sic R5-202564 sic@ // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(p_GMM_MobilityInfo_Type); // @sic R5-195328, R5s190388 sic@ ................................................. }

## Change 7

| Function name | f_NR5GC_RRC_Idle_Steps5_9_AKA |
| --- | --- |
| Reason for change | Same as above Correction |
| Summary of change | Same as above Correction |
| TTCN module | NR_ NR5GC_NASSteps |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRC_Idle_Steps5_9_AKA (inout GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC return NasCount_Type { var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var NG_NAS_MSG_Indication_Type v_ReceivedMsg := p_ReceivedMsg; var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var SecurityHeaderType v_SentSecurityHeader; var template (present) SecurityHeaderType v_SecurityHeaderResponse; if(p_RegType == Initial_NoSecurity) { // @sic R5s190109 sic@ v_SentSecurityHeader := tsc_SHT_NoSecurityProtection; v_SecurityHeaderResponse := tsc_SHT_NoSecurityProtection; } else { v_SentSecurityHeader := tsc_SHT_IntegrityProtected_Ciphered; v_SecurityHeaderResponse := tsc_SHT_IntegrityProtected_Ciphered; // @sic R5s190109 sic@ } if (v_ReceivedMsg.Pdu.Msg.registration_Request.ngNasKSI.nasKeySetId != '111'B) { // @sic R5s190929 sic@ v_SecurityParams.KSIamf := v_ReceivedMsg.Pdu.Msg.registration_Request.ngNasKSI.nasKeySetId; if(p_RegType == Initial_NoSecurity) { // @sic R5s200465 sic@ v_SecurityHeaderResponse := tsc_SHT_IntegrityProtected; // @sic R5s190109, R5-200477 sic@ } f_NR5GC_Security_Set(v_SecurityParams); } if (px_NAS_5GC_AuthenticationType == AKA_5G) { f_NR_Authentication5G_AKA(p_NR_CellId, v_SentSecurityHeader, v_SecurityHeaderResponse, p_PLMN); } else { f_NR_AuthenticationEAP_AKA(p_NR_CellId, v_SentSecurityHeader, v_SecurityHeaderResponse, p_PLMN); } // Step 8 - 9 ................................................. }

After Change

function f_NR5GC_RRC_Idle_Steps5_9_AKA (inout GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, integer p_AssignmentMode := 1) runs on NR5GC_PTC return NasCount_Type { var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var NG_NAS_MSG_Indication_Type v_ReceivedMsg := p_ReceivedMsg; var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var SecurityHeaderType v_SentSecurityHeader; var template (present) SecurityHeaderType v_SecurityHeaderResponse; if(p_RegType == Initial_NoSecurity) { // @sic R5s190109 sic@ v_SentSecurityHeader := tsc_SHT_NoSecurityProtection; v_SecurityHeaderResponse := tsc_SHT_NoSecurityProtection; } else { v_SentSecurityHeader := tsc_SHT_IntegrityProtected_Ciphered; v_SecurityHeaderResponse := tsc_SHT_IntegrityProtected_Ciphered; // @sic R5s190109 sic@ } if (v_ReceivedMsg.Pdu.Msg.registration_Request.ngNasKSI.nasKeySetId != '111'B) { // @sic R5s190929 sic@ v_SecurityParams.KSIamf := v_ReceivedMsg.Pdu.Msg.registration_Request.ngNasKSI.nasKeySetId; if(p_RegType == Initial_NoSecurity) { // @sic R5s200465 sic@ v_SecurityHeaderResponse := tsc_SHT_IntegrityProtected; // @sic R5s190109, R5-200477 sic@ } f_NR5GC_Security_Set(v_SecurityParams); } if (px_NAS_5GC_AuthenticationType == AKA_5G) { f_NR_Authentication5G_AKA(p_NR_CellId, v_SentSecurityHeader, v_SecurityHeaderResponse, p_PLMN, -, p_AssignmentMode); } else { f_NR_AuthenticationEAP_AKA(p_NR_CellId, v_SentSecurityHeader, v_SecurityHeaderResponse, p_PLMN); } // Step 8 - 9 ................................................. }

## Change 8

| Function name | f_NR_Authentication5G_AKA |
| --- | --- |
| Reason for change | Same as above Correction |
| Summary of change | Same as above Correction |
| TTCN module | NR_ NR5GC_NASSteps |
| MCC160 Comment |  |

Before Change

function f_NR_Authentication5G_AKA(NR_CellId_Type p_NR_CellId, SecurityHeaderType p_SecurityStatusAuthRequest, template (present) SecurityHeaderType p_SecurityStatusAuthResponse, NAS_PlmnId p_PLMN, NR_RadioBearerId_Type p_SRBId := tsc_NR_RbId_SRB1) runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var NR_SecurityParams_Type v_RRC_SecurityParams := f_NR_Security_Get(); var NID_r16 v_NID_ASN; var template (omit) hexstring v_NID := omit; // build AUTHENTICATION REQUEST msg, input PLMN+NID if SNPN test case if(f_GetTestcaseAttrib_NR_SNPN(testcasename()) or pc_SNPN_only){ // @sic R5s220753, R5s221231 sic@ v_NID_ASN := f_NR_InitNDI(p_NR_CellId); // returns a bitstring v_NID := bit2hex(v_NID_ASN); } // send AUTHENTICATION REQUEST ................................................. }

After Change

function f_NR_Authentication5G_AKA(NR_CellId_Type p_NR_CellId, SecurityHeaderType p_SecurityStatusAuthRequest, template (present) SecurityHeaderType p_SecurityStatusAuthResponse, NAS_PlmnId p_PLMN, NR_RadioBearerId_Type p_SRBId := tsc_NR_RbId_SRB1, integer p_AssignmentMode := 1) runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var NR_SecurityParams_Type v_RRC_SecurityParams := f_NR_Security_Get(); var NID_r16 v_NID_ASN; var template (omit) hexstring v_NID := omit; // build AUTHENTICATION REQUEST msg, input PLMN+NID if SNPN test case if(f_GetTestcaseAttrib_NR_SNPN(testcasename()) or pc_SNPN_only){ // @sic R5s220753, R5s221231 sic@ v_NID_ASN := f_NR_InitNDI(p_NR_CellId, p_AssignmentMode); // returns a bitstring v_NID := bit2hex(v_NID_ASN); } // send AUTHENTICATION REQUEST ................................................. }

## Change 9

| Function name | f_TC_9_1_11_2_NR5GC |
| --- | --- |
| Reason for change | Same as above Correction |
| Summary of change | Same as above Correction |
| TTCN module | SNPN_NAS_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_9_1_11_2_NR5GC() runs on NR5GC_PTC { //SNPN / Initial registration / Rejected / Permanently not authorized for this SNPN var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_12); f_NR_CellInfo_SetSIB1_SNPN_Id (v_NGC_NASCellA, 0, 1); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_UT_SwitchOnUE(UT); f_UT_ProvideSubscriberDataList(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), 1); f_UT_ClearRegisteredSNPN_Select(UT); f_UT_AutomaticSNPN_Select(UT); f_UT_SwitchOffUE(UT, false); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_9_1_11_2_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }

After Change

function f_TC_9_1_11_2_NR5GC() runs on NR5GC_PTC { //SNPN / Initial registration / Rejected / Permanently not authorized for this SNPN var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var integer v_AssignmentMode := 0; f_NR5GC_Init_NAS(NR_12); f_NR_CellInfo_SetSIB1_SNPN_Id (v_NGC_NASCellA, v_AssignmentMode, 1); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_UT_SwitchOnUE(UT); f_UT_ProvideSubscriberDataList(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), 1); f_UT_ClearRegisteredSNPN_Select(UT); f_UT_AutomaticSNPN_Select(UT); f_UT_SwitchOffUE(UT, false); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0A, -, -, -, -, -, v_AssignmentMode); f_NR_TestBody_Set(true); fl_TC_9_1_11_2_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }

## Change 10

| Function name | fl_TC_9_1_11_2_Body |
| --- | --- |
| Reason for change | Same as above Correction |
| Summary of change | Same as above Correction |
| TTCN module | SNPN_NAS_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_9_1_11_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); timer t_Wait := 60.0; f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2-13" siclog@ //Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); //@siclog "Steps 14" siclog@ //The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #75 "Permanently not authorized for this SNPN ". SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, '01001011'B))))); //@siclog "Step 15" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 16" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A in the next 60 seconds? t_Wait.start; alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellA, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12: RRCSetupRequest received"); } [] t_Wait.timeout {} } //@siclog "Step 17" siclog@ //The UE is made to perform manual SNPN search and select SNPN identified by NGC Cell A. f_UT_ManualSNPN_Select(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), 1); //@siclog "Step 18" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?,?), //Check the SUCI tsc_NasKsi_NoKey, //check the NAS key set identifier -, -, -, -, -, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); //@siclog "Step 19-34a1" siclog@ //Steps 5-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NGC Cell A. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity); //@siclog "Step 35" siclog@ //The user sets the UE in Automatic SNPN/Network selection mode. f_UT_AutomaticSNPN_Select(UT); } //End of fl_TC_9_1_11_2_Body

After Change

function fl_TC_9_1_11_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); timer t_Wait := 60.0; var integer v_AssignmentMode := 0; f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2-13" siclog@ //Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity, -, v_AssignmentMode); //@siclog "Steps 14" siclog@ //The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #75 "Permanently not authorized for this SNPN ". SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, '01001011'B))))); //@siclog "Step 15" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 16" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A in the next 60 seconds? t_Wait.start; alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellA, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12: RRCSetupRequest received"); } [] t_Wait.timeout {} } //@siclog "Step 17" siclog@ //The UE is made to perform manual SNPN search and select SNPN identified by NGC Cell A. f_UT_ManualSNPN_Select(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), 1); //@siclog "Step 18" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?,?), //Check the SUCI tsc_NasKsi_NoKey, //check the NAS key set identifier -, -, -, -, -, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); //@siclog "Step 19-34a1" siclog@ //Steps 5-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NGC Cell A. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity, -, -, -, -, v_AssignmentMode); //@siclog "Step 35" siclog@ //The user sets the UE in Automatic SNPN/Network selection mode. f_UT_AutomaticSNPN_Select(UT); } //End of fl_TC_9_1_11_2_Body