3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230114

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2944 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the Function f_NR_GetFrequencySA_f1tof4 for FDD band n28 for Redcap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of 22wk49, FDD band n28 test frequencies are not implemented using 20 MHz channel bandwidth for RedCap UE as per theTS 38.508-1 Table 4.3.1.1.1.28-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented the test frequencies for NR FDD band n28 using 20 MHz channel bandwidth for RedCap UE as per theTS 38.508-1 Table 4.3.1.1.1.28-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All RedCap testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_NR_GetFrequencySA_f1tof4 ()	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_NR_GetFrequencySA_f1tof4 ()

| Function name | function f_NR_GetFrequencySA_f1tof4 |
| --- | --- |
| Reason for change | In the current TTCN implementation of 22wk49, FDD band n28 test frequencies are not implemented using 20 MHz channel bandwidth for RedCap UE as per theTS 38.508-1 Table 4.3.1.1.1.28-1. |
| Summary of change | Implemented the test frequencies for NR FDD band n28 using 20 MHz channel bandwidth for RedCap UE as per theTS 38.508-1 Table 4.3.1.1.1.28-1. |
| TTCN module | .\Common\NR_CommonFrequency\NR_FrequencyInit_SA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_GetFrequencySA_f1tof4 ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) return template(value) NR_Frequencyf1tof4_Type { var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(); select ( p_FreqBandIndicatorNR ) { … case( 28 ) { //@sic R5-226894, R5-227084 sic@ if ( (f_GetTestcaseAttrib_ENDC(testcasename()) and (px_ENDC_BandCombination == DC_21A_n28A)) ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 157210 , 28 , 152992 , 102 , kHz15 , 52 ), 10 , kHz15 , 2 , 2 , 107 , 1 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 127520 , 504 , kHz15 , 52 , v_P_Max), 10 ); } else { //@sic R5-186834, R5-190636, R5-198217, R5-197977, R5-201020, R5-200504, R5-201970, R5-206002 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 152210 , 28 , 151664 , 0 , kHz15 , 52 ), 10 , kHz15 , 2 , 2 , 5 , 1 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 140664 , 0 , kHz15 , 52 , v_P_Max), 10 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 154370 , 28 , 153572 , 12 , kHz15 , 52 ), 10 , kHz15 , 2 , 0 , 12 , 0 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 141708 , 36 , kHz15 , 52 , v_P_Max), 10 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 156750 , 28 , 155460 , 24 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 25 , 1 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 141220 , 114 , kHz15 , 52 , v_P_Max), 10 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f4 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 159150 , 28 , 140520 , 504 , kHz15 , 52 ), 10 , kHz15 , 6 , 1 , 507 , 1 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f4 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 147448 , 6 , kHz15 , 52 , v_P_Max), 10 ); } } … }

After Change:

function f_NR_GetFrequencySA_f1tof4 ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) return template(value) NR_Frequencyf1tof4_Type { var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(); select ( p_FreqBandIndicatorNR ) { … case( 28 ) { //@sic R5-226894, R5-227084 sic@ if ( (f_GetTestcaseAttrib_ENDC(testcasename()) and (px_ENDC_BandCombination == DC_21A_n28A)) ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 157210 , 28 , 152992 , 102 , kHz15 , 52 ), 10 , kHz15 , 2 , 2 , 107 , 1 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 127520 , 504 , kHz15 , 52 , v_P_Max), 10 ); } else { //@sic R5-186834, R5-190636, R5-198217, R5-197977, R5-201020, R5-200504, R5-201970, R5-206002 sic@ if( pc_supportOfRedCap_r17 ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 152190 , 28 , 151692 , 0 , kHz15 , 106 ), 20 , kHz15 , 10 , 1 , 3 , 1 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 140692 , 0 , kHz15 , 106 , v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 155070 , 28 , 151020 , 102 , kHz15 , 106 ), 20 , kHz15 , 6 , 0 , 102 , 0 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 125548 , 504 , kHz15 , 106 , v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 157250 , 28 , 138548 , 504 , kHz15 , 106 ), 20 , kHz15 , 6 , 2, 509 , 1 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 125476 , 6 , kHz15 , 106 , v_P_Max), 20 ); } else{ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 152210 , 28 , 151664 , 0 , kHz15 , 52 ), 10 , kHz15 , 2 , 2 , 5 , 1 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 140664 , 0 , kHz15 , 52 , v_P_Max), 10 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 154370 , 28 , 153572 , 12 , kHz15 , 52 ), 10 , kHz15 , 2 , 0 , 12 , 0 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 141708 , 36 , kHz15 , 52 , v_P_Max), 10 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 156750 , 28 , 155460 , 24 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 25 , 1 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 141220 , 114 , kHz15 , 52 , v_P_Max), 10 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f4 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 159150 , 28 , 140520 , 504 , kHz15 , 52 ), 10 , kHz15 , 6 , 1 , 507 , 1 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f4 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 28 }, 147448 , 6 , kHz15 , 52 , v_P_Max), 10 ); } } } … }