3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230122

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2947 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 38.321 clause 6.1.5,if a Backoff Indicator is included it has to be the first element. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move BackoffIndicator to be the first element. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2.NR5GC,7.1.1.1.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1  Overview	2

2 Corrections required	2

2.1 Correction to cs_NR_RAR_SubPduList_71112_2	2

2.2 Correction to cs_NR_RAR_SubPduList_71112_3	3

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Ting Gao

gaoting@starpointcomm.com

- Corrections required

  - Correction to cs_NR_RAR_SubPduList_71112_2

| Function name | cs_NR_RAR_SubPduList_71112_2 |
| --- | --- |
| Reason for change | According to TS 38.321 clause 6.1.5,if a Backoff Indicator is included it has to be the first element. |
| Summary of change | Move BackoffIndicator to be the first element. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) NR_RAR_SubPduList_Type cs_NR_RAR_SubPduList_71112_2 (NR_PhysicalParameters_Type p_PhysicalParameters) := { /* @status APPROVED (ENDC, NR5GC) */ cs_NR_RAR_SubPdu_RapIdAndPayload(cs_RapIdCtrl_UnMatched, tsc_NR_RACH_TA_Def, /* !!!! NR: to be retrieved from the cell's PhysicalParameters !!!! (i.e. NR_RACH_TimingAdvance_Type to be added to NR_PhysicalParameters_Type and !!!! initialised during cell initialisation) !!!! */ f_NR_RAR_UplinkGrant(p_PhysicalParameters.UL_BWPs[0]), tsc_NR_TempC_RNTI_SameAsC_RNTI), cs_NR_RAR_SubPdu_BackoffIndicator(12) };

After Change:

template (value) NR_RAR_SubPduList_Type cs_NR_RAR_SubPduList_71112_2 (NR_PhysicalParameters_Type p_PhysicalParameters) := { /* @status APPROVED (ENDC, NR5GC) */ cs_NR_RAR_SubPdu_BackoffIndicator(12), cs_NR_RAR_SubPdu_RapIdAndPayload(cs_RapIdCtrl_UnMatched, tsc_NR_RACH_TA_Def, /* !!!! NR: to be retrieved from the cell's PhysicalParameters !!!! (i.e. NR_RACH_TimingAdvance_Type to be added to NR_PhysicalParameters_Type and !!!! initialised during cell initialisation) !!!! */ f_NR_RAR_UplinkGrant(p_PhysicalParameters.UL_BWPs[0]), tsc_NR_TempC_RNTI_SameAsC_RNTI) };

  - Correction to cs_NR_RAR_SubPduList_71112_3

| Function name | cs_NR_RAR_SubPduList_71112_3 |
| --- | --- |
| Reason for change | According to TS 38.321 clause 6.1.5,if a Backoff Indicator is included it has to be the first element |
| Summary of change | Move BackoffIndicator to be the first element. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) NR_RAR_SubPduList_Type cs_NR_RAR_SubPduList_71112_3 (NR_PhysicalParameters_Type p_PhysicalParameters) := { /* @status APPROVED (ENDC, NR5GC) */ cs_NR_RAR_SubPdu_RapIdAndPayload(-, tsc_NR_RACH_TA_Def, /* !!!! NR: to be retrieved from the cell's PhysicalParameters !!!! (i.e. NR_RACH_TimingAdvance_Type to be added to NR_PhysicalParameters_Type and !!!! initialised during cell initialisation) !!!! */ f_NR_RAR_UplinkGrant(p_PhysicalParameters.UL_BWPs[0],56), tsc_NR_TempC_RNTI_SameAsC_RNTI), cs_NR_RAR_SubPdu_BackoffIndicator(13) };

After Change:

template (value) NR_RAR_SubPduList_Type cs_NR_RAR_SubPduList_71112_3 (NR_PhysicalParameters_Type p_PhysicalParameters) := { /* @status APPROVED (ENDC, NR5GC) */ cs_NR_RAR_SubPdu_BackoffIndicator(13), cs_NR_RAR_SubPdu_RapIdAndPayload(-, tsc_NR_RACH_TA_Def, /* !!!! NR: to be retrieved from the cell's PhysicalParameters !!!! (i.e. NR_RACH_TimingAdvance_Type to be added to NR_PhysicalParameters_Type and !!!! initialised during cell initialisation) !!!! */ f_NR_RAR_UplinkGrant(p_PhysicalParameters.UL_BWPs[0],56), tsc_NR_TempC_RNTI_SameAsC_RNTI) };