3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230138

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0895 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_IMS_InviteResponse_181_MessageHeaderTX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 181 Call is being forwarded it is used in supplementary test case scenarios when caller calls User-A and it does not get any response (8.41TC) or User-A has Call forwarding unconditional supplementary service activated (8.8 TC), in both cases call is forwarded to User-B. From [TS 24.604, clause 4.2.1.2]: The CFU service enables a served user to have the network redirect to another user communications which are addressed to the served user's address. From above statement can be pointed that 181 message is not sent by user-A or user-B but by the server, meaning than To tag set in 181 message cannot be one used in previous or subsequent dialogs being established. During test case execution, 181 To tag must be set to a different value only use for 181 Call is being forwarded message. Examples for 8.8 and 8.41 TCs flows can be found in RFC 5359 clause 2.7. Call Forwarding Unconditional and 2.9. Call Forwarding - No Answer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected To tag in 181 Call is being forwarded message. - Improvement: modified To tag of forwarded Call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.NR5GC, 8.41.NR5GC, 15.8, J.15.6 and G.15.7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 34.229-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	f_IMS_InviteResponse_181_MessageHeaderTX	3

2.2	f_TC_8_8_NR5GC_IMS1	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## f_IMS_InviteResponse_181_MessageHeaderTX

| Function name | f_IMS_InviteResponse_181_MessageHeaderTX |
| --- | --- |
| Reason for change | 181 message is not sent by user-A or user-B but by the server, meaning than To tag set in 181 message cannot be one used in previous or subsequent dialogs being established. During test case execution, 181 To tag must be set to a different value only use for 181 Call is being forwarded message. |
| Summary of change | Corrected To tag in 181 Call is being forwarded message. |
| TTCN module | \Common\IMS\IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_InviteResponse_181_MessageHeaderTX(INVITE_Request p_InviteRequest, template (value) SipUrl p_TargetedTo) runs on IMS_PTC return template (value) MessageHeader { /* @sic R5-174544: SessionId removed sic@ */ var MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader; var template (value) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderTX(v_MessageHeader_Invite); /* @sic R5s190882: f_IMS_BaseResponse_MessageHeaderTX sic@ */ v_MessageHeader_Response.historyInfo := cs_HistoryInfo(p_TargetedTo); return v_MessageHeader_Response; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_InviteResponse_181_MessageHeaderTX(INVITE_Request p_InviteRequest, template (value) SipUrl p_TargetedTo) runs on IMS_PTC return template (value) MessageHeader { /* @sic R5-174544: SessionId removed sic@ */ var MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader; var template (value) charstring v_ToTag := f_IMS_GenerateTag("181"); var template (value) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderTX(v_MessageHeader_Invite, v_ToTag); /* @sic R5s190882: f_IMS_BaseResponse_MessageHeaderTX sic@ */ v_MessageHeader_Response.historyInfo := cs_HistoryInfo(p_TargetedTo); return v_MessageHeader_Response; } <<SKIPPED CODE>>

## f_TC_8_8_NR5GC_IMS1

| Function name | f_TC_8_8_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Improvement: In this case change is not going to modify the behavior but will help identify that call has been forwarded. |
| Summary of change | Improvement: modified To tag of forwarded Call. |
| TTCN module | \Common\IMS\IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_TC_8_8_NR5GC_IMS1() runs on IMS_PTC { // Communication Forwarding Unconditional / Signalling / 5GS var template (value) SipUrl v_TargetedTo := f_SIP_BuildSipUri_TX("sip:user@company.com"); var template (value) HistoryInfo v_HistoryInfo := cs_HistoryInfo(v_TargetedTo); var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var template (value) MessageHeader v_MessageHeader18X; var template (value) SDP_Message v_SDP_Message; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1 - 7" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 8" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(); // @sic R5-225936: A_4_1 -> A_4_1a sic@ v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); // @siclog "Step 9" siclog@ f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp); // @siclog "Step 10" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine181, f_IMS_InviteResponse_181_MessageHeaderTX(v_InviteRequestWithSdp.Invite, v_TargetedTo)))); // @siclog "Step 11" siclog@ v_MessageHeader18X := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite); v_SDP_Message := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2, 65); // @sic R5-225936: EVS_A2 sic@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine183, v_MessageHeader18X, cs_MessageBody_SDP(v_SDP_Message)))); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_TC_8_8_NR5GC_IMS1() runs on IMS_PTC { // Communication Forwarding Unconditional / Signalling / 5GS var template (value) SipUrl v_TargetedTo := f_SIP_BuildSipUri_TX("sip:user@company.com"); var template (value) HistoryInfo v_HistoryInfo := cs_HistoryInfo(v_TargetedTo); var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var template (value) MessageHeader v_MessageHeader18X; var template (value) SDP_Message v_SDP_Message; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1 - 7" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 8" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(); // @sic R5-225936: A_4_1 -> A_4_1a sic@ v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); // @siclog "Step 9" siclog@ f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp); // @siclog "Step 10" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine181, f_IMS_InviteResponse_181_MessageHeaderTX(v_InviteRequestWithSdp.Invite, v_TargetedTo)))); // @siclog "Step 11" siclog@ f_IMS_PTC_ImsInfo_DialogSetLocalTag(f_IMS_GenerateTag("INVITE-FORWARDED")); v_MessageHeader18X := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite); v_SDP_Message := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2, 65); // @sic R5-225936: EVS_A2 sic@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine183, v_MessageHeader18X, cs_MessageBody_SDP(v_SDP_Message)))); <<SKIPPED CODE>>