3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230147

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2963 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle mode test case 6.2.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Before step 20, where SS is initiating deregistration, the IMS component should be reset to allow for new IMS registration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reseted IMS component before step 20 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function fl_TC_6_2_1_5_TestBody() |
| --- | --- |
| Reason for change | Before step 20, where SS is initiating deregistration, the IMS component should be reset to allow for new IMS registration |
| Summary of change | Reseted IMS component before step 20 |
| TTCN module | Idle_PLMNSelection_NR5GC_NR.TTCN |
| MCC160 Comment |  |

Before Change

function fl_TC_6_2_1_5_TestBody() runs on NR5GC_PTC {// @sic R5-204406, R5s200889 sic@ ................................ // @siclog "Step 19" siclog@ // The SS adjusts cell levels according to row T1 of table 6.2.1.5.3.2-1. if (not f_NR_CellInfo_GetIsFR1(nr_Cell1)) { // SS change NR Cell 1 SIB1 with modification of Qrxlevmin. v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell1, -91.0, v_DeltaPrimaryBand.DeltaNRf1); f_NR_ModifySysinfo(nr_Cell1, false); //Modify Sysinfo in SS without Paging } f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 20" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 after tmin but before tmax expires? ................................ }

After Change

function fl_TC_6_2_1_5_TestBody() runs on NR5GC_PTC {// @sic R5-204406, R5s200889 sic@ ................................ // @siclog "Step 19" siclog@ // The SS adjusts cell levels according to row T1 of table 6.2.1.5.3.2-1. if (not f_NR_CellInfo_GetIsFR1(nr_Cell1)) { // SS change NR Cell 1 SIB1 with modification of Qrxlevmin. v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell1, -91.0, v_DeltaPrimaryBand.DeltaNRf1); f_NR_ModifySysinfo(nr_Cell1, false); //Modify Sysinfo in SS without Paging } f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_IMS_IPCAN_SendCoOrdMsg(IMS[f_PDN_PdnIndex2Integer(PDN_1)], cms_IPCAN_IMS_Reset); //KS#23xxxx // @siclog "Step 20" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 after tmin but before tmax expires? ................................ }