3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230157

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2975 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MDT test case 8.1.6.1.2.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per TS 24.501, clause 5.3.1.3: 5.3.1.3 Release of the N1 NAS signalling connection …………….. To allow the network to release the N1 NAS signalling connection, the UE: a) shall start the timer T3540 if the UE receives any of the 5GMM cause values #7, #11, #12, #13, #15, #27, #31, #62, #72, #73, #74, #75, #76 and the UE does not consider the received 5GMM cause value as abnormal case as specified in subclauses 5.5.1.2.7, 5.5.1.3.7 and 5.5.2.3.4; b) shall start the timer T3540 for a UE in 3GPP access if: 1) the UE receives a REGISTRATION ACCEPT message which does not include a Pending NSSAI IE or UE radio capability ID deletion indication IE; 2) the UE has set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message; …………….. Upon expiry of T3540, - in cases a), b), f), g), h), i), j) and k) the UE shall locally release the established N1 NAS signalling connection; - in cases c) and d) the UE shall locally release the established N1 NAS signalling connection and the UE shall initiate the registration procedure as described in subclause 5.5.1.3 or 5.6.1.5; or - in case e), the UE shall locally release the established N1 NAS signalling connection and perform a new registration procedure as specified in subclause 5.5.1.3.2. …………….. UE locally releases when T3540 expires before SS transmits an RRCRelease message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Shorten waiting time to 8 seconds at step 12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1  Overview	3

2 Corrections required	3

2.1 Correction to the Function f_TC_8_1_6_1_2_11_NR5GC_TestBody	3

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Ting Gao

gaoting@starpointcomm.com

- Corrections required

  - Correction to the Function f_TC_8_1_6_1_2_11_NR5GC_TestBody

| Function name | f_TC_8_1_6_1_2_11_NR5GC_TestBody |
| --- | --- |
| Reason for change | As per TS 24.501, clause 5.3.1.3: 5.3.1.3 Release of the N1 NAS signalling connection …………….. To allow the network to release the N1 NAS signalling connection, the UE: a) shall start the timer T3540 if the UE receives any of the 5GMM cause values #7, #11, #12, #13, #15, #27, #31, #62, #72, #73, #74, #75, #76 and the UE does not consider the received 5GMM cause value as abnormal case as specified in subclauses 5.5.1.2.7, 5.5.1.3.7 and 5.5.2.3.4; b) shall start the timer T3540 for a UE in 3GPP access if: 1) the UE receives a REGISTRATION ACCEPT message which does not include a Pending NSSAI IE or UE radio capability ID deletion indication IE; 2) the UE has set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message; …………….. Upon expiry of T3540, - in cases a), b), f), g), h), i), j) and k) the UE shall locally release the established N1 NAS signalling connection; - in cases c) and d) the UE shall locally release the established N1 NAS signalling connection and the UE shall initiate the registration procedure as described in subclause 5.5.1.3 or 5.6.1.5; or - in case e), the UE shall locally release the established N1 NAS signalling connection and perform a new registration procedure as specified in subclause 5.5.1.3.2. …………….. UE locally releases when T3540 expires before SS transmits an RRCRelease message. |
| Summary of change | Shorten waiting time to 8 seconds at step 12. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_2_11_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5s220089 sic@ var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var PLMN_Identity v_PlmnID_NRCell11 := f_NR_CellInfo_GetPLMN (nr_Cell11); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var CellIdentity v_CellIdentity_NRCell11 := f_NR_CellInfo_GetCellIdentity(nr_Cell11); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var TrackingAreaCode v_TAC_NRCell11 := f_NR_CellInfo_GetTAC (nr_Cell11); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var boolean v_EntryFound_NR_Cell_1 := false; var boolean v_EntryFound_NR_Cell_11 := false; var integer i:=0; var float v_WaitForLogging := 10.0; var float v_WaitNoReaction := 15.0; ... //@siclog "Step 7-11" siclog@ // Steps 1 to 5 of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 are performed on NR Cell 11 f_NR5GC_MobilityRegistration(nr_Cell11, -,-,-,-, noRrcConnectionRelease); //@siclog "Step 12" siclog@ // Wait 15 seconds. Note: It is checked that UE does not perform the logging while in RRC_CONNECTED state f_Delay(v_WaitNoReaction); //@siclog "Step 13" siclog@ // The SS transmits an RRCRelease message to move the UE to RRC_INACTIVE state. f_NR_RRCRelease(nr_Cell11); ... }

After Change:

function f_TC_8_1_6_1_2_11_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5s220089 sic@ var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var PLMN_Identity v_PlmnID_NRCell11 := f_NR_CellInfo_GetPLMN (nr_Cell11); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var CellIdentity v_CellIdentity_NRCell11 := f_NR_CellInfo_GetCellIdentity(nr_Cell11); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var TrackingAreaCode v_TAC_NRCell11 := f_NR_CellInfo_GetTAC (nr_Cell11); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var boolean v_EntryFound_NR_Cell_1 := false; var boolean v_EntryFound_NR_Cell_11 := false; var integer i:=0; var float v_WaitForLogging := 10.0; var float v_WaitNoReaction := 15.0; ... //@siclog "Step 7-11" siclog@ // Steps 1 to 5 of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 are performed on NR Cell 11 f_NR5GC_MobilityRegistration(nr_Cell11, -,-,-,-, noRrcConnectionRelease); //@siclog "Step 12" siclog@ // Wait 15 seconds. Note: It is checked that UE does not perform the logging while in RRC_CONNECTED state f_Delay(8.0); //@siclog "Step 13" siclog@ // The SS transmits an RRCRelease message to move the UE to RRC_INACTIVE state. f_NR_RRCRelease(nr_Cell11); ... }