3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230166

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2978 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT test case 8.1.4.2.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Prose 36.508 Table 4.7.2-27: TRACKING AREA UPDATE REQUEST is saying EPS bearer context status can be present or not. Prose CR R5-227426 change 1 is not included on TTCN “the UE may include EPS Bearer Context Status which has EPS Bearer ID 10 and EPS Bearer ID 7 as both active, or the UE may not include this IE at all as this is optional.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added if present for cr_EPS_BearerContextStatus_Any |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody () |
| --- | --- |
| Reason for change | Prose 36.508 Table 4.7.2-27: TRACKING AREA UPDATE REQUEST is saying EPS bearer context status can be present or not. Prose CR R5-227426 change 1 is not included on TTCN “the UE may include EPS Bearer Context Status which has EPS Bearer ID 10 and EPS Bearer ID 7 as both active, or the UE may not include this IE at all as this is optional.” |
| Summary of change | Added if present for cr_EPS_BearerContextStatus_Any |
| TTCN module | IRAT_Handover_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var template (value) MobileIdentity v_MappedGUTI; -----skipping code-- // @siclog "Step 2" siclog@ SRB.receive( car_SRB1_RrcPdu_IND( eutra_Cell1, cr_RRCConnectionReconfigurationComplete_r15)) -> value v_ReceivedMsg; f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_ReceivedMsg.Signalling.Rrc.Dcch.message_.c1.rrcConnectionReconfigurationComplete.criticalExtensions.rrcConnectionReconfigurationComplete_r8.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.scg_ConfigResponseNR_r15, "Complete")); /* SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the indicated security key and the indicated security algorithms */ f_EUTRA_PreliminaryPass( __FILE__, __LINE__, "Step 2" ); //Configure UL grant configuration f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def)); // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), v_SecurityRx.KSI, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, -, tsc_NasKsi_MappedSecurityContext, cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent, -, -, cr_EPS_BearerContextStatus_Any))))-> value v_ReceivedMsg; // @sic R5s221153 sic@ // @siclog "Step 4-5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_ReceivedMsg.Signalling.Nas[0], -, noRrcConnectionRelease, -, true); // @siclog "Step 6" siclog@ v_EPSBearerId := f_Get_EPSBearerIdForAPNType(Internet_DNN); if (isvalue(v_EPSBearerId)) { v_RbId_DRB := cs_RadioBearerId_DRB((valueof(v_EPSBearerId) - 4)); // @sic R5s220853 sic@ f_EUTRA_CheckDataPath(eutra_Cell1, PDCP, defaultBearer, "Step 6", v_RbId_DRB, -, -, f_GetMultiPdnIndex (Internet_DNN)); } // @sic R5s220853 sic@ f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger ("Check Data")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Step 7"); }

After Change

function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var template (value) MobileIdentity v_MappedGUTI; ----skipping code // @siclog "Step 2" siclog@ SRB.receive( car_SRB1_RrcPdu_IND( eutra_Cell1, cr_RRCConnectionReconfigurationComplete_r15)) -> value v_ReceivedMsg; f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_ReceivedMsg.Signalling.Rrc.Dcch.message_.c1.rrcConnectionReconfigurationComplete.criticalExtensions.rrcConnectionReconfigurationComplete_r8.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.scg_ConfigResponseNR_r15, "Complete")); /* SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the indicated security key and the indicated security algorithms */ f_EUTRA_PreliminaryPass( __FILE__, __LINE__, "Step 2" ); //Configure UL grant configuration f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def)); // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), v_SecurityRx.KSI, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, -, tsc_NasKsi_MappedSecurityContext, cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent, -, -, cr_EPS_BearerContextStatus_Any ifpresent))))-> value v_ReceivedMsg; // @sic R5s221153 sic@ // @siclog "Step 4-5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_ReceivedMsg.Signalling.Nas[0], -, noRrcConnectionRelease, -, true); // @siclog "Step 6" siclog@ v_EPSBearerId := f_Get_EPSBearerIdForAPNType(Internet_DNN); if (isvalue(v_EPSBearerId)) { v_RbId_DRB := cs_RadioBearerId_DRB((valueof(v_EPSBearerId) - 4)); // @sic R5s220853 sic@ f_EUTRA_CheckDataPath(eutra_Cell1, PDCP, defaultBearer, "Step 6", v_RbId_DRB, -, -, f_GetMultiPdnIndex (Internet_DNN)); } // @sic R5s220853 sic@ f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger ("Check Data")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Step 7"); }