3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230169

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2980 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EN-DC CA test cases 8.2.4.2.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For Step 7, PSCell is changed to NR Cell 12 without any SCell. Current implementation is not aligned with prose as it’s setting the SCell to NR Cell 2 incorrectly. - Both functions f_NR_ConfigPCell_AddRelSCells() and f_NR_Rel_SCell_Common() expect ServingCell index, not SCell index. In addition to this, Steps 2/5/7 are not aligned with prose Tables 8.2.4.2.1.1.3.3-3/7/10 which all state that servCellIndex == 1, not 2. This must be corrected. - Following Change 1, passing omit to p_TargetSCellId of type NR_CellId_Type will cause unintended behavior in current implementation. The proper data type should be used to pass an empty list of SCells. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove NR Cell 2 as target SCell in Step 7 - Use a different constant (tsc_PCellIndex_1) to set ServCellIndex to 1. This is in line with similar test cases 8.2.4.3.1.x. - Check the presence of p_TargetSCellId to determine if an empty list of SCells (NR_CellIdList_Type) should be passed on to f_NR_ConfigPCell_AddRelSCells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2.1.x.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody() |
| --- | --- |
| Reason for change | For Step 7, PSCell is changed to NR Cell 12 without any SCell. Current implementation is not aligned with prose as it’s setting the SCell to NR Cell 2 incorrectly. |
| Summary of change | Remove NR Cell 2 as target SCell in Step 7 |
| TTCN module | RRC_CarrierAggregation_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; ******************** *** SKIPPED CODE *** ******************** //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? //@sic R5s220809 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; f_NR_ReconfigSCellToNormalCell(p_SCell2, v_RadioBearerList); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, p_PSCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //@sic R5s221199 sic@ }

After Change:

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; ******************** *** SKIPPED CODE *** ******************** //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? //@sic R5s220809 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; f_NR_ReconfigSCellToNormalCell(p_SCell2, v_RadioBearerList); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, omit, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //@sic R5s221199 sic@ }

## Change 2

| Function name | function f_NR_RRC_PSCellChangeCA_InterCell() |
| --- | --- |
| Reason for change | - Both functions f_NR_ConfigPCell_AddRelSCells() and f_NR_Rel_SCell_Common() expect ServingCell index, not SCell index. In addition to this, Steps 2/5/7 are not aligned with prose Tables 8.2.4.2.1.1.3.3-3/7/10 which all state that servCellIndex == 1, not 2. This must be corrected. - Following Change 1, passing omit to p_TargetSCellId of type NR_CellId_Type will cause unintended behavior in current implementation. The proper data type should be used to pass an empty list of SCells. |
| Summary of change | - Use a different constant (tsc_PCellIndex_1) to set ServCellIndex to 1. This is in line with similar test cases 8.2.4.3.1.x. - Check the presence of p_TargetSCellId to determine if an empty list of SCells (NR_CellIdList_Type) should be passed on to f_NR_ConfigPCell_AddRelSCells. |
| TTCN module | RRC_CarrierAggregation_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_RRC_PSCellChangeCA_InterCell(NR_CellId_Type p_SourcePSCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPSCellId, NR_CellId_Type p_TargetSCellId, boolean p_RBConfig_KeyChange := true, template (omit) RadioBearerConfig p_RadioBearerConfig, template (value) CellGroupConfig p_SCGConfig, template (value) NR_RadioBearerList_Type p_RadioBearerToReleaseList ) runs on NR_BASE_PTC { var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := f_NR_GetRACHProcedureConfigPSCell( p_TargetPSCellId, -); // Steps 1 to 6 f_NR_RRC_PSCellChange_InterCell_Steps1To6(p_SourcePSCellId, p_TargetPSCellId, -, //tsc_C_RNTI_Value3 p_RBConfig_KeyChange, false, //p_IsSrb3Configured, v_RachProcedureConfig ); //Step 7. NR Target PSCell: Configure Target PSCell as PSCell. // NOTE 5: The FollowOnFlag is set to true in the ASP reconfiguring PSCell. f_NR_ConfigPCell_AddRelSCells (p_TargetPSCellId, {p_TargetSCellId}, tsc_SCellIndex_2, cs_TimingInfo_Now, tsc_C_RNTI_Value3, -, tsc_NoCnfReq, tsc_FollowOnFlag ); //@sic R5-206369 sic@ //Step 8. NR Target SCell: Configure Target SCell as SCell with new PSCell association. if ( ispresent(p_SCGConfig.sCellToAddModList)) { //@sic R5s220809 change 2 sic@ // In step 7 there is no SCell f_NR_ConfigSCell(p_TargetSCellId, p_SCGConfig, cs_TimingInfo_Now); } // Step 9. E-UTRA Cell: Send RRCConnectionReconfiguration. // Step 10. E-UTRA Cell: Receive RRCConnectionReconfigurationComplete. f_NR_SendRRCReconfigurationContentsToEUTRA(p_RadioBearerConfig, p_SCGConfig, -, -, -, reconfigurationWithSync_CheckRACH,p_TargetPSCellId); //@sic R5s221199 sic@ //Anritsu TTCN CR R5s230131 // Step 11. NR Target PSCell: Inform the SS about completion of the PSCell change (e.g. to trigger PDCP STATUS REPORT PDU). f_NR_SS_PdcpHandoverCtrl(p_TargetPSCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetPSCellId)); // Step 12. NR Source PSCell: If source PSCell is not the same as target SCell, Release SRB3 (if necessary) and DRBs, reconfigure the cell as a normal cell. if (p_SourcePSCellId != p_TargetSCellId) { //@sic R5-204475 sic@ f_NR_SS_CommonRadioBearerConfig(p_SourcePSCellId, p_RadioBearerToReleaseList); } //Step 13. NR Source SCell: If source SCell is not the same as target SCell or target PSCell, reconfigure the cell as a normal cell //NOTE 6: The FollowOnFlag is set to true in the ASP reconfiguring PSCell if (p_SourceSCellId != p_TargetPSCellId) { f_NR_Rel_SCell_Common(p_SourcePSCellId, p_SourceSCellId, tsc_SCellIndex_2, -, -, tsc_NoCnfReq, tsc_FollowOnFlag ); //@sic R5-206369 sic@ } }

After Change:

function f_NR_RRC_PSCellChangeCA_InterCell(NR_CellId_Type p_SourcePSCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPSCellId, NR_CellId_Type p_TargetSCellId, boolean p_RBConfig_KeyChange := true, template (omit) RadioBearerConfig p_RadioBearerConfig, template (value) CellGroupConfig p_SCGConfig, template (value) NR_RadioBearerList_Type p_RadioBearerToReleaseList ) runs on NR_BASE_PTC { var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := f_NR_GetRACHProcedureConfigPSCell( p_TargetPSCellId, -); var NR_CellIdList_Type v_SCellList := {}; // Steps 1 to 6 f_NR_RRC_PSCellChange_InterCell_Steps1To6(p_SourcePSCellId, p_TargetPSCellId, -, //tsc_C_RNTI_Value3 p_RBConfig_KeyChange, false, //p_IsSrb3Configured, v_RachProcedureConfig ); //Step 7. NR Target PSCell: Configure Target PSCell as PSCell. // NOTE 5: The FollowOnFlag is set to true in the ASP reconfiguring PSCell. if (isvalue (p_TargetSCellId)) { v_SCellList := {p_TargetSCellId}; } f_NR_ConfigPCell_AddRelSCells (p_TargetPSCellId, v_SCellList, tsc_PCellIndex_1, cs_TimingInfo_Now, tsc_C_RNTI_Value3, -, tsc_NoCnfReq, tsc_FollowOnFlag ); //@sic R5-206369 sic@ //Step 8. NR Target SCell: Configure Target SCell as SCell with new PSCell association. if ( ispresent(p_SCGConfig.sCellToAddModList)) { //@sic R5s220809 change 2 sic@ // In step 7 there is no SCell f_NR_ConfigSCell(p_TargetSCellId, p_SCGConfig, cs_TimingInfo_Now); } // Step 9. E-UTRA Cell: Send RRCConnectionReconfiguration. // Step 10. E-UTRA Cell: Receive RRCConnectionReconfigurationComplete. f_NR_SendRRCReconfigurationContentsToEUTRA(p_RadioBearerConfig, p_SCGConfig, -, -, -, reconfigurationWithSync_CheckRACH,p_TargetPSCellId); //@sic R5s221199 sic@ //Anritsu TTCN CR R5s230131 // Step 11. NR Target PSCell: Inform the SS about completion of the PSCell change (e.g. to trigger PDCP STATUS REPORT PDU). f_NR_SS_PdcpHandoverCtrl(p_TargetPSCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetPSCellId)); // Step 12. NR Source PSCell: If source PSCell is not the same as target SCell, Release SRB3 (if necessary) and DRBs, reconfigure the cell as a normal cell. if (p_SourcePSCellId != p_TargetSCellId) { //@sic R5-204475 sic@ f_NR_SS_CommonRadioBearerConfig(p_SourcePSCellId, p_RadioBearerToReleaseList); } //Step 13. NR Source SCell: If source SCell is not the same as target SCell or target PSCell, reconfigure the cell as a normal cell //NOTE 6: The FollowOnFlag is set to true in the ASP reconfiguring PSCell if (p_SourceSCellId != p_TargetPSCellId) { f_NR_Rel_SCell_Common(p_SourcePSCellId, p_SourceSCellId, tsc_PCellIndex_1, -, -, tsc_NoCnfReq, tsc_FollowOnFlag ); //@sic R5-206369 sic@ } }