3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230174

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4719 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE PLMN Selection testcase 6.1.1.2a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step27B, UE moves from VPLMN(Cell_11) to HPLMN(Cell_1) and UE should be allowed to perform IMS Re-registration procedure after TAU procedure as per TS 36.508 subclause 4.5A.30. But in the present ttcn implementation, this is not handled. At Step29, UE moves from HPLMN(Cell_1) to VPLMN(Cell_4) and UE should be allowed to perform IMS Re-registration procedure after TAU procedure as per TS 36.508 subclause 4.5A.30. But in the present ttcn implementation, this is not handled. Note: The corresponding prose CR(R5-230591) will be raised at RAN5#98 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new optional steps for performing IMS Re-registration procedure after Step 27B and Step 34 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_6_1_1_2a_EUTRA |
| --- | --- |
| Reason for change | At Step27B, UE moves from VPLMN(Cell_11) to HPLMN(Cell_1) and UE should be allowed to perform IMS Re-registration procedure after TAU procedure as per TS 36.508 subclause 4.5A.30. But in the present ttcn implementation this is not handled. At Step29, UE moves from HPLMN(Cell_1) to VPLMN(Cell_4) and UE should be allowed to perform IMS Re-registration procedure after TAU procedure as per TS 36.508 subclause 4.5A.30. But in the present ttcn implementation, this is not handled. |
| Summary of change | Added a new optional step for performing IMS Re-registration procedure after Step 27B and Step 34 |
| TTCN module | LTE\6_1\Idle_PLMNSelection.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_6_1_1_2a_EUTRA() runs on EUTRA_PTC

{

//<=== SKIPPED ===>

//@siclog "Step 22 - 26" siclog@

//Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11.

f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq (eutra_Cell11);

//@sic "Step 27Aa1" sic@

//The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

//@sic R5-201099 sic@

f_EUTRA_WaitForIMSReRegister(eutra_Cell11);

//@siclog "Step 27" siclog@

//Check: Is PLMN2 indicated by the UE?

//@sic R5-203812 sic@

f_UT_PLMN_Check(UT, tsc_UT_PLMN2);

//@verdict pass PLMN2 indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.2a Step 27");

//@siclog "Step 27A" siclog@

//SS adjusts cell levels according to row T5 of table 6.1.1.2a.3.2-1 and change PLMN identifiers of Cell 11 to PLMN4

f_EUTRA_SetCellPowerList(v_CellPowerList_T2toT5);

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_Cell11, cs_HPLMN_004_31);   //PLMN4 according to 36.523-1 Table 6.0.1-1

f_EUTRA_SS_ConfigureSysinfo(eutra_Cell11);  //Cell 11 is off - no need to page UE and no activation time

//@siclog "Step 27B" siclog@

//The generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1.

//NOTE: The UE performs a TAU procedure and the RRC connection is released.

f_EUTRA_TrackingAreaUpdate(eutra_Cell1);

//@siclog "Step 28" siclog@

//SS adjusts cell levels according to row T3 of table 6.1.1.2a.3.2-1

f_EUTRA_SetCellPowerList(v_CellPowerList_T5toT3);

t_IdleMode_GenericTimer.start;

//@siclog "Step 29" siclog@

//Check: Does the UE send an RRCConnectionRequest on Cell 4?

f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell4);   // @sic R5-120721 change 3 sic@

t_IdleMode_GenericTimer.stop;

//@verdict pass RRCConnectionRequest message received on Cell 4

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.2a Step 29");

//@siclog "Step 30 - 34" siclog@

//Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 4.

f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq (eutra_Cell4);

//@siclog "Step 35" siclog@

//Check: Is PLMN3 indicated by the UE?

f_UT_PLMN_Check(UT, tsc_UT_PLMN3);

//<=== SKIPPED ===>

}

After change:

function f_TC_6_1_1_2a_EUTRA() runs on EUTRA_PTC

{

//<=== SKIPPED ===>

//@siclog "Step 22 - 26" siclog@

//Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11.

f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq (eutra_Cell11);

//@sic "Step 27Aa1" sic@

//The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

//@sic R5-201099 sic@

f_EUTRA_WaitForIMSReRegister(eutra_Cell11);

//@siclog "Step 27" siclog@

//Check: Is PLMN2 indicated by the UE?

//@sic R5-203812 sic@

f_UT_PLMN_Check(UT, tsc_UT_PLMN2);

//@verdict pass PLMN2 indicated by UE

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.2a Step 27");

//@siclog "Step 27A" siclog@

//SS adjusts cell levels according to row T5 of table 6.1.1.2a.3.2-1 and change PLMN identifiers of Cell 11 to PLMN4

f_EUTRA_SetCellPowerList(v_CellPowerList_T2toT5);

f_EUTRA_CellInfo_SetPLMN_1Entry(eutra_Cell11, cs_HPLMN_004_31);   //PLMN4 according to 36.523-1 Table 6.0.1-1

f_EUTRA_SS_ConfigureSysinfo(eutra_Cell11);  //Cell 11 is off - no need to page UE and no activation time

//@siclog "Step 27B" siclog@

//The generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1.

//NOTE: The UE performs a TAU procedure and the RRC connection is released.

f_EUTRA_TrackingAreaUpdate(eutra_Cell1);

//@sic "Step 27Ca1" sic@

//The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

f_EUTRA_WaitForIMSReRegister(eutra_Cell1);

//@siclog "Step 28" siclog@

//SS adjusts cell levels according to row T3 of table 6.1.1.2a.3.2-1

f_EUTRA_SetCellPowerList(v_CellPowerList_T5toT3);

t_IdleMode_GenericTimer.start;

//@siclog "Step 29" siclog@

//Check: Does the UE send an RRCConnectionRequest on Cell 4?

f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell4);   // @sic R5-120721 change 3 sic@

t_IdleMode_GenericTimer.stop;

//@verdict pass RRCConnectionRequest message received on Cell 4

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.2a Step 29");

//@siclog "Step 30 - 34" siclog@

//Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 4.

f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq (eutra_Cell4);

//@sic "Step 34Aa1" sic@

//The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

f_EUTRA_WaitForIMSReRegister(eutra_Cell4);

//@siclog "Step 35" siclog@

//Check: Is PLMN3 indicated by the UE?

f_UT_PLMN_Check(UT, tsc_UT_PLMN3);

//<=== SKIPPED ===>

}