3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230178

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4695 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for function f_EUTRA_ResetIPandIMS() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-12-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the function f_EUTRA_ResetIPandIMS(), the function f_EUTRA_ResetIMS() is called which will send a signal cms_IPCAN_IMS_Reset to IMS PTC but will not wait for any confirmation. This trigger will cause IMS PTC to reset including sending IMS_SecurityRelease_REQ to IP PTC. In the case where IMS servers and security context are still active, this will cause security contexts and sockets to be closed which can take some time. This can happen especially in the case where the UE is switched off during a test case but does not perform the (optional) IMS de-registration procedure, and then EUTRA PTC calls f_EUTRA_ResetIPandIMS() before switching the UE back on again at at a subsequent step. Because EUTRA PTC is not waiting for a response after resetting IMS, it will continue and call f_IP_Handling_StartPDN() which will send startPDN signal to IP PTC which may still be processing the IMS security release. This can cause IP PTC to lock up as sockets are being closed and opened at the same time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a short delay following IMS reset, to allow IMS and IP reset procedures to complete before configuring new PDN handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE with IMS enabled may fail some test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B20202-09_D22wk43 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_EUTRA_ResetIPandIMS() |
| --- | --- |
| Reason for change | During the function f_EUTRA_ResetIPandIMS(), the function f_EUTRA_ResetIMS() is called which will send a signal cms_IPCAN_IMS_Reset to IMS PTC but will not wait for any confirmation. This trigger will cause IMS PTC to reset including sending IMS_SecurityRelease_REQ to IP PTC. In the case where IMS servers and security context are still active, this will cause security contexts and sockets to be closed which can take some time. This can happen in the case where the UE is switched off during a test case but does not perform the (optional) IMS de-registration procedure, and then EUTRA PTC calls f_EUTRA_ResetIPandIM() before swiwtching the UE back on again at at a subsequent step. Because EUTRA PTC is not waiting for a response after resetting IMS, it will continue and call f_IP_Handling_StartPDN() which will send startPDN signal to IP PTC which may still be processing the IMS security release. This can cause IP PTC to lock up as sockets are being closed and opened at the same time. |
| Summary of change | Add a short delay following IMS reset, to allow IMS and IP reset procedures to complete before configuring new PDN handling. |
| TTCN module | EUTRA_InitialRegistration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA_ResetIPandIMS(EUTRA_CellId_Type p_CellId, PDN_Index_Type p_PdnIndex := PDN_1, IP_DrbId_Type p_DrbId := tsc_DRB1, boolean p_ResetIMS := true) runs on EUTRA_PTC { if (p_ResetIMS) { f_EUTRA_ResetIMS(p_PdnIndex); /* @sic R5s150031 sic@ */ } f_IP_Routing_RestoreRoutingTable(IP); /* @sic R5s201313 sic@ */ f_IP_Handling_StartPDN(IP, p_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, p_DrbId)); }

After Change:

function f_EUTRA_ResetIPandIMS(EUTRA_CellId_Type p_CellId, PDN_Index_Type p_PdnIndex := PDN_1, IP_DrbId_Type p_DrbId := tsc_DRB1, boolean p_ResetIMS := true) runs on EUTRA_PTC { if (p_ResetIMS) { f_EUTRA_ResetIMS(p_PdnIndex); /* @sic R5s150031 sic@ */ f_Delay(3.0); } f_IP_Routing_RestoreRoutingTable(IP); /* @sic R5s201313 sic@ */ f_IP_Handling_StartPDN(IP, p_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, p_DrbId)); }