3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230191

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0884 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS NR5GC test case 6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - From step 4 the P-CSCF has changed, so this has to be reflected in the RoutingInfo - without the suggested change this is especially problem for UDP, because of how fl_IP_IMS_Server_SendDL populates local address from RoutingInfo in case of UDP as opposed to TCP where it is taken from ImsServer.TCP which is correctly updated thanks to f_IMS_RestartPCSCF. - It is possible that UE changes from using IPv6 to IPv4 address (or vice versa) after receiving step 6 “Service Unavailable”. To reflect such potential change the RoutingInfo needs to be updated. Similarly to above change this is especially problem for UDP because of how fl_IP_IMS_Server_SendDL works with the routing information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated RoutingInfo from the step 4 REGISTER. - Updated RoutingInfo from the step 6 REGISTER. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SIP messages for a UE using UDP protocol may be routed incorrectly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_6_2_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - From step 4 the P-CSCF has changed, so this has to be reflected in the RoutingInfo - without the suggested change this is especially problem for UDP, because of how fl_IP_IMS_Server_SendDL populates local address from RoutingInfo in case of UDP as opposed to TCP where it is taken from ImsServer.TCP which is correctly updated thanks to f_IMS_RestartPCSCF. - It is possible that UE changes from using IPv6 to IPv4 address (or vice versa) after receiving step 6 “Service Unavailable”. To reflect such potential change the RoutingInfo needs to be updated. Similarly to above change this is especially problem for UDP because of how fl_IP_IMS_Server_SendDL works with the routing information. |
| Summary of change | - Updated RoutingInfo from the step 4 REGISTER. - Updated RoutingInfo from the step 6 REGISTER. |
| TTCN module | IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_2_NR5GC_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterReq; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var template (omit) RetryAfter v_RetryAfterForREGISTER; var integer v_RetryAfterForPublish := 60; // as per "503 Service Unavailable" default message contents (34.229-1 A.4.2), var integer v_NumOfIPAddressesToAllocate := 2; // @sic R5s210720 sic@ var charstring v_RegisterExpiration; var integer v_Cseq; timer t_Timer; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security, -, -, v_NumOfIPAddressesToAllocate); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); // @siclog "Step 2" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); // @siclog "Step 3" siclog@ v_RetryAfterForREGISTER := omit; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); f_IMS_RestartPCSCF(PDN_1, px_IPv4_Address_Other, px_IPv6_Address_Other); /* @sic R5-213534: support of 2nd P-CSCF, R5-217830 sic@ NOTE: IP addresses need to be consistent with what is used as 2nd P-CSCF at NAS signalling */ f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 4" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ v_RetryAfterForREGISTER := cs_RetryAfter(10); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); t_Timer.start(f_IMS_SetTimerToleranceMin(10.0)); /* @sic R5s210720: timer to be started before RRC connection release sic@ */ f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling); f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 6" siclog@ t_Timer.timeout; /* @sic R5s210720: t_Timer.start/stop instead of f_Delay sic@ */ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_Cseq := fl_TC_6_2_CheckGetCseq(v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ v_RegisterExpiration := "800000"; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_RegisterExpiration)))); // @siclog "Step 8" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(-, -, -, v_RegisterExpiration); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; fl_TC_6_2_CheckGetCseq(v_RegisterReq, v_Cseq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 10" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, -, v_RegisterExpiration); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 11" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_RegisterExpiration)))); // @siclog "Step 12 - 15" siclog@ f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, tsc_IMS_PublishDuringSubscribeForRegEvent, v_RetryAfterForPublish); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling); }

After Change:

function f_TC_6_2_NR5GC_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterReq; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var template (omit) RetryAfter v_RetryAfterForREGISTER; var integer v_RetryAfterForPublish := 60; // as per "503 Service Unavailable" default message contents (34.229-1 A.4.2), var integer v_NumOfIPAddressesToAllocate := 2; // @sic R5s210720 sic@ var charstring v_RegisterExpiration; var integer v_Cseq; timer t_Timer; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security, -, -, v_NumOfIPAddressesToAllocate); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); // @siclog "Step 2" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); // @siclog "Step 3" siclog@ v_RetryAfterForREGISTER := omit; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); f_IMS_RestartPCSCF(PDN_1, px_IPv4_Address_Other, px_IPv6_Address_Other); /* @sic R5-213534: support of 2nd P-CSCF, R5-217830 sic@ NOTE: IP addresses need to be consistent with what is used as 2nd P-CSCF at NAS signalling */ f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 4" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); // @siclog "Step 5" siclog@ v_RetryAfterForREGISTER := cs_RetryAfter(10); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); t_Timer.start(f_IMS_SetTimerToleranceMin(10.0)); /* @sic R5s210720: timer to be started before RRC connection release sic@ */ f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling); f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 6" siclog@ t_Timer.timeout; /* @sic R5s210720: t_Timer.start/stop instead of f_Delay sic@ */ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_Cseq := fl_TC_6_2_CheckGetCseq(v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ v_RegisterExpiration := "800000"; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_RegisterExpiration)))); // @siclog "Step 8" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(-, -, -, v_RegisterExpiration); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; fl_TC_6_2_CheckGetCseq(v_RegisterReq, v_Cseq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 10" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, -, v_RegisterExpiration); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 11" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_RegisterExpiration)))); // @siclog "Step 12 - 15" siclog@ f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, tsc_IMS_PublishDuringSubscribeForRegEvent, v_RetryAfterForPublish); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling); }