3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230201

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4720 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EUTRA CA frequencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2023-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The DL EARFCN for EUTRA band 71 is currently incorrect for CA bands. The mid-range EARCFN value should be 68761 according to 36.508 Table 4.3.1.1.71-1 but in f_EUTRA_CA_InitFrequenciesInterBand() it is implemented as as 68671. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the value of DL EARFCN for CA band 71 to 68761. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect EARFCN used in CA test configurations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | CA test cases using band 71 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_EUTRA_CA_InitFrequenciesInterBand() |
| --- | --- |
| Reason for change | The DL EARFCN for EUTRA band 71 is currently incorrect for CA bands. The mid-range EARCFN value should be 68761 according to 36.508 Table 4.3.1.1.71-1 but in f_EUTRA_CA_InitFrequenciesInterBand() it is implemented as as 68671. |
| Summary of change | Corrected the value of DL EARFCN for CA band 71 to 68761. |
| TTCN module | EUTRA_CellInfoInit_CA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA_CA_InitFrequenciesInterBand(CA_BandCombination_Type p_CA_FrequencyBand) return Frequency_fList_CAInterBand_Type { // @sic R5-155910 New CA band combination pixit sic@ var template(value) Frequency_fList_CAInterBand_Type v_Freq := cs_FrequencyCAInterBand_Dummy; //@sic R5s150438 sic@ select (p_CA_FrequencyBand) { ……………………………………………… case(CA_66A_71A){ //CA_66A-71A @sic R5-184771 sic@ v_Freq.f1.dl_CarrierFreq := maxEARFCN; v_Freq.f1.ul_CarrierFreq := maxEARFCN; v_Freq.f1.dl_CarrierFreq_v9e0 := 66886; v_Freq.f1.ul_CarrierFreq_v9e0 := 132422; v_Freq.f1_DL_ChBandwidth := n100; v_Freq.f1_UL_ChBandwidth := n100; v_Freq.f2.dl_CarrierFreq := maxEARFCN; v_Freq.f2.ul_CarrierFreq := maxEARFCN; v_Freq.f2.dl_CarrierFreq_v9e0 := 67036; v_Freq.f2.ul_CarrierFreq_v9e0 := 132572; v_Freq.f2_DL_ChBandwidth := n100; v_Freq.f2_UL_ChBandwidth := n100; v_Freq.f5.dl_CarrierFreq := maxEARFCN; v_Freq.f5.ul_CarrierFreq := maxEARFCN; v_Freq.f5.dl_CarrierFreq_v9e0 := 68671; v_Freq.f5.ul_CarrierFreq_v9e0 := 133297; v_Freq.f5_DL_ChBandwidth := n50; v_Freq.f5_UL_ChBandwidth := n50; v_Freq.f6.dl_CarrierFreq := maxEARFCN; v_Freq.f6.ul_CarrierFreq := maxEARFCN; v_Freq.f6.dl_CarrierFreq_v9e0 := 68886; v_Freq.f6.ul_CarrierFreq_v9e0 := 133422; v_Freq.f6_DL_ChBandwidth := n50; v_Freq.f6_UL_ChBandwidth := n50; } case(CA_66A_71A_SwitchAllocation){ //CA_66A-71A SwitchAllocation@sic R5-184771 sic@ v_Freq.f5.dl_CarrierFreq := maxEARFCN; v_Freq.f5.ul_CarrierFreq := maxEARFCN; v_Freq.f5.dl_CarrierFreq_v9e0 := 66886; v_Freq.f5.ul_CarrierFreq_v9e0 := 132422; v_Freq.f5_DL_ChBandwidth := n100; v_Freq.f5_UL_ChBandwidth := n100; v_Freq.f6.dl_CarrierFreq := maxEARFCN; v_Freq.f6.ul_CarrierFreq := maxEARFCN; v_Freq.f6.dl_CarrierFreq_v9e0 := 67036; v_Freq.f6.ul_CarrierFreq_v9e0 := 132572; v_Freq.f6_DL_ChBandwidth := n100; v_Freq.f6_UL_ChBandwidth := n100; v_Freq.f1.dl_CarrierFreq := maxEARFCN; v_Freq.f1.ul_CarrierFreq := maxEARFCN; v_Freq.f1.dl_CarrierFreq_v9e0 := 68671; v_Freq.f1.ul_CarrierFreq_v9e0 := 133297; v_Freq.f1_DL_ChBandwidth := n50; v_Freq.f1_UL_ChBandwidth := n50; v_Freq.f2.dl_CarrierFreq := maxEARFCN; v_Freq.f2.ul_CarrierFreq := maxEARFCN; v_Freq.f2.dl_CarrierFreq_v9e0 := 68886; v_Freq.f2.ul_CarrierFreq_v9e0 := 133422; v_Freq.f2_DL_ChBandwidth := n50; v_Freq.f2_UL_ChBandwidth := n50; } ....................... //Assign EARFCN_r9 Handle band extension @sic R5-165892 sic@ v_Freq := fl_AssignInterBandCarrierFreq_r9(v_Freq, p_CA_FrequencyBand); return valueof(v_Freq); }

After Change:

function f_EUTRA_CA_InitFrequenciesInterBand(CA_BandCombination_Type p_CA_FrequencyBand) return Frequency_fList_CAInterBand_Type { // @sic R5-155910 New CA band combination pixit sic@ var template(value) Frequency_fList_CAInterBand_Type v_Freq := cs_FrequencyCAInterBand_Dummy; //@sic R5s150438 sic@ select (p_CA_FrequencyBand) { …………………………… case(CA_66A_71A){ //CA_66A-71A @sic R5-184771 sic@ v_Freq.f1.dl_CarrierFreq := maxEARFCN; v_Freq.f1.ul_CarrierFreq := maxEARFCN; v_Freq.f1.dl_CarrierFreq_v9e0 := 66886; v_Freq.f1.ul_CarrierFreq_v9e0 := 132422; v_Freq.f1_DL_ChBandwidth := n100; v_Freq.f1_UL_ChBandwidth := n100; v_Freq.f2.dl_CarrierFreq := maxEARFCN; v_Freq.f2.ul_CarrierFreq := maxEARFCN; v_Freq.f2.dl_CarrierFreq_v9e0 := 67036; v_Freq.f2.ul_CarrierFreq_v9e0 := 132572; v_Freq.f2_DL_ChBandwidth := n100; v_Freq.f2_UL_ChBandwidth := n100; v_Freq.f5.dl_CarrierFreq := maxEARFCN; v_Freq.f5.ul_CarrierFreq := maxEARFCN; v_Freq.f5.dl_CarrierFreq_v9e0 := 68761; v_Freq.f5.ul_CarrierFreq_v9e0 := 133297; v_Freq.f5_DL_ChBandwidth := n50; v_Freq.f5_UL_ChBandwidth := n50; v_Freq.f6.dl_CarrierFreq := maxEARFCN; v_Freq.f6.ul_CarrierFreq := maxEARFCN; v_Freq.f6.dl_CarrierFreq_v9e0 := 68886; v_Freq.f6.ul_CarrierFreq_v9e0 := 133422; v_Freq.f6_DL_ChBandwidth := n50; v_Freq.f6_UL_ChBandwidth := n50; } case(CA_66A_71A_SwitchAllocation){ //CA_66A-71A SwitchAllocation@sic R5-184771 sic@ v_Freq.f5.dl_CarrierFreq := maxEARFCN; v_Freq.f5.ul_CarrierFreq := maxEARFCN; v_Freq.f5.dl_CarrierFreq_v9e0 := 66886; v_Freq.f5.ul_CarrierFreq_v9e0 := 132422; v_Freq.f5_DL_ChBandwidth := n100; v_Freq.f5_UL_ChBandwidth := n100; v_Freq.f6.dl_CarrierFreq := maxEARFCN; v_Freq.f6.ul_CarrierFreq := maxEARFCN; v_Freq.f6.dl_CarrierFreq_v9e0 := 67036; v_Freq.f6.ul_CarrierFreq_v9e0 := 132572; v_Freq.f6_DL_ChBandwidth := n100; v_Freq.f6_UL_ChBandwidth := n100; v_Freq.f1.dl_CarrierFreq := maxEARFCN; v_Freq.f1.ul_CarrierFreq := maxEARFCN; v_Freq.f1.dl_CarrierFreq_v9e0 := 68761; v_Freq.f1.ul_CarrierFreq_v9e0 := 133297; v_Freq.f1_DL_ChBandwidth := n50; v_Freq.f1_UL_ChBandwidth := n50; v_Freq.f2.dl_CarrierFreq := maxEARFCN; v_Freq.f2.ul_CarrierFreq := maxEARFCN; v_Freq.f2.dl_CarrierFreq_v9e0 := 68886; v_Freq.f2.ul_CarrierFreq_v9e0 := 133422; v_Freq.f2_DL_ChBandwidth := n50; v_Freq.f2_UL_ChBandwidth := n50; } ..................... //Assign EARFCN_r9 Handle band extension @sic R5-165892 sic@ v_Freq := fl_AssignInterBandCarrierFreq_r9(v_Freq, p_CA_FrequencyBand); return valueof(v_Freq); }