3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230202

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2873 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of wk49, PDCP configuration of DRB is set as cs_38508_PDCP_Config(), which sets PDCP SN for DL and UL as len18bits when not(pc_supportOfRedCap_r17). This is contrary to the network configuration where Pdcp configuration of DRB is set as len12bits and against the test purpose “UE in RRC_CONNECTED state with PDCP configured for 12 bit SN” |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDCP configuration of DRB is set to have PDCP SN for DL and UL as len12bits |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.1.1.ENDC, 7.1.3.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_TC_7_1_3_1_1_NR5GC	2

2.2	Correction to the Function f_TC_7_1_3_1_1_ENDC_NR	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_7_1_3_1_1_NR5GC

| Function name | f_TC_7_1_3_1_1_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation of wk49, PDCP configuration of DRB is set as cs_38508_PDCP_Config() , which sets PDCP SN for DL and UL as len18bits when not(pc_supportOfRedCap_r17). This is contrary to the network configuration where Pdcp configuration of DRB is set as len12bits and against the test purpose “UE in RRC_CONNECTED state with PDCP configured for 12 bit SN” |
| Summary of change | PDCP configuration of DRB is set to have PDCP SN for DL and UL as len12bits |
| TTCN module | ttcn\develop\NR5GC\7_1_3\PDCP_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_1_1_NR5GC() runs on NR5GC_PTC {//Maintenance of PDCP sequence numbers / User plane / 12 bit SN var DRB_Identity v_DRBId; var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) DRB_ToAddMod v_DRB_ToAddMod; var template (value) NR_RadioBearer_Type v_SS_Drb_Config; f_NR5GC_Init_NAS(NR_1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //@sic R5s199039 sic@ f_NR_AS_CipheringAlgorithm_Set(nea0); v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_Config := f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, cs_NR_PDCP_RbConfig_TransparentMode(PDCP_SNLength12), -, cs_SDAP_Configuration_None); //@sic R5s199039,R5s200074 sic@ v_DRB_ToAddMod := cs_38508_NRDRB_ToAddMod(f_NR_GetSDAP_InDRBList(v_DRBId), v_DRBId, cs_38508_PDCP_Config(ms300));//@sic R5s199039, R5-227518 sic@ f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, -, -, {v_DRB_ToAddMod}, {v_SS_Drb_Config}); //Initialize PDCP Parameters f_NR_PDCP_Init(v_NR_PDCP_State, PDCP_SNLength12, v_DRBId); f_NR_TestBody_Set(true); f_TC_7_1_3_1_1_NR_TestBody(v_NR_PDCP_State); // @sic R5-211387 sic@ f_NR_TestBody_Set(false); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); //Postamble f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_3_1_1_NR5GC() runs on NR5GC_PTC {//Maintenance of PDCP sequence numbers / User plane / 12 bit SN var DRB_Identity v_DRBId; var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) DRB_ToAddMod v_DRB_ToAddMod; var template (value) NR_RadioBearer_Type v_SS_Drb_Config; f_NR5GC_Init_NAS(NR_1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //@sic R5s199039 sic@ f_NR_AS_CipheringAlgorithm_Set(nea0); v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_Config := f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, cs_NR_PDCP_RbConfig_TransparentMode(PDCP_SNLength12), -, cs_SDAP_Configuration_None); //@sic R5s199039,R5s200074 sic@ v_DRB_ToAddMod := cs_38508_NRDRB_ToAddMod(f_NR_GetSDAP_InDRBList(v_DRBId), v_DRBId, /*cs_38508_PDCP_Config(ms300, len12bits, len12bits)*/ cs_PDCP_Config_Def(cs_38508_PDCP_Config_DRB(ms300, len12bits, len12bits, true_)));//@sic R5s199039, R5-227518 sic@ f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, -, -, {v_DRB_ToAddMod}, {v_SS_Drb_Config}); //Initialize PDCP Parameters f_NR_PDCP_Init(v_NR_PDCP_State, PDCP_SNLength12, v_DRBId); f_NR_TestBody_Set(true); f_TC_7_1_3_1_1_NR_TestBody(v_NR_PDCP_State); // @sic R5-211387 sic@ f_NR_TestBody_Set(false); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); //Postamble f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_ReleaseAllCells(); }

## Correction to the Function f_TC_7_1_3_1_1_ENDC_NR

| Function name | f_TC_7_1_3_1_1_ENDC_NR |
| --- | --- |
| Reason for change | In the current TTCN implementation of wk49, PDCP configuration of DRB is set as cs_38508_PDCP_Config() , which sets PDCP SN for DL and UL as len18bits when not(pc_supportOfRedCap_r17). This is contrary to the network configuration where Pdcp configuration of DRB is set as len12bits and against the test purpose “UE in RRC_CONNECTED state with PDCP configured for 12 bit SN” |
| Summary of change | PDCP configuration of DRB is set to have PDCP SN for DL and UL as len12bits |
| TTCN module | ttcn\develop\ENDC\7_1_3\PDCP_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_1_1_ENDC_NR() runs on ENDC_NR_PTC {//Maintenance of PDCP sequence numbers / User plane / 12 bit SN var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) DRB_ToAddModList v_DRB_ToAddModList; f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR_PDCP_Init (v_NR_PDCP_State, PDCP_SNLength12); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, tsc_NR_DRB2, -, cs_NR_PDCP_RbConfig_TransparentMode(PDCP_SNLength12))}; v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(tsc_NR_DRB2, cs_38508_PDCP_Config())}; //@sic R5-227518 sic@ f_NR_ENDC_ConfigAM_UM(nr_Cell1,1, 0, v_SS_Drb_ConfigList, -, -, v_DRB_ToAddModList); f_NR_TestBody_Set(true); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_TC_7_1_3_1_1_NR_TestBody(v_NR_PDCP_State); // @sic R5-211387 sic@ f_NR_TestBody_Set(false); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s190203 sic@ //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_3_1_1_ENDC_NR() runs on ENDC_NR_PTC {//Maintenance of PDCP sequence numbers / User plane / 12 bit SN var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) DRB_ToAddModList v_DRB_ToAddModList; f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR_PDCP_Init (v_NR_PDCP_State, PDCP_SNLength12); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, tsc_NR_DRB2, -, cs_NR_PDCP_RbConfig_TransparentMode(PDCP_SNLength12))}; v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(tsc_NR_DRB2, cs_PDCP_Config_Def(cs_38508_PDCP_Config_DRB(ms300, len12bits, len12bits, true_)))}; //@sic R5-227518 sic@ f_NR_ENDC_ConfigAM_UM(nr_Cell1,1, 0, v_SS_Drb_ConfigList, -, -, v_DRB_ToAddModList); f_NR_TestBody_Set(true); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_TC_7_1_3_1_1_NR_TestBody(v_NR_PDCP_State); // @sic R5-211387 sic@ f_NR_TestBody_Set(false); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s190203 sic@ //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }