3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230208

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2876 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 11.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - With the implementation of the R5-216188, specifically parallel behaviour in table 11.1.2.3.2-2, the implementation in TTCN is hardconding the expected APN to be of pc_APN_ID_Internet. However depending on the UE setting of the default APN, it may also be of pc_APN_ID_IMS. - The DHCP and ICMPv6 servers don’t have to be started again for this PDN as they are already running (the same PDN has been established already at the beginning of the test). Also in case of IMS PDN being established there is a wait timer (“waitForIMS”) to allow for optional SIP signalling. However in this case of paralel behaviour where UE is solely transfering the remaining PDU session from 5GS to EPS there shall be no SIP registration and the wait timer may interfere with execution of further steps (UE not getting the dedicated EPS bearer in steps 24A for too long may trigger cancelation of the procedure). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TTCN now accepts both APNs (pc_APN_ID_IMS and pc_APN_ID_Internet) in its implementation of the parallel behaviour of the table 11.1.2.3.2-2 - To skip “f_IP_Handling_StartPDN” and to avoid adding wait timer (“waitForIMS”) in case of IMS PDN being established in this parallel behaviour, there is a new parameter added to functions f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 f_EUTRA38_MultiPDN_EstablishAdditionalPDNs (see Change 2 and Change 3) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE configured with multi PDN with “internet” as default PDN may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

2.2	Change 2	5

2.3	Change 2	6

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_11_1_2_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | With the implementation of the R5-216188, specifically parallel behaviour in table 11.1.2.3.2-2, the implementation in TTCN is hardconding the expected APN to be of pc_APN_ID_Internet. However depending on the UE setting of the default APN, it may also be of pc_APN_ID_IMS. |
| Summary of change | TTCN now accepts both APNs (pc_APN_ID_IMS and pc_APN_ID_Internet) in its implementation of the parallel behaviour of the table 11.1.2.3.2-2 |
| TTCN module | NR5GC_IRAT\11_1\EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var integer v_EPSBearerId; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5AA" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); f_TAUReject_AttachWithoutN26 (eutra_Cell1); // @siclog "Parallel Behaviour" siclog@ f_EUTRA38_MultiPDN_EstablishAdditionalPDNs (eutra_Cell1, (pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection), false, tsc_SRB2, omit, '010'B, cr_AccessPointName(f_DomainName_Encode(pc_APN_ID_Internet))); // @sic R5-216188 sic@ // @siclog "Step 24A - 24C" siclog@ // @sic R5-223430 sic@ v_EPSBearerId := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @sic R5s220448 sic@ // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(v_EPSBearerId, 1)); // @sic R5s220448 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change:

function fl_TC_11_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var integer v_EPSBearerId; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5AA" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); f_TAUReject_AttachWithoutN26 (eutra_Cell1); // @siclog "Parallel Behaviour" siclog@ f_EUTRA38_MultiPDN_EstablishAdditionalPDNs (eutra_Cell1, (pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection), false, tsc_SRB2, omit, '010'B, (cr_AccessPointName(f_DomainName_Encode(pc_APN_ID_Internet)),cr_AccessPointName(f_DomainName_Encode(pc_APN_ID_IMS))), -,true); // @siclog "Step 24A - 24C" siclog@ // @sic R5-223430 sic@ v_EPSBearerId := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @sic R5s220448 sic@ // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(v_EPSBearerId, 1)); // @sic R5s220448 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

## Change 2

| Function name | f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 |
| --- | --- |
| Reason for change | New parameter needs to be added to f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 for the possibility of skipping f_IP_Handling_StartPDN in case special cases like the parallel behaviour in 11.1.2.3.2-2. |
| Summary of change | Adding new parameter which optionally skips execution of f_IP_Handling_StartPDN. |
| TTCN module | Common\EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(EUTRA_CellId_Type p_CellId, PDN_CONNECTIVITY_REQUEST p_PDNConnReq, boolean p_ConfigureSRB2 := false, template (omit) NasCount_Type p_NasCountUL := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit, boolean p_N1disabled := false) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { … if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@ f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, v_NewDRBId)); } …

After Change:

function f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(EUTRA_CellId_Type p_CellId, PDN_CONNECTIVITY_REQUEST p_PDNConnReq, boolean p_ConfigureSRB2 := false, template (omit) NasCount_Type p_NasCountUL := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit, boolean p_N1disabled := false, boolean p_ParallelBehaviour := false) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { … if (p_ParallelBehaviour) {v_StartPDN := false}; if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@ f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, v_NewDRBId)); } …

## Change 3

| Function name | f_EUTRA38_MultiPDN_EstablishAdditionalPDNs |
| --- | --- |
| Reason for change | New parameter needs to be added to f_EUTRA38_MultiPDN_EstablishAdditionalPDNs for the possibility to be passed down to f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 and to avoid adding wait timer for optional SIP signalling in case of IMS PDN. |
| Summary of change | Adding new parameter which is passed down to f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 and to avoid adding wait timer for optional SIP signalling in case of IMS PDN. |
| TTCN module | Common\EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(EUTRA_CellId_Type p_CellId, integer p_ExpectedNoOfNewPDNs, boolean p_FailWhenK_LessN := true, integer p_SRB := tsc_SRB2, // @sic R5s210341 sic@ template (omit) NasCount_Type p_NasCountUL := omit, B3_Type p_RequestType := '001'B, template AccessPointName p_AccessPointName := cr_AccessPointName_Any, boolean p_N1disabled := false ) runs on EUTRA_Multi5GS_PTC return integer { var SRB_COMMON_IND v_ReceivedAsp; var NAS_MSG_Indication_Type v_NasInd; var PDN_CONNECTIVITY_REQUEST v_PDNConnReq; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var EPS_BearerIdentity v_EpsBearerId; var integer v_L_NumOfReqsReceivedInParallel := 0; var integer v_K_NumOfPDNsProcessed := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var integer v_SRB := p_SRB; var B3_Type v_PDNType := f_GetPdnType(); var default v_MyDefaultVar := null; timer t_Wait := 10.0; // First clear store of previously received messages f_EUTRA38_MsgInDefault_ResetStoreOfPDNConnectivityReq(); // Then activate store for any new msgs received v_MyDefaultVar := activate(a_EUTRA38_AdditionalPDNConnectivityReq(p_CellId, cr_PDNConnReqAdditionalPDN(v_PDNType, p_AccessPointName, p_RequestType))); // @sic R5-216188 sic@ while ((v_K_NumOfPDNsProcessed < p_ExpectedNoOfNewPDNs) and not v_EndLoop) { v_RxdMsg := false; t_Wait.start; if (f_EUTRA38_MsgInDefault_GetNumOfPDNConnectivityReq() > v_L_NumOfReqsReceivedInParallel) { t_Wait.stop; v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel +1; v_ReceivedAsp := f_EUTRA38_MsgInDefault_GetOnePDNConnectivityReq(v_L_NumOfReqsReceivedInParallel); v_RxdMsg := true; } else { // no outstanding msgs, so must wait for next one alt { [] SRB.receive(car_SRB_NasPdu_IND(p_CellId, v_SRB, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDNConnReqAdditionalPDN(v_PDNType, p_AccessPointName, p_RequestType)))) // @sic R5-216188 sic@ -> value v_ReceivedAsp /* NOTE: acc. to the prose the ESM information transfer flag is still "Not present or any allowed value" * nevertheless if the UE sets the ESM information transfer flag to true this may cause issues * as the generic procedure does not handle any requested ESM information transfer */ { t_Wait.stop; v_RxdMsg := true; } [] t_Wait.timeout { // @sic R5-206291 sic@ if (p_FailWhenK_LessN and (v_K_NumOfPDNsProcessed < p_ExpectedNoOfNewPDNs)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Established " & int2str(v_K_NumOfPDNsProcessed) & "PDNs instead of " & int2str(p_ExpectedNoOfNewPDNs)); } else if (not p_FailWhenK_LessN) { // New message not received in time, so just end the loop v_EndLoop := true; } } } } if (v_RxdMsg) { // this will be false if either just triggered a new PDN, or about to end the loop // Update K now as latest msg is about to be processed v_K_NumOfPDNsProcessed := v_K_NumOfPDNsProcessed +1; v_NasInd := v_ReceivedAsp.Signalling.Nas[0]; v_PDNConnReq := v_NasInd.Pdu.Msg.pDN_CONNECTIVITY_REQUEST; v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(p_CellId, v_PDNConnReq, (v_SRB != tsc_SRB2), p_NasCountUL, -, p_N1disabled); // @sic R5-227443 sic@ a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(p_CellId); v_SRB := tsc_SRB2; // SRB2 will have been configured by now, even if not earlier a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(p_CellId, v_EpsBearerId); select (f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { v_WaitForUserplaneSignalling := dontWaitForIMS; } } } } // Now all PDNs on same connection have been established, wait for any signalling on user plane to finish before releasing connection f_EUTRA_DelayForUserPlaneSignalling (v_WaitForUserplaneSignalling); // all expected msgs have been processed, so deactivate the extra default deactivate(v_MyDefaultVar); return v_SRB; }

After Change:

function f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(EUTRA_CellId_Type p_CellId, integer p_ExpectedNoOfNewPDNs, boolean p_FailWhenK_LessN := true, integer p_SRB := tsc_SRB2, // @sic R5s210341 sic@ template (omit) NasCount_Type p_NasCountUL := omit, B3_Type p_RequestType := '001'B, template AccessPointName p_AccessPointName := cr_AccessPointName_Any, boolean p_N1disabled := false, boolean p_ParallelBehaviour := false ) runs on EUTRA_Multi5GS_PTC return integer { var SRB_COMMON_IND v_ReceivedAsp; var NAS_MSG_Indication_Type v_NasInd; var PDN_CONNECTIVITY_REQUEST v_PDNConnReq; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var EPS_BearerIdentity v_EpsBearerId; var integer v_L_NumOfReqsReceivedInParallel := 0; var integer v_K_NumOfPDNsProcessed := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var integer v_SRB := p_SRB; var B3_Type v_PDNType := f_GetPdnType(); var default v_MyDefaultVar := null; timer t_Wait := 10.0; // First clear store of previously received messages f_EUTRA38_MsgInDefault_ResetStoreOfPDNConnectivityReq(); // Then activate store for any new msgs received v_MyDefaultVar := activate(a_EUTRA38_AdditionalPDNConnectivityReq(p_CellId, cr_PDNConnReqAdditionalPDN(v_PDNType, p_AccessPointName, p_RequestType))); // @sic R5-216188 sic@ while ((v_K_NumOfPDNsProcessed < p_ExpectedNoOfNewPDNs) and not v_EndLoop) { v_RxdMsg := false; t_Wait.start; if (f_EUTRA38_MsgInDefault_GetNumOfPDNConnectivityReq() > v_L_NumOfReqsReceivedInParallel) { t_Wait.stop; v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel +1; v_ReceivedAsp := f_EUTRA38_MsgInDefault_GetOnePDNConnectivityReq(v_L_NumOfReqsReceivedInParallel); v_RxdMsg := true; } else { // no outstanding msgs, so must wait for next one alt { [] SRB.receive(car_SRB_NasPdu_IND(p_CellId, v_SRB, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDNConnReqAdditionalPDN(v_PDNType, p_AccessPointName, p_RequestType)))) // @sic R5-216188 sic@ -> value v_ReceivedAsp /* NOTE: acc. to the prose the ESM information transfer flag is still "Not present or any allowed value" * nevertheless if the UE sets the ESM information transfer flag to true this may cause issues * as the generic procedure does not handle any requested ESM information transfer */ { t_Wait.stop; v_RxdMsg := true; } [] t_Wait.timeout { // @sic R5-206291 sic@ if (p_FailWhenK_LessN and (v_K_NumOfPDNsProcessed < p_ExpectedNoOfNewPDNs)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Established " & int2str(v_K_NumOfPDNsProcessed) & "PDNs instead of " & int2str(p_ExpectedNoOfNewPDNs)); } else if (not p_FailWhenK_LessN) { // New message not received in time, so just end the loop v_EndLoop := true; } } } } if (v_RxdMsg) { // this will be false if either just triggered a new PDN, or about to end the loop // Update K now as latest msg is about to be processed v_K_NumOfPDNsProcessed := v_K_NumOfPDNsProcessed +1; v_NasInd := v_ReceivedAsp.Signalling.Nas[0]; v_PDNConnReq := v_NasInd.Pdu.Msg.pDN_CONNECTIVITY_REQUEST; v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(p_CellId, v_PDNConnReq, (v_SRB != tsc_SRB2), p_NasCountUL, -, p_N1disabled,p_ParallelBehaviour); // @sic R5-227443 sic@ a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(p_CellId); v_SRB := tsc_SRB2; // SRB2 will have been configured by now, even if not earlier a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(p_CellId, v_EpsBearerId); if (not(p_ParallelBehaviour)) { select (f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { v_WaitForUserplaneSignalling := dontWaitForIMS; } } } } } // Now all PDNs on same connection have been established, wait for any signalling on user plane to finish before releasing connection f_EUTRA_DelayForUserPlaneSignalling (v_WaitForUserplaneSignalling); // all expected msgs have been processed, so deactivate the extra default deactivate(v_MyDefaultVar); return v_SRB; }