3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230224

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2995 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.2.3.7 for Redcap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As per step 40, there are 2 NACK SN list to be set. So NACK sequence must have E1 set to 1 and next should be 0 - At step 40, There is next NACK SN to be set after first, so E1 should be set to 1 for the first one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added 3rd argument on crs_NR_RLC_NACK_SN12_Range () at step 40 as 1. - Added p_E1 with default value as '0'B, and from calling function passed 1 to set E1:=1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to f_TC_7_1_2_3_6_NR_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_2_3_7_NR_TestBody |
| --- | --- |
| Reason for change | As per step 40, there are 2 NACK list to be set. So NACK sequence must have E1 set to 1 and next should be 0 |
| Summary of change | Added 3rd argument on crs_NR_RLC_NACK_SN12_Range () at step 40 as 1. |
| TTCN module | RLC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_2_3_7_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingDL1; var SubFrameTiming_Type v_TimingDL2; var SubFrameTiming_Type v_TimingDL3; var SubFrameTiming_Type v_TimingDL4; ………… //@siclog "Step 38" siclog@ // After 300 ms the SS transmits an AMD PDU with SN=16 and P=1. v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); p_RLC_Rec.AM_TX_Next := 16; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_Poll, cs_TimingInfo_NR(v_TimingDL)); //@siclog "Step 39" siclog@ // 30 ms after step 38 the SS assigns an UL grant (UL grant allocation type 3) of size 112 bits. // TBS = Status Report with ACK_SN (3 Bytes) and 1 NACK_SNs with NACK Range(4 Bytes) + NACK SN (3 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Byte) = 3 + 4 + 3 + 2 + 2 = 14 octets = 112 bits // TBS = Status Report with ACK_SN (3 Bytes) and 1 NACK_SNs with NACK Range(3 Bytes) + NACK SN (2 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Bytes) + Padding (2 Bytes) = 3 + 3 + 2 + 2 + 2 + 2 = 14 octets = 112 bits (RedCap) //@sic R5s190791 change 5.1/8, R5s200250 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 30); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Step 40" siclog@ // Check: Does the UE transmit a Status Report with ACK_SN=20 and NACK_SN: 13 including NACK Range 3 (SN 13, 14, 15) and NACK_SN=18 without NACK Range? p_RLC_Rec.AM_RX_Next_Ack := 20; if (pc_supportOfRedCap_r17 == false) { // @sic R5-227164 sic@ v_NR_NackSN18Bit_1 := crs_NR_RLC_NACK_SN18_Range (13, 3, '1'B); //@sic R5s190791 change 5.1/7 sic@ //Anritsu TTCN CR R5s230064 v_NR_NackSN18Bit_2 := crs_NR_RLC_NACK_SN18_Simple (18); v_NR_NackSN18Bit_List := { v_NR_NackSN18Bit_1, v_NR_NackSN18Bit_2 }; v_TimingUL := f_NR_RxSTATUS_PDU_GetTimingInfo(p_RLC_Rec, p_NR_DRB_Id, v_NR_NackSN18Bit_List); //@sic R5s200250 sic@ } else { v_NR_NackSN12Bit_1 := crs_NR_RLC_NACK_SN12_Range (13, 3); v_NR_NackSN12Bit_2 := crs_NR_RLC_NACK_SN12_Simple (18); v_NR_NackSN12Bit_List := { v_NR_NackSN12Bit_1, v_NR_NackSN12Bit_2 }; v_TimingUL := f_NR_RxSTATUS_PDU_GetTimingInfo(p_RLC_Rec, p_NR_DRB_Id, -, v_NR_NackSN12Bit_List); } //@siclog "Step 41" siclog@ // 60 ms after step 38 the SS transmits 4 AMD PDUs with SN=13, 14, 15 and 18. A time spacing of 20 ms is applied. v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 60); //@sic R5s200250, R5s200540 sic@ v_TimingDL1 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20); v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 40); v_TimingDL3 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 60); p_RLC_Rec.AM_TX_Next := 13; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL)); f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL1)); f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL2)); p_RLC_Rec.AM_TX_Next := 18; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL3)); //@siclog "Step 42" siclog@ ………………….. }

After Change:

function f_TC_7_1_2_3_7_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingDL1; var SubFrameTiming_Type v_TimingDL2; var SubFrameTiming_Type v_TimingDL3; var SubFrameTiming_Type v_TimingDL4; ………… //@siclog "Step 38" siclog@ // After 300 ms the SS transmits an AMD PDU with SN=16 and P=1. v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); p_RLC_Rec.AM_TX_Next := 16; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_Poll, cs_TimingInfo_NR(v_TimingDL)); //@siclog "Step 39" siclog@ // 30 ms after step 38 the SS assigns an UL grant (UL grant allocation type 3) of size 112 bits. // TBS = Status Report with ACK_SN (3 Bytes) and 1 NACK_SNs with NACK Range(4 Bytes) + NACK SN (3 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Byte) = 3 + 4 + 3 + 2 + 2 = 14 octets = 112 bits // TBS = Status Report with ACK_SN (3 Bytes) and 1 NACK_SNs with NACK Range(3 Bytes) + NACK SN (2 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Bytes) + Padding (2 Bytes) = 3 + 3 + 2 + 2 + 2 + 2 = 14 octets = 112 bits (RedCap) //@sic R5s190791 change 5.1/8, R5s200250 sic@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 30); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Step 40" siclog@ // Check: Does the UE transmit a Status Report with ACK_SN=20 and NACK_SN: 13 including NACK Range 3 (SN 13, 14, 15) and NACK_SN=18 without NACK Range? p_RLC_Rec.AM_RX_Next_Ack := 20; if (pc_supportOfRedCap_r17 == false) { // @sic R5-227164 sic@ v_NR_NackSN18Bit_1 := crs_NR_RLC_NACK_SN18_Range (13, 3, '1'B); //@sic R5s190791 change 5.1/7 sic@ //Anritsu TTCN CR R5s230064 v_NR_NackSN18Bit_2 := crs_NR_RLC_NACK_SN18_Simple (18); v_NR_NackSN18Bit_List := { v_NR_NackSN18Bit_1, v_NR_NackSN18Bit_2 }; v_TimingUL := f_NR_RxSTATUS_PDU_GetTimingInfo(p_RLC_Rec, p_NR_DRB_Id, v_NR_NackSN18Bit_List); //@sic R5s200250 sic@ } else { v_NR_NackSN12Bit_1 := crs_NR_RLC_NACK_SN12_Range (13, 3, '1'B); v_NR_NackSN12Bit_2 := crs_NR_RLC_NACK_SN12_Simple (18); v_NR_NackSN12Bit_List := { v_NR_NackSN12Bit_1, v_NR_NackSN12Bit_2 }; v_TimingUL := f_NR_RxSTATUS_PDU_GetTimingInfo(p_RLC_Rec, p_NR_DRB_Id, -, v_NR_NackSN12Bit_List); } //@siclog "Step 41" siclog@ // 60 ms after step 38 the SS transmits 4 AMD PDUs with SN=13, 14, 15 and 18. A time spacing of 20 ms is applied. v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 60); //@sic R5s200250, R5s200540 sic@ v_TimingDL1 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20); v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 40); v_TimingDL3 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 60); p_RLC_Rec.AM_TX_Next := 13; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL)); f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL1)); f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL2)); p_RLC_Rec.AM_TX_Next := 18; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL3)); //@siclog "Step 42" siclog@ …………….. }

## Change 2

| Function name | template crs_NR_RLC_NACK_SN12_Range |
| --- | --- |
| Reason for change | At step 40, There is next NACK SN to be set after first, so E1 should be set to 1 |
| Summary of change | Added p_E1 with default value as '0'B, and from calling function passed 1 to set E1:=1 |
| TTCN module | NR_RLC_Common.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) NR_RLC_Status_NackSN12Bit_Type crs_NR_RLC_NACK_SN12_Range(integer p_NackSN, integer p_NACKrange) := { /* serves to transmit or receive */ /* @status APPROVED (ENDC, NR5GC) */ SequenceNumberNACK := int2bit(p_NackSN, 12), /* 12 bits SN */ E1 := '1'B, /* 1 bit E1 field */ E2 := '0'B, /* 1 bit E2 field */ E3 := '1'B, /* 1 bit E3 field */ Reserved := '0'B, /* 1 bits reserved */ SOstart := omit, SOstop := omit, NACKrange := int2bit(p_NACKrange, 8) };

After Change:

template (value) NR_RLC_Status_NackSN12Bit_Type crs_NR_RLC_NACK_SN12_Range(integer p_NackSN, integer p_NACKrange, B1_Type p_E1 := '0'B) := { /* serves to transmit or receive */ /* @status APPROVED (ENDC, NR5GC) */ SequenceNumberNACK := int2bit(p_NackSN, 12), /* 12 bits SN */ E1 := p_E1, /* 1 bit E1 field */ E2 := '0'B, /* 1 bit E2 field */ E3 := '1'B, /* 1 bit E3 field */ Reserved := '0'B, /* 1 bits reserved */ SOstart := omit, SOstop := omit, NACKrange := int2bit(p_NACKrange, 8) };