3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230257

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0899 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TTCN type definitions, Annex H |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-03-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Annex H for the TTCN type definitions should be updated to document the changes in the TTCN delivery iwd-23wk12 (Friday 24th March 2023). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Annex H. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASP changes will not be correctly documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex H (informative):
TTCN-3 Definitions of Common Interfaces

The multi-testers model according to clause 5 provides interfaces which can be re-used by implementations of the IP-CAN test model according to TS 34.229-4 [50].

NOTE: Common type definitions are according to annex D of TS 36.523-3 [30].

# H.1	IMS_PTC_CoordMsg

IMS_TestProcedure_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IMS_TestProcedure_Type |
| Comment |  |
| IPCAN_InitialRegistration | EUTRA/EPS signalling acc. to 36.508 cl. 4.5.2.3 without RRC Connection Release at the end of the procedure NOTE: As working assumption the UE does IMS REGISTRATION automatically after RRC/NAS registration |
| IPCAN_EmergencyCall_NormalService | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.4.3 |
| IPCAN_EmergencyCall_OptionalPDNRelease | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.4.3, but with Call Release according to 34.229-1 C.32a |
| IPCAN_EmergencyCall_LimitedService | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.5.3 |
| IPCAN_MO_SpeechCall | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.6.3 |
| IPCAN_MT_SpeechCall | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.7.3 |
| IPCAN_MO_VideoCall | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.8.3 |
| IPCAN_MT_VideoCall | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.9.3 |
| IPCAN_MO_AddVideo | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.11.3 |
| IPCAN_MT_AddVideo | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.12.3 |
| IPCAN_ReleaseVideo | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.15.3 |
| IPCAN_MO_IMS_Signalling | EUTRA/EPS signalling acc. to 36.508 cl. 4.5.3.3 with m = n = 0; used e.g. for MT SMS test case 18.2 |
| IPCAN_MT_IMS_Signalling | EUTRA/EPS signalling acc. to 36.508 cl. 4.5.3.3 Steps 3 to 9 with m = n = 0; used e.g. for MT SMS test case 18.1 |
| IPCAN_XCAP_Signalling | EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.14 |
| IPCAN_MO_DataCall | for MCData test cases |
| IPCAN_MT_DataCall | for MCData test cases |
| IPCAN_AdditionalIPAddress | CR 213400 |
| IPCAN_Deregistration |  |
| IPCAN_UseVPLMN | Change cell used from Cell 1 (HPLMN) to Cell 12 (VPLMN) |

IMS_TestConfiguration_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IMS_TestConfiguration_Type |
| Comment |  |
| IPCAN_SignallingOnly | EUTRA: default DRB is used only |
| IPCAN_SpeechCall | EUTRA: one dedicated UM bearer; for normal speech calls and emergency call for limited services |
| IPCAN_VideoCall | EUTRA: two dedicated UM bearers |
| IPCAN_EmergencyCall | EUTRA: second default bearer (AM) and one dedicated UM bearer |
| IPCAN_SpeechAndEmergencyCall | EUTRA: default bearer + dedicated bearer for normal speech call and another default bearer + dedicated bearer for emergency call |
| IPCAN_XCAP | EUTRA: second default bearer (AM) for second PDN used for XCAP signalling |
| IPCAN_DataCall | MCData: one dedicated UM bearer |

IPCAN_INFO_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPCAN_INFO_Type |  |  |
| Comment |  |  |  |
| RanType | IPCAN_RAN_Type | opt |  |
| UE_Release | integer | opt |  |
| AuthResLength | integer | opt |  |
| NetworkSupportsVoiceOverIMS | boolean |  |  |

IMS_IPCAN_CommandName_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IMS_IPCAN_CommandName_Type |
| Comment |  |
| IPCAN_INIT | trigger the IPCAN_PTC to create a cell and do further appropriate initialisation; which RAN technology to be use is decided by the IPCAN_PTC based on PIXITs; as test procedure shall be specified which procedure is used during the test body to know which DRBs need to be pre-configured; IPCAN returns response indicating the RAN type |
| IPCAN_STARTPROCEDURE | trigger the IPCAN to expect (MO) or page (MT) the UE to establish an RRC connection; depending on the connection type triggers may need to be sent from IPCAN to IMS or from IMS to IPCAN to synchronise establishment of dedicated DRBs (EUTRA) or secondary PDP contexts (UTRAN) |
| IPCAN_ENDPROCEDURE | trigger RRC connection release by the IPCAN_PTC; as a SIP message may be sent out just before end of the procedure in general a delay of 2s shall be added before sending IPCAN_ENDPROCEDURE to IPCAN; for UTRAN it is up to IPCAN and SS implementation to cope with possible/necessary release of (secondary) PDP context; a trigger shall be sent from IPCAN to IMS to indicate when RRC connection is released |
| IPCAN_RELEASE | Detach UE and release cell (postamble); a trigger is shall be sent from IPCAN to IMS to indicate when IPCAN is released |
| IPCAN_QUERY | query information from the IPCAN PTC |
| IPCAN_PROVIDELOCATIONINFORMATION | trigger IPCAN to provide UELocationInformation to the UE acc. to 36.509 |

IMS_IPCAN_Command_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_IPCAN_Command_Type |  |  |
| Comment | Messages IMS_PTC -> IPCAN |  |  |
| Name | IMS_IPCAN_CommandName_Type |  |  |
| TestConfiguration | IMS_TestConfiguration_Type | opt |  |
| TestProcedure | IMS_TestProcedure_Type | opt |  |

IMS_IPCAN_ResponseName_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IMS_IPCAN_ResponseName_Type |
| Comment |  |
| IPCAN_INIT | response for INIT command: carries the RAN type as used by the IPCAN PTC; the RAN type depends on PIXIT settings: part 4 model: px_RANTech part 3 model: EUTRA_FDD or EUTRA_TDD depending on px_ePrimaryFrequencyBand (px_ePrimaryFrequencyBand < 33 => FDD) |
| IPCAN_QUERY |  |

IPCAN_IMS_Response_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPCAN_IMS_Response_Type |  |  |
| Comment |  |  |  |
| Name | IMS_IPCAN_ResponseName_Type |  |  |
| IpcanInfo | IPCAN_INFO_Type | opt |  |

IPCAN_IMS_Reset_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IPCAN_IMS_Reset_Type |
| Comment |  |
| WithCNF |  |
| WithoutCNF |  |

IPCAN_IMS_Control_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IPCAN_IMS_Control_Type |  |
| Comment |  |  |
| Reset | IPCAN_IMS_Reset_Type | to reset IMS after UE has been switched off and gets switched on again |
| IpcanInd | Null_Type | to indicate to IMS which IPCAN is active (at port IPCAN or at port OtherIPCAN) |
| ReregistrationStart | IPCAN_RAN_Type | to prepare IMS PTC for (optional) re-registration |
| ReregistrationStop | Null_Type | to indicate to IMS PTC that no re-registration will happen anymore |
| NoRegistration | Null_Type | no IMS registration shall happen (e.g. due to test loop mode being activated) |

IMS_IPCAN_Coordination_MSG

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IMS_IPCAN_Coordination_MSG |  |
| Comment |  |  |
| TriggerEvent | CoOrdination_Trigger_Type | any trigger of confirmation |
| AbortEvent | Null_Type | sent instead of TriggerEvent if procedure shall be aborted |
| IPCAN_IMS_Control | IPCAN_IMS_Control_Type | IMS <- IPCAN: to control IMS (default behaviour) |
| IMS_IPCAN_Command | IMS_IPCAN_Command_Type | IMS -> IPCAN: command from the IPCAN to control IMS (mainly for IMS default behaviour) |
| IPCAN_IMS_Response | IPCAN_IMS_Response_Type | IMS <- IPCAN: response for previous command |
| ProtocolConfigOptions | NAS_ProtocolConfigOptions_Type | IMS <-> IPCAN: PCOs to be used in NAS signalling |
| IPCAN_IMS_Data | charstring | IMS <- IPCAN: to send data from IPCAN to IMS |
| DrbInfo | IP_DrbInfo_Type | IMS <- IPCAN: to send DrbInfo for MediaControlInfo (MCX) and HTTP to MCX_IMS_PTC |

IMS_IPCAN_CO_ORD_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IMS_IPCAN_CO_ORD_PORT |  |
| Comment |  |  |
| out | IMS_IPCAN_Coordination_MSG |  |
| in | IMS_IPCAN_Coordination_MSG |  |

CoOrdination_Trigger_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CoOrdination_Trigger_Type |  |  |
| Comment |  |  |  |
| Reason | charstring |  |  |

# H.2	IMS_ASP_TypeDefs

IMS_ASP_TypeDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| IMS_Request_Type | RequestUnion | Alias for "RequestUnion" as defined in LibSip_SIPTypesAndValues |
| IMS_Response_Type | Response | Alias for "Response" as defined in LibSip_SIPTypesAndValues |
| IMS_PortsAndSecurityConfigCnf_Type | Null_Type | SPIs and protected ports are fully controlled by the IMS PTC => it is not necessary anymore to return IMS_ProtectedPorts_Type, IMS_SPIs_Type to the IMS PTC |

IMS_SecurityContextEnum_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IMS_SecurityContextEnum_Type |
| Comment |  |
| unprotected |  |
| protectedContext1 |  |
| protectedContext2 |  |

IMS_RoutingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_RoutingInfo_Type |  |  |
| Comment |  |  |  |
| Protocol | InternetProtocol_Type |  | UDP or TCP |
| Security | IMS_SecurityContextEnum_Type | opt | protected or unprotected (in DL omit when IP PTC shall decide what to do) |
| UE_Address | IP_AddrInfo_Type | opt | sent by the IP PTC when there is an initial request on unprotected connection |
| NW_Address | IP_AddrInfo_Type | opt | sent by the IP PTC when there is an initial request on unprotected connection |

IMS_DATA_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_DATA_REQ |  |  |
| Comment |  |  |  |
| RoutingInfo | IMS_RoutingInfo_Type |  |  |
| Request | IMS_Request_Type |  |  |

IMS_DATA_RSP

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_DATA_RSP |  |  |
| Comment |  |  |  |
| RoutingInfo | IMS_RoutingInfo_Type |  |  |
| Response | IMS_Response_Type |  |  |

IMS_ProtectedPorts_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_ProtectedPorts_Type |  |  |
| Comment |  |  |  |
| Port_us | PortNumber_Type |  | UE side: Server |
| Port_uc | PortNumber_Type |  | UE side: Client |
| Port_ps | PortNumber_Type |  | network side: Server |
| Port_pc | PortNumber_Type |  | network side: Client |

IMS_SPIs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_SPIs_Type |  |  |
| Comment |  |  |  |
| SPI_us | IPsec_SPI_Type |  | SPI at UE side: assigned by the UE |
| SPI_uc | IPsec_SPI_Type |  | SPI at UE side: assigned by the UE |
| SPI_ps | IPsec_SPI_Type | opt | SPI at network side: to be assigned by TTCN |
| SPI_pc | IPsec_SPI_Type | opt | SPI at network side: to be assigned by TTCN |

IMS_SecurityInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_SecurityInfo_Type |  |  |
| Comment |  |  |  |
| ProtectedPorts | IMS_ProtectedPorts_Type |  |  |
| SPIs | IMS_SPIs_Type |  |  |
| IntegrityAlgorithm | IPsec_IntegrityAlgorithm_Type |  |  |
| CipheringAlgorithm | IPsec_CipheringAlgorithm_Type |  |  |

IMS_RegistrationInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_RegistrationInfo_Type |  |  |
| Comment |  |  |  |
| NW_Address | IP_AddrInfo_Type |  | network address of the chosen IMS server (e.g. IPv4, IPv6) |
| UE_Address | IP_AddrInfo_Type |  | UE address as used for security protected connections |
| SecurityInfo | IMS_SecurityInfo_Type | opt | omit in case of GIBA |

IMS_PortsAndSecurityConfigReq_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_PortsAndSecurityConfigReq_Type |  |  |
| Comment |  |  |  |
| UnprotectedPort_us | PortNumber_Type | opt | 5060 per default |
| RegistrationInfo | IMS_RegistrationInfo_Type | opt |  |

IMS_SecurityReleaseMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IMS_SecurityReleaseMode_Type |
| Comment |  |
| full | all protected and unprotected security contexts shall be released (e.g. at de-registration) |
| keepUnprotected | all protected security contexts shall be released but the unprotected security context shall be kept |
| oldestProtectedContextOnly | only the oldest protected security context shall be released |

IMS_SecurityRelease_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_SecurityRelease_Type |  |  |
| Comment |  |  |  |
| Mode | IMS_SecurityReleaseMode_Type |  |  |
| WaitForCNF | boolean |  |  |

IMS_RestartPCSCF_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IMS_RestartPCSCF_Type |  |  |
| Comment |  |  |  |
| PdnIndex | PDN_Index_Type |  |  |
| IPAddressIPv4 | charstring |  |  |
| IPAddressIPv6 | charstring |  |  |

IMS_CONFIG_REQ

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IMS_CONFIG_REQ |  |
| Comment |  |  |
| InstallKey | IPsec_SecurityKeys_Type |  |
| PortsAndSecurityConfig | IMS_PortsAndSecurityConfigReq_Type |  |
| SecurityRelease | IMS_SecurityRelease_Type |  |
| CloseTCP | Null_Type |  |
| RestartPCSCF | IMS_RestartPCSCF_Type |  |

IMS_CONFIG_CNF

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IMS_CONFIG_CNF |  |
| Comment |  |  |
| InstallKey | Null_Type |  |
| PortsAndSecurityConfig | IMS_PortsAndSecurityConfigCnf_Type |  |
| SecurityRelease | Null_Type |  |
| CloseTCP | Null_Type |  |
| RestartPCSCF | Null_Type |  |

IMS_IP_CTRL_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IMS_IP_CTRL_PORT |  |
| Comment | Control port at the IMS PTC to configure IP for IMS |  |
| out | IMS_CONFIG_REQ |  |
| in | IMS_CONFIG_CNF |  |

IMS_IP_CLIENT_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IMS_IP_CLIENT_PORT |  |
| Comment | IMS client: send requests, receive response |  |
| out | IMS_DATA_REQ |  |
| in | IMS_DATA_RSP |  |

IMS_IP_SERVER_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IMS_IP_SERVER_PORT |  |
| Comment | IMS server: send response, receive requests |  |
| out | IMS_DATA_RSP |  |
| in | IMS_DATA_REQ |  |

# H.3	HTTP_ASP_TypeDefs

## H.3.1	HTTP_ASP_Definitions

HttpServerInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HttpServerInfo_Type |  |  |
| Comment |  |  |  |
| serverAddr | IP_AddrInfo_Type |  | IP address of simulated server |
| serverPort | PortNumber_Type |  | Port number of simulated server |
| drbInfo | IP_DrbInfo_Type |  | DRB info as used by the IP PTC (LTE model, see TS 36.523-3 [30]) |
| tlsConfig | TLSConfig_Type | opt | TLS parameters as used by the IP PTC (LTE model, see TS 36.523-3 [30]) (omit: no TLS) |

HttpServerList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | HttpServerList_Type |
| Comment |  |
| record length (1 .. infinity) of HttpServerInfo_Type |  |

HTTP_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HTTP_CTRL_REQ |  |  |
| Comment | ASP type to configure the http layer; when any of the optional fields is omitted the previous configuration of this field is kept |  |  |
| httpServerList | HttpServerList_Type | opt | IP address and port of simulated HTTP servers |

HTTP_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HTTP_CTRL_CNF |  |  |
| Comment | ASP type to confirm HTTP_CTRL_REQ |  |  |
| errorInfo | charstring | opt | string indicating a system error |

HTTP_DATA_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HTTP_DATA_IND |  |  |
| Comment | ASP type for sending a message from the http layer to TTCN; it transports relevant information of a http Request from the UE to the Tester |  |  |
| routingInfo | IP_Connection_Type |  | TCP connection from which the request has been received; => response shall use the same |
| httpRequest | HttpRequest_Type |  |  |

HttpMessageBody

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HttpMessageBody |  |
| Comment |  |  |
| text | charstring |  |
| binary | octetstring |  |
| mime | MIME_Message |  |

HttpRequest_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HttpRequest_Type |  |  |
| Comment |  |  |  |
| requestLine | HttpRequestLine_Type |  | RFC 2616 clause 5.1 |
| userAgent | UserAgent | opt | User Agent according to RFC 2616 [27] clause 14.43 |
| authorization | Authorization | opt | Authorization according to RFC 2616 [27] clause 14.8 (optional; NOTE: Same type definition as for SIP type definitions) |
| host | Host | opt | host according to RFC 2616 [27] clause 14.23 |
| contentType | ContentType | opt | Content-Type according to RFC 2616 [27] clause 14.17 (optional, NOTE: Same type definition as for SIP type definitions) |
| cacheControl | HttpCacheControl_Type | opt | RFC 2616 clause 14.9 |
| x3GPPIntendedIdentity | charstring | opt | 3GPP TS 24.109 [33] clause G.2 |
| messageBody | HttpMessageBody | opt | e.g. XCAP Message; |

HttpResponse_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HttpResponse_Type |  |  |
| Comment |  |  |  |
| statusLine | HttpStatusLine_Type |  | Status-Line in RFC 2616 [27] clause 6.1 |
| date | Date | opt | Date according to RFC 2616 [27] clause 14.18 |
| eTag | SIP_ETag | opt | ETag according to RFC 2616 [27] clause 14.19 |
| server | Server | opt | Server according to RFC 2616 [27] clause 14.38 |
| wwwauthenticate | WwwAuthenticate | opt | WWW-Authenticate in RFC 2616 [27] clause 14.47 (NOTE: Same type definition as for SIP type definitions) |
| authenticationInfo | AuthenticationInfo | opt | Authentication-Info in RFC 2617 [37] clause 3.2.3 (NOTE: Same type definition as for SIP type definitions) |
| contentType | ContentType | opt | Content-Type in RFC 2616 [27] clause 14.17 (NOTE: Same type definition as for SIP type definitions) |
| location | HttpLocation_Type | opt | RFC 2616 clause 14.30 |
| contentLength | ContentLength | opt | Content-Length in RFC 2616 [27] clause 14.13 (NOTE: Same type definition as for SIP type definitions) |
| expires | Expires | opt | Expires in RFC 2616 [27] clause 14.21 (NOTE: Same type definition as for SIP type definitions) |
| cacheControl | HttpCacheControl_Type | opt | Cache-Control in RFC 2616 [27] clause 14.9 |
| pragma | HttpPragma_Type | opt | Pragma in RFC 2616 [27] clause 14.32 |
| messageBody | HttpMessageBody | opt | e.g. XCAP Message; |

HTTP_DATA_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HTTP_DATA_REQ |  |  |
| Comment | ASP type for sending messages from TTCN to the http layer; it transports information needed by the http layer to generate a http Response to the UE |  |  |
| routingInfo | IP_Connection_Type |  | TCP connection to be used in DL: shall be the same as for corresponding request |
| httpResponse | HttpResponse_Type |  |  |

HttpRequestLine_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HttpRequestLine_Type |  |  |
| Comment | request line according to RFC 2616 [27] clause 5.1 |  |  |
| method | charstring |  |  |
| uri | charstring |  | XCAP: selection expression, RFC 4825 [26] |
| version | charstring |  |  |

HttpStatusLine_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HttpStatusLine_Type |  |  |
| Comment | status line according to RFC 2616 [27] clause 5.1 |  |  |
| version | charstring |  |  |
| code | integer |  |  |
| reasonPhrase | charstring |  |  |

HttpCacheControl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HttpCacheControl_Type |  |  |
| Comment | RFC 2616 clause 14.9 |  |  |
| cacheDirective | charstring |  |  |

HttpPragma_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HttpPragma_Type |  |  |
| Comment | RFC 2616 clause 14.32 |  |  |
| pragmaDirective | charstring |  |  |

HttpLocation_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HttpLocation_Type |  |  |
| Comment | RFC 2616 clause 14.30, see also RFC 3986 section 4.3 |  |  |
| uri | charstring |  |  |

## H.3.2	HTTP_Port_Definitions

HTTP_CTRL_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | HTTP_CTRL_PORT |  |
| Comment |  |  |
| out | HTTP_CTRL_REQ |  |
| in | HTTP_CTRL_CNF |  |

HTTP_DATA_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | HTTP_DATA_PORT |  |
| Comment |  |  |
| in | HTTP_DATA_IND |  |
| out | HTTP_DATA_REQ |  |

# H.4	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| IMS_PTC_CoordMsg | IMS/IMS_PTC_CoordMsg.ttcn | Rev 35214 |
| IMS_ASP_TypeDefs | IMS/IMS_ASP_TypeDefs.ttcn | Rev 35130 |
| HTTP_ASP_TypeDefs | HTTP/HTTP_ASP_TypeDefs.ttcn | Rev 33197 |