3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230279

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3018 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The variable v_BearerContextNumber is initialized in function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 and can cause an unhandled exception when UE is set with incorrect PDN_Type (other then Internet_DNN or IMS_DNN). Unhandled exception can occur during calling the function f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the initial value to omit for v_BearerContextNumber to avoid TC end with unhandled exceptions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A non conformant UE may cause the test case finish with unhandle exception |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.5.NR5GC or other IRAT testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 |
| --- | --- |
| Reason for change | The variable v_BearerContextNumber is initialized in function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 and can cause an unhandled exception when UE is set with incorrect PDN_Type (other then Internet_DNN or IMS_DNN). Unhandled exception can occurred during calling the function f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams |
| Summary of change | Set the initial value to omit for v_BearerContextNumber to avoid TC end with unhandled exceptions |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(EUTRA_CellId_Type p_CellId, EUTRA_ATTACH_Type p_ForcedAttach, template (omit) AdditionalUpdateType p_AdditionalUpdateType, ProcedureTransactionIdentifier p_EPS_TI, boolean p_IPv4AllocationViaNasFlag, template (omit) ProtocolConfigOptions p_PcoFromUE, template (omit) AccessPointName p_APN, template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact, template (omit) GPRS_Timer p_T3402 := omit, template (omit) GPRS_Timer p_T3423 := omit, template (omit) PLMN_List p_EquivalentPlmnList := omit, integer p_NoOfEmergencyNumbers := 0, template (omit) EMM_Cause p_EMM_Cause := omit, template (omit) DCN_ID p_DCN_Id := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPcoFromUE := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit, template (omit) UERadioCapId p_UERadioCapId:= omit, boolean p_N1disabled := false) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed(); var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(p_ForcedAttach); var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity); var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(p_CellId); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, p_ForcedAttach); var TrackingAreaCode v_TAC := f_EUTRA_CellInfo_GetTAC(p_CellId); var template (value) AccessPointName v_APN_Tx := f_EUTRA38_MultiPDN_GetAPN(p_CellId, p_APN); var template (omit) EmergNumList v_EmergNumList := omit; var template (omit) MobileIdentity v_MSId := f_GetMSId(p_ForcedAttach); var PDU_PDN_DNN_Type v_PDN_Type := f_EUTRA38_MultiPDN_GetPDNType(p_APN); // @sic R5-206296 sic@ var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (v_PDN_Type); // @sic R5-206296 sic@ var boolean v_IsIMS := false; var EPS_BearerIdentity v_EpsDefaultBearerId; var template (omit) ESM_Cause v_ESM_Cause := omit; var template (value) PDN_Address v_PDN_Address; var EmergencyNumList v_EmergencyNumList; var boolean v_StartPDN := true; // @sic R5s210149 sic@ var template (value) ProtocolConfigOptions v_PcoToUE; var BEARER_CONTEXT_TYPE v_BearerContextNumber; var integer i; if (pc_IPv6 and pc_IPv4) { select (v_PDN_TypeToBeUsed) { case (forceIPv4only) { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); } // PDN type IPv4 only allowed case (forceIPv6only) { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); } // PDN type IPv6 only allowed case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; } } } if (p_NoOfEmergencyNumbers > 0) { v_EmergencyNumList := f_Get_EmergencyNumList (p_NoOfEmergencyNumbers, f_EUTRA_MobileInfo_GetLocalEmergencyNumberList()); f_EUTRA_MobileInfo_SetLocalEmergencyNumberList (v_EmergencyNumList); // save the list of locally usable emergency numbers v_EmergNumList := f_Build_EmergNumList(p_NoOfEmergencyNumbers, v_EmergencyNumList); } select(v_PDN_Type) { // @sic R5-206296 sic@ other options not relevant case (Internet_DNN) { v_BearerContextNumber := DEF_1; } case (IMS_DNN) { v_IsIMS := true; v_BearerContextNumber := DEF_2; } } v_EpsDefaultBearerId := f_EUTRA38_MultiPDN_ConfigureDRB(p_CellId, v_PDN_Type, v_IsIMS); v_PcoToUE := f_NAS_ProtocolConfigOptionsTX(f_EUTRA38_MultiPDN_GetPCOs(p_CellId, v_PDN_Type, // @sic R5s220883 sic@ v_IsIMS, v_EpsDefaultBearerId, p_ExtdPcoFromUE, p_PcoFromUE, p_N1disabled)); // @sic R5-227443 sic@ if (isvalue (p_ExtdPcoFromUE)) { // FFS - At the moment, 36.508 table 4.7.3-6 specifies ExtdPCO should be not present } if (isvalue(p_PduInfoFromNR)) { // @sic R5s210149 sic@ // The UE has just moved from NR. If so, data from NR should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromNR); i := i + 1) { if (v_PdnIndex == valueof(p_PduInfoFromNR[i].PdnIndex)) { // Data from NR should be used, rather than created anew v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function break; } } } if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@ f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, (hex2int(v_EpsDefaultBearerId)-4))); } v_PDN_Address := f_EUTRA_GetPDNAddress(p_IPv4AllocationViaNasFlag, v_PdnIndex); f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId, v_EpsDefaultBearerId, tsc_RRC_TI_Def, p_EPS_TI, v_AttachType, v_PDN_Address, v_Guti, cs_TAIListNonConsecutive_lv(v_PLMN, {bit2oct(v_TAC)}), v_LAI, v_MSId, v_ESM_Cause, v_APN_Tx, // @sic R5-206296 sic@ v_PcoToUE, f_GetAdditionalUpdateResult(p_AdditionalUpdateType, p_ForcedAttach), p_T3412, p_T3402, p_T3423, p_EquivalentPlmnList, v_EmergNumList, -, -, v_BearerContextNumber, p_EMM_Cause, omit, p_DCN_Id, omit, -, p_UERadioCapId); // @sic R5-216280 sic@ return v_EpsDefaultBearerId; }/

After Change:

function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(EUTRA_CellId_Type p_CellId, EUTRA_ATTACH_Type p_ForcedAttach, template (omit) AdditionalUpdateType p_AdditionalUpdateType, ProcedureTransactionIdentifier p_EPS_TI, boolean p_IPv4AllocationViaNasFlag, template (omit) ProtocolConfigOptions p_PcoFromUE, template (omit) AccessPointName p_APN, template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact, template (omit) GPRS_Timer p_T3402 := omit, template (omit) GPRS_Timer p_T3423 := omit, template (omit) PLMN_List p_EquivalentPlmnList := omit, integer p_NoOfEmergencyNumbers := 0, template (omit) EMM_Cause p_EMM_Cause := omit, template (omit) DCN_ID p_DCN_Id := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPcoFromUE := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit, template (omit) UERadioCapId p_UERadioCapId:= omit, boolean p_N1disabled := false) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed(); var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(p_ForcedAttach); var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity); var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(p_CellId); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, p_ForcedAttach); var TrackingAreaCode v_TAC := f_EUTRA_CellInfo_GetTAC(p_CellId); var template (value) AccessPointName v_APN_Tx := f_EUTRA38_MultiPDN_GetAPN(p_CellId, p_APN); var template (omit) EmergNumList v_EmergNumList := omit; var template (omit) MobileIdentity v_MSId := f_GetMSId(p_ForcedAttach); var PDU_PDN_DNN_Type v_PDN_Type := f_EUTRA38_MultiPDN_GetPDNType(p_APN); // @sic R5-206296 sic@ var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (v_PDN_Type); // @sic R5-206296 sic@ var boolean v_IsIMS := false; var EPS_BearerIdentity v_EpsDefaultBearerId; var template (omit) ESM_Cause v_ESM_Cause := omit; var template (value) PDN_Address v_PDN_Address; var EmergencyNumList v_EmergencyNumList; var boolean v_StartPDN := true; // @sic R5s210149 sic@ var template (value) ProtocolConfigOptions v_PcoToUE; var template (omit) BEARER_CONTEXT_TYPE v_BearerContextNumber := omit; var integer i; if (pc_IPv6 and pc_IPv4) { select (v_PDN_TypeToBeUsed) { case (forceIPv4only) { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); } // PDN type IPv4 only allowed case (forceIPv6only) { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); } // PDN type IPv6 only allowed case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; } } } if (p_NoOfEmergencyNumbers > 0) { v_EmergencyNumList := f_Get_EmergencyNumList (p_NoOfEmergencyNumbers, f_EUTRA_MobileInfo_GetLocalEmergencyNumberList()); f_EUTRA_MobileInfo_SetLocalEmergencyNumberList (v_EmergencyNumList); // save the list of locally usable emergency numbers v_EmergNumList := f_Build_EmergNumList(p_NoOfEmergencyNumbers, v_EmergencyNumList); } select(v_PDN_Type) { // @sic R5-206296 sic@ other options not relevant case (Internet_DNN) { v_BearerContextNumber := DEF_1; } case (IMS_DNN) { v_IsIMS := true; v_BearerContextNumber := DEF_2; } } v_EpsDefaultBearerId := f_EUTRA38_MultiPDN_ConfigureDRB(p_CellId, v_PDN_Type, v_IsIMS); v_PcoToUE := f_NAS_ProtocolConfigOptionsTX(f_EUTRA38_MultiPDN_GetPCOs(p_CellId, v_PDN_Type, // @sic R5s220883 sic@ v_IsIMS, v_EpsDefaultBearerId, p_ExtdPcoFromUE, p_PcoFromUE, p_N1disabled)); // @sic R5-227443 sic@ if (isvalue (p_ExtdPcoFromUE)) { // FFS - At the moment, 36.508 table 4.7.3-6 specifies ExtdPCO should be not present } if (isvalue(p_PduInfoFromNR)) { // @sic R5s210149 sic@ // The UE has just moved from NR. If so, data from NR should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromNR); i := i + 1) { if (v_PdnIndex == valueof(p_PduInfoFromNR[i].PdnIndex)) { // Data from NR should be used, rather than created anew v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function break; } } } if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@ f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, (hex2int(v_EpsDefaultBearerId)-4))); } v_PDN_Address := f_EUTRA_GetPDNAddress(p_IPv4AllocationViaNasFlag, v_PdnIndex); f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId, v_EpsDefaultBearerId, tsc_RRC_TI_Def, p_EPS_TI, v_AttachType, v_PDN_Address, v_Guti, cs_TAIListNonConsecutive_lv(v_PLMN, {bit2oct(v_TAC)}), v_LAI, v_MSId, v_ESM_Cause, v_APN_Tx, // @sic R5-206296 sic@ v_PcoToUE, f_GetAdditionalUpdateResult(p_AdditionalUpdateType, p_ForcedAttach), p_T3412, p_T3402, p_T3423, p_EquivalentPlmnList, v_EmergNumList, -, -, v_BearerContextNumber, p_EMM_Cause, omit, p_DCN_Id, omit, -, p_UERadioCapId); // @sic R5-216280 sic@ return v_EpsDefaultBearerId; }