3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230284

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3023 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC TC 9.1.1.2 against SNPN-only UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Derivation of keys CK' and IK' for EAP-AKA' requires the Serving Network Name which needs to be updated to SNPN identity in case of SNPN. From TS 33.501 I.3.2: SN Id = PLMN ID:NID |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialize and pass the NID for NGC Cell A to the functions building the Authentication Request in Step 5, Step 7, and Step 9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

3	References	7

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

Change 1 of R5s230229[1] is also required to pass the TC in SNPN.

## Change 1

| Function name | function fl_TC_9_1_1_2_TestBody() |
| --- | --- |
| Reason for change | Derivation of keys CK' and IK' for EAP-AKA' requires the Serving Network Name which needs to be updated to SNPN identity in case of SNPN. From TS 33.501 I.3.2: SN Id = PLMN ID:NID |
| Summary of change | Initialize and pass the NID for NGC Cell A to the functions building the Authentication Request in Step 5, Step 7, and Step 9 |
| TTCN module | Authentication_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_1_2_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var PLMN_Identity v_ASN_WrongPLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellE); var NAS_PlmnId v_WrongPLMN := f_NR_Asn2Nas_PlmnId(v_ASN_WrongPLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var octetstring v_EAPMsg; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ //@siclog "Steps 2-4" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@siclog "Step 5" siclog@ // use incorrect SNN SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_AuthenticationReqMsg(v_NAS_SecurityParams, v_WrongPLMN)))); //@siclog "Step 6" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAP_AuthReject_AuthenticationResponse(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } //@siclog "Step 6A" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_AuthenticationResultMsg(v_NAS_SecurityParams, cs_EAP_Failure)))); // @sic R5-205579 sic@ //@siclog "Step 7" siclog@ // Cannot call Get function as need to use same security parameters in both EAP and Authentication Req msg v_NAS_SecurityParams := f_NG_EAP_AuthenticationInit(v_NAS_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)); // @sic R5s210111 sic@ v_EAPMsg := f_NG_EAP_ChallengeReq (v_NAS_SecurityParams, v_PLMN, '01'O); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_AUTHENTICATION_REQUEST (v_NAS_SecurityParams.KSIamf, cs_ABBA (v_NAS_SecurityParams.ABBA, omit), -, -, cs_EAP_Message(v_EAPMsg))))); // @sic R5s201483, R5s210111 sic@ //@siclog "Step 8" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAPAKA_AuthenticationResponse(v_NAS_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, -, '01'O)) { // @sic R5-205579, R5s210111 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { // @sic R5s201483 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } //@siclog "Step 9" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_OtherAuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, f_NG_EAP_Notification(cs_EAP_AT_NOTIFICATION ('0'B, '1'B, int2bit (0, 14))))))); // @sic R5s201021 sic@ //@siclog "Step 10" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAP_Notification_AuthenticationResponse(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } else { // @sic R5s201483 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); } //@siclog "Step 11" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_AUTHENTICATION_REJECT(cs_EAP_Message(bit2oct(encvalue(cs_EAP_Failure('01'O)))))))); //@siclog "Step 12" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 13" siclog@ f_NR_CheckNoRRCSetupReq (v_NGC_NASCellA, 30.0); //@siclog "Step 14" siclog@ f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_DEREGISTERED); //@siclog "Step 15" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s211299 sic@ //@siclog "Step 16" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); // @sic R5s201025 sic@ if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?, ?), '111'B, ?, -, -, -, -, omit)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } //@siclog Step 17 siclog@ f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, rrcConnectionRelease, Initial_NoSecurity); // @sic R5s201025 sic@ }

After Change:

import from NR_CellInfoInit all; import from NR_Parameters all; function fl_TC_9_1_1_2_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var PLMN_Identity v_ASN_WrongPLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellE); var NAS_PlmnId v_WrongPLMN := f_NR_Asn2Nas_PlmnId(v_ASN_WrongPLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var octetstring v_EAPMsg; var NID_r16 v_NID_ASN; var template (omit) hexstring v_NID := omit; if(pc_SNPN_only){ v_NID_ASN := f_NR_InitNDI(v_NGC_NASCellA); // returns a bitstring v_NID := bit2hex(v_NID_ASN); } //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ //@siclog "Steps 2-4" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@siclog "Step 5" siclog@ // use incorrect SNN SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_AuthenticationReqMsg(v_NAS_SecurityParams, v_WrongPLMN, v_NID)))); //@siclog "Step 6" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAP_AuthReject_AuthenticationResponse(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } //@siclog "Step 6A" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_AuthenticationResultMsg(v_NAS_SecurityParams, cs_EAP_Failure)))); // @sic R5-205579 sic@ //@siclog "Step 7" siclog@ // Cannot call Get function as need to use same security parameters in both EAP and Authentication Req msg v_NAS_SecurityParams := f_NG_EAP_AuthenticationInit(v_NAS_SecurityParams, v_PLMN, hex2str(px_IMSI_Def), -, -, v_NID); // @sic R5s210111 sic@ v_EAPMsg := f_NG_EAP_ChallengeReq (v_NAS_SecurityParams, v_PLMN, '01'O, v_NID); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_AUTHENTICATION_REQUEST (v_NAS_SecurityParams.KSIamf, cs_ABBA (v_NAS_SecurityParams.ABBA, omit), -, -, cs_EAP_Message(v_EAPMsg))))); // @sic R5s201483, R5s210111 sic@ //@siclog "Step 8" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAPAKA_AuthenticationResponse(v_NAS_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, -, '01'O)) { // @sic R5-205579, R5s210111 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { // @sic R5s201483 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } //@siclog "Step 9" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_OtherAuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, f_NG_EAP_Notification(cs_EAP_AT_NOTIFICATION ('0'B, '1'B, int2bit (0, 14))), v_NID)))); // @sic R5s201021 sic@ //@siclog "Step 10" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAP_Notification_AuthenticationResponse(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } else { // @sic R5s201483 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); } //@siclog "Step 11" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_AUTHENTICATION_REJECT(cs_EAP_Message(bit2oct(encvalue(cs_EAP_Failure('01'O)))))))); //@siclog "Step 12" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 13" siclog@ f_NR_CheckNoRRCSetupReq (v_NGC_NASCellA, 30.0); //@siclog "Step 14" siclog@ f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_DEREGISTERED); //@siclog "Step 15" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s211299 sic@ //@siclog "Step 16" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); // @sic R5s201025 sic@ if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?, ?), '111'B, ?, -, -, -, -, omit)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } //@siclog Step 17 siclog@ f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, rrcConnectionRelease, Initial_NoSecurity); // @sic R5s201025 sic@ }

# References

| [1] | R5s230229: NR5GC FR1 : Addition of Rel-16 SNPN test case 9.1.11.3 |
| --- | --- |