3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230293

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3029 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1: Addition of Rel-17 Redcap test case 6.1.2.27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Anritsu, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 6.1.2.27 to the IWD_23wk12 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.1.2.27 required for approval. See the detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.27.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessNotAllowed()	4

2.2	Function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed()	5

3	Branches executed	6

4	Execution Log Files	6

4.1	MTK MT6835	6

4.2	MTK MT6835	7

4.3	HiSilicon Balong 5000	7

5	References	7

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.1.2.27 which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D23wk12.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:

Shaun Harry

Shaun.harry@keysight.com

Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

Narendra Kalahasti

narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.1.2.27

ATS Version:	iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset solution, Anritsu Protocol Conformance Test System and R&S® 5G Protocol Conformance Test

platform

UE used:	MTK6835, Hisilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessNotAllowed()

| Function name | f_NR5GC_CellInfo_SetSIB4_RedCap_AccessNotAllowed() |
| --- | --- |
| Reason for change | 1. InterFreqCarrierFreqInfo_v1700 template is not created yet for Sib4 so will get Runtime exception on accessing its IEs. i.e., TemplateConversionException., f_NR5GC_CellInfo_SetSIB4_RedCap_AccessNotAllowed(), in script: Idle_CellSelection_NR5GC, line: 83 |
| Summary of change | 1. Created InterFreqCarrierFreqInfo_v1700 template and set .redCapAccessAllowed_r17 := omit and initialized other parameters to omit as well. |
| TTCN module | 6_1_2\Idle_CellSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessNotAllowed(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var integer v_SIB4_SI_Index := f_NR_GetSIB4_SI_Index(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination); v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700[0].redCapAccessAllowed_r17 := omit; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

After Change:

function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessNotAllowed(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var integer v_SIB4_SI_Index := f_NR_GetSIB4_SI_Index(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination); //WA#WI=1077557 var template(omit) InterFreqCarrierFreqInfo_v1700 v_InterFreqCarrierFreqList_v1700 := omit; v_InterFreqCarrierFreqList_v1700.redCapAccessAllowed_r17 := omit; v_InterFreqCarrierFreqList_v1700.highSpeedMeasInterFreq_r17 := omit; v_InterFreqCarrierFreqList_v1700.interFreqNeighCellList_v1710 := omit; v_InterFreqCarrierFreqList_v1700.interFreqNeighHSDN_CellList_r17 := omit; v_InterFreqCarrierFreqList_v1700.ssb_PositionQCL_Common_r17 := omit; v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700[0] := valueof(v_InterFreqCarrierFreqList_v1700); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

## Function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed()

| Function name | f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed |
| --- | --- |
| Reason for change | - With Change 1 above in f_NR5GC_CellInfo_SetSIB4_RedCap_AccessNotAllowed() InterFreqCarrierFreqList_v1700 template would be created and this function can work but if we keep these two as independent functions to be called in any order for other cases if needed later on, then InterFreqCarrierFreqList_v1700 template needs to be created and initialized as well. |
| Summary of change | 1. Created InterFreqCarrierFreqInfo_v1700 template and set .redCapAccessAllowed_r17 := true_ and initialized other parameters to omit. |
| TTCN module | 6_1_2\Idle_CellSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var integer v_SIB4_SI_Index := f_NR_GetSIB4_SI_Index(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination); v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700[0].redCapAccessAllowed_r17 := true_; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

After Change:

function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var integer v_SIB4_SI_Index := f_NR_GetSIB4_SI_Index(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination); //WA#WI=1077557 var template(omit) InterFreqCarrierFreqInfo_v1700 v_InterFreqCarrierFreqList_v1700 := omit; v_InterFreqCarrierFreqList_v1700.redCapAccessAllowed_r17 := true_; v_InterFreqCarrierFreqList_v1700.highSpeedMeasInterFreq_r17 := omit; v_InterFreqCarrierFreqList_v1700.interFreqNeighCellList_v1710 := omit; v_InterFreqCarrierFreqList_v1700.interFreqNeighHSDN_CellList_r17 := omit; v_InterFreqCarrierFreqList_v1700.ssb_PositionQCL_Common_r17 := omit; v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700[0] := valueof(v_InterFreqCarrierFreqList_v1700); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

# Branches executed

## This NR TC was executed in NR5GC FR1 Band NR band n78 using NR ciphering nea2 and integrity algorithm nia2 on Keysight S8704A Protocol Conformance Toolset solution, on NR5GC FR1 Band NR band n41 using NR ciphering nea2 and integrity algorithm nia2 on Anritsu Protocol Conformance Test System ME7834NR and on NR band n41 with NEA1 and NIA1on R&S 5G Conformance Test Platform.

# Execution Log Files

## MTK MT6835

The Mediatek MT6835 UE passed this test case on Keysight S8704A Protocol Conformance Toolset solution. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_1_2_27_LOG.html

- PICS/PIXIT parameter file:

TC_6_1_2_27_PICS_PIXIT.xml

## MTK MT6835

The Mediatek MT6835 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_1_2_27_LOG.xml

- PICS/PIXIT parameter file:

TC_6_1_2_27_PIXIT.txt

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_6_1_2_27_Hisilicon.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230294: NR5GC FR1: Supporting information for addition of Rel-17 Redcap test case 6.1.2.27 |
| --- | --- |