3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230297

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3032 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR UE capability testcases 8.1.5.1.1 and 8.2.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the Current TTCN Implementation of the function fl_NRCap_CommonChecks_r16 , before calling the function fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension) , presence of NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension to be checked to avoid the runtime issue - In the current TTCN Implementation of the function f_NR_UESupportsFR2Band , v_FoundFR2 is decided based on the presence of FR2 band in the constant defined list tsc_SupportedBandListNR which is anyway having FR2 band in it . Function f_NR_UESupportsFR2Band always sets v_FoundFR2 = True, even for FR1 UE, which is incorrect. The function check f_NR_UESupportsFR2Band is used in the function definitions fl_Check_maxNumberMIMO_LayersPDSCH and in fl_Check_MeasAndMobParametersCommon_supportedGapPattern to decide FR2 based implementation. To avoid the issue, it is proposed to check against the “pc_nrFR2” instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - ispresent is checked before calling fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension) - In the function definitions fl_Check_maxNumberMIMO_LayersPDSCH and fl_Check_MeasAndMobParametersCommon_supportedGapPattern, “pc_nrFR2” is used instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC, 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function fl_NRCap_CommonChecks_r16	3

2.2	Correction to the Function fl_Check_maxNumberMIMO_LayersPDSCH	5

2.3	Correction to the Function fl_Check_MeasAndMobParametersCommon_supportedGapPattern	7

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_NRCap_CommonChecks_r16

| Function name | fl_NRCap_CommonChecks_r16 |
| --- | --- |
| Reason for change | In the Current TTCN Implementation of the function fl_NRCap_CommonChecks_r16 , before calling the function fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension) , presence of NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension to be checked to avoid the run time issue . |
| Summary of change | ispresent is checked before calling fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension) |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_NRCap_CommonChecks_r16(UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; v_Received_NRCapabilityMsg := p_NRCapability; fl_NRCap_CommonChecks_r15(v_Received_NRCapabilityMsg, p_Configuration_Type); fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension); // Input(s) in field UE-NR-Capability-v1610 if (p_Configuration_Type == NR_SA) { //@sic R5-225384 sic@ fl_Check_sidelinkParameters_r16(v_Received_NRCapabilityMsg); // Only in Test Case 8.1.5.1.1.NR5GC } } // end of fl_NRCap_CommonChecks_r16

After Change:

function fl_NRCap_CommonChecks_r16(UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; v_Received_NRCapabilityMsg := p_NRCapability; fl_NRCap_CommonChecks_r15(v_Received_NRCapabilityMsg, p_Configuration_Type); if(ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) //WA#WI=1078071 { fl_Check_NR_Notes_r16(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension); // Input(s) in field UE-NR-Capability-v1610 } if (p_Configuration_Type == NR_SA) { //@sic R5-225384 sic@ fl_Check_sidelinkParameters_r16(v_Received_NRCapabilityMsg); // Only in Test Case 8.1.5.1.1.NR5GC } } // end of fl_NRCap_CommonChecks_r16

## Correction to the Function fl_Check_maxNumberMIMO_LayersPDSCH

| Function name | fl_Check_maxNumberMIMO_LayersPDSCH |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_NR_UESupportsFR2Band , v_FoundFR2 is decided based on the presence of FR2 band in the constant defined list tsc_SupportedBandListNR which is anyway having FR2 band in it . Function f_NR_UESupportsFR2Band always sets v_FoundFR2 = True, even for FR1 UE, which is incorrect. The function check f_NR_UESupportsFR2Band is used in the function definitions fl_Check_maxNumberMIMO_LayersPDSCH and in fl_Check_MeasAndMobParametersCommon_supportedGapPattern to decide FR2 based implementation. To avoid the issue, it is proposed to check against the “pc_nrFR2” instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not.. |
| Summary of change | In the function definition fl_Check_maxNumberMIMO_LayersPDSCH, “pc_nrFR2” is used instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_Check_maxNumberMIMO_LayersPDSCH (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { << SKIPPED CODE >> // TS 38.306, clause 4.2.7.6 FeatureSetDownlinkPerCC parameters. // For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. if ((p_Configuration_Type == NR_SA) and (lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) == 1)) { if ((v_Current_MAX_MIMO_LayersDL < 4) and (f_NR_Is5GCoreBand(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR))) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory"); } if ((v_Current_MAX_MIMO_LayersDL == 0) and (f_NR_UESupportsFR2Band()) ) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 2 MIMO layers in FR2"); } } } else if (v_MIMO_LayersDLPICS_Set > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "A maxNumberMIMO_LayersPDSCH PICS is wrongly set as this feature is not supported by the UE"); } }

After Change:

function fl_Check_maxNumberMIMO_LayersPDSCH (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { << SKIPPED CODE >> // TS 38.306, clause 4.2.7.6 FeatureSetDownlinkPerCC parameters. // For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. if ((p_Configuration_Type == NR_SA) and (lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) == 1)) { if ((v_Current_MAX_MIMO_LayersDL < 4) and (f_NR_Is5GCoreBand(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR))) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory"); } if ((v_Current_MAX_MIMO_LayersDL == 0) and (/*f_NR_UESupportsFR2Band()*/pc_nrFR2) ) { //WA#WI=1078071 f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 2 MIMO layers in FR2"); } } } else if (v_MIMO_LayersDLPICS_Set > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "A maxNumberMIMO_LayersPDSCH PICS is wrongly set as this feature is not supported by the UE"); } }

## Correction to the Function fl_Check_MeasAndMobParametersCommon_supportedGapPattern

| Function name | fl_Check_MeasAndMobParametersCommon_supportedGapPattern |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_NR_UESupportsFR2Band , v_FoundFR2 is decided based on the presence of FR2 band in the constant defined list tsc_SupportedBandListNR which is anyway having FR2 band in it . Function f_NR_UESupportsFR2Band always sets v_FoundFR2 = True, even for FR1 UE, which is incorrect. The function check f_NR_UESupportsFR2Band is used in the function definitions fl_Check_maxNumberMIMO_LayersPDSCH and in fl_Check_MeasAndMobParametersCommon_supportedGapPattern to decide FR2 based implementation. To avoid the issue, it is proposed to check against the “pc_nrFR2” instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not.. |
| Summary of change | In the function definition fl_Check_MeasAndMobParametersCommon_supportedGapPattern, “pc_nrFR2” is used instead of making use of the function f_NR_UESupportsFR2Band() to decide if it is FR2 UE or not |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (f_NR_UESupportsFR2Band())) // This check applies only to option 2 and FR2 Band supported. { //The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 if (not((pc_gp17_nr and pc_gp18_nr and pc_gp19_nr))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_gp17_nr, pc_gp18_nr or pc_gp19_nr not correctly set for a NR standalone capable UE that supports a band in FR2"); } // v_PICS_GapPatterns_13and14_nr := '11'B; << SKIPPED CODE >>

After Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (/*f_NR_UESupportsFR2Band()*/pc_nrFR2)) // This check applies only to option 2 and FR2 Band supported. //WA#WI=1078071 { //The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 if (not((pc_gp17_nr and pc_gp18_nr and pc_gp19_nr))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_gp17_nr, pc_gp18_nr or pc_gp19_nr not correctly set for a NR standalone capable UE that supports a band in FR2"); } // v_PICS_GapPatterns_13and14_nr := '11'B; << SKIPPED CODE >>