3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230309

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3042 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR/5GC FR1: re-verification of Rel-16 power saving test case 7.1.1.12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 7.1.1.12.3 to the 5G/NR5GC ATS for FR1 path |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.12.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	10

2.3	Change 3	12

3	Branches executed	14

4	Execution Log Files	14

4.1	MediaTek MT6879	14

5	References	15

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.1.12.3, which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D23wk12.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.1.1.12.3.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	MediaTek MT6879

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_12_3_NR5GC |
| --- | --- |
| Reason for change | 1, According to R5-231902, searchspace ID for DCI_2_6 is 7. But it is configured to 6 2, searchSpacesToAddModList, searchSpacesToAddModListExt_r16 in rrcReconfiguration at step1 is incorrect 3, spCellConfigDedicated.tag_Id is not configured 4, v_DownlinkBWP.Pdcch.SearchSpaceArray is filed incorrect 5, SearchSpaceExt is not configured at SS 6, at step 7-8. v_PhysicalCellGroupConfig is changed, but it is not saved 7, at step 19A, the current ps_PositionDCI_2_6_r16 is 0. So the DCI_2_6 sent at this step should be configured with the 1st block 8, at step 20, Measurement Gap is configured. According to TP4, UE shall always starts the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH. So, it is not needed to send DCI_2_6 explicitly before postamble |
| Summary of change | 1, change searchSpaceId to 7 2, correct the context of rrcReconfiguration 3, set spCellConfigDedicated.tag_Id 4, correct v_DownlinkBWP.Pdcch.SearchSpaceArray 5, configure SearchSpaceExt at SS (also see Change 3) 6, call f_NR_CellInfo_SetPhysicalCellGroupConfig to save PhysicalCellGroupConfig at step 7-8 7, correct the configuration of DCI_2_6 blocks at step 19A 8, remove the DCI_2_6 before postamble |
| TTCN module | MAC_PowerSaving_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication ……… ……… var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_NextOccasion; var template(value) SearchSpace v_SearchSpace; //Init Cell parameters f_NR5GC_Init(NR_1); ……… ……… //Table 7.1.1.12.3.3.3-4 v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := 1; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet} v_SearchSpace := f_NR_CellInfo_GetSearchSpaceConfigAtUE(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_SearchSpace.searchSpaceId := 6; v_SearchSpace.controlResourceSetId := 1; v_SearchSpace.searchSpaceType := cs_NR_SearchSpace_SearchSpaceType_CSS_AllOmit; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModList[1] := v_SearchSpace; //@sic R5s230001 Ch1.3, R5-231902 sic@ v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_AllOmit, //@sic R5s230001 Ch.1.3, R5-231902 sic@ cs_SearchSpaceExt_r16_Dci_Format2_6} v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R15 := v_SearchSpace; v_DownlinkBWP.Pdcch.SearchSpaceArray[0].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_AllOmit; v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_Dci_Format2_6; f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); //@sic R5s230001 Ch.1.3 sic@ v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, -, -, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); v_LongDRXCycle := 10240; //@sic R5s230001 Ch.1.4 sic@ v_DrxStartOffset := 0; //@siclog Steps 3-6 siclog@ f_TC_7_1_1_12_3_NR_Steps3_6(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); //@siclog Steps 7-8 siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := true_; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog Steps 9-11 siclog@ ………………………………………………….. //@siclog Step 19A siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast(-,'1'B))); //@sic R5s230001, R5-231906 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(v_NextOccasion, 5); // add 5ms //@siclog Step 20 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) ……… ………. //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // leaving UE test state 3 v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); //@sic R5s230001 Ch1.9 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast( -,'1'B))); //@sic R5s230001 sic@ f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def), cs_TimingInfo_NR(v_NextOccasion)); //@sic R5s230001 sic@ f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication ………. ………. var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_NextOccasion; var template(value) SearchSpace v_SearchSpace; var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates; //Init Cell parameters f_NR5GC_Init(NR_1); ………. ……… //Table 7.1.1.12.3.3.3-4 v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := 1; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet} v_SearchSpace := f_NR_CellInfo_GetSearchSpaceConfigAtUE(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_SearchSpace.searchSpaceId := 7; v_SearchSpace.controlResourceSetId := 1; v_SearchSpace.searchSpaceType := cs_NR_SearchSpace_SearchSpaceType_CSS_AllOmit; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModList[0] := v_SearchSpace; //@sic R5s230001 Ch1.3, R5-231902 sic@ v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_Dci_Format2_6} //@sic R5s230001 Ch.1.3, R5-231902 sic@ v_SpCellConfig.spCellConfigDedicated.tag_Id := 0; v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_DownlinkBWP.Pdcch.SearchSpaceArray[0].SearchSpaceConfigAtUE.R15 := v_SearchSpace; v_DownlinkBWP.Pdcch.SearchSpaceArray[0].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_Dci_Format2_6; f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); //@sic R5s230001 Ch.1.3 sic@ if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_NrofCandidates := cs_38508_NrofCandidates_Def; } else { v_NrofCandidates := cds_38508_NrofCandidates_FR2 } f_NR_SS_SearchSpaceExtConfig(nr_Cell1, -, v_NrofCandidates); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, -, -, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); v_LongDRXCycle := 10240; //@sic R5s230001 Ch.1.4 sic@ v_DrxStartOffset := 0; //@siclog "Steps 3-6" siclog@ f_TC_7_1_1_12_3_NR_Steps3_6(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); //@siclog "Steps 7-8" siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := true_; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog "Steps 9-11" siclog@ …………………………………………….. //@siclog Step 19A siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast('1'B))); //@sic R5s230001, R5-231906 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(v_NextOccasion, 5); // add 5ms //@siclog Step 20 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) ……… ………. //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // leaving UE test state 3 v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); //@sic R5s230001 Ch1.9 sic@ //REMOVE f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast( -,'1'B))); //@sic R5s230001 sic@ f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def), cs_TimingInfo_NR(v_NextOccasion)); //@sic R5s230001 sic@ f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); };

## Change 2

| Function name | f_TC_7_1_1_12_3_NR_Steps14_19 |
| --- | --- |
| Reason for change | 1, according to Table 7.1.1.12.3.3.2-1, should delay 1280ms at step 14 2, p_LongDRXCycle is in unit ms. But f_NR_SubFrameTiming_AddSlots is called at step 18 |
| Summary of change | 1, delay 1.28s at step 14 2, replace f_NR_SubFrameTiming_AddSlots with f_SubFrameTiming_AddMilliSeconds at step18 |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_12_3_NR_Steps14_19(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication ……… ……… //@siclog Step 14 siclog@ //Wait 20ms to ensure UE is out DRX active time. f_Delay(0.02); //@siclog Step 15 siclog@ ……… ……… //@siclog "Step 18" siclog@ //In the PDCCH occasion when the next Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing ,p_LongDRXCycle*2); //@sic R5s230001 sic@ f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_WakeUpTiming))); //@sic R5s230001 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); ……… ……… }

After Change

function f_TC_7_1_1_12_3_NR_Steps14_19(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication ……… ……… //@siclog Step 14 siclog@ //Wait 1280ms to ensure UE is out DRX active time. f_Delay(1.28); //@siclog Step 15 siclog@ ……… ……… //@siclog "Step 18" siclog@ //In the PDCCH occasion when the next Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing ,p_LongDRXCycle*2); //@sic R5s230001 sic@ f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_WakeUpTiming))); //@sic R5s230001 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); ……… ……… }

## Change 3

| Function name | (new) f_NR_SS_SearchSpaceExtConfig (new) cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6 (new) cads_NR_SSExtConfig_REQ |
| --- | --- |
| Reason for change | For change 1.5 |
| Summary of change | new function and template to configure searchspaceExt at SS |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment |  |

(New)

template (value) NR_BWP_SearchSpaceConfig_Type cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates, SearchSpaceId p_SearchSpaceId := 7, ControlResourceSetId p_ControlResourceSetId := tsc_NR_ControlResourceSetId) := { TypeAndPriorityList :={ tsc_NR_SearchSpaceTypeAndPriority_CssType3}, AggregationLevel := AggregationLevel4, SearchSpaceConfigAtUE := { R15 := cs_38508_SearchSpace(p_SearchSpaceId, p_ControlResourceSetId, cs_NR_SearchSpace_SearchSpaceType_CSS_AllOmit, p_NrofCandidates), R16 := cs_SearchSpaceExt_r16_Dci_Format2_6 } }; template (value) NR_SYSTEM_CTRL_REQ cads_NR_SSExtConfig_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates ) modifies cas_NR_CellConfigAllOmit_REQ := { Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo), Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := omit, BWPs := omit, BWPs := { ActiveBWP := omit, BwpArray := { { Id := omit, BWP := omit, Pdcch := { SearchSpaceArray := {cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(p_NrofCandidates)}, CoresetArray := omit, RelativeTxPower := omit, Coreset0_OffsetRBs := omit }, Pdsch := omit, Sps := omit } } }, CsiConfig := omit }, Uplink := omit } } } } }; function f_NR_SS_SearchSpaceExtConfig(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates) runs on NR_BASE_PTC { f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_SSExtConfig_REQ(p_NR_CellId, p_TimingInfo, p_NrofCandidates)); }

# Branches executed

he test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## MediaTek MT6879

The MediaTek MT6879UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_12_3_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_1_12_3_PIXIT.xml

# References

| [1] | R5s230310: NR/5GC FR1: Supporting information for re-verification of Rel-16 power saving test case 7.1.1.12.3 |
| --- | --- |