3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230315

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3045 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR UE capability test cases 8.1.5.1.1 and 8.2.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the Current TTCN Implementation of the function fl_Check_MeasAndMobParametersCommon_supportedGapPattern , Gap Patterns 13,14,17,18 and 19 are expected to be “1” if UE configuration is NR_SA and it supports FR2 . Looks like this implementation is based on 38.306 broader statement “The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2“.More detailed explanation is given in 38.133 As per 38.133 , UE shall set Gap patterns 12-23 only if UE is NR SA FR2. In the current TTCN Implementation of the function f_MIMO_LayersDLParameterSet , else case verdict has to be corrected to match the scope of the function |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the function fl_Check_MeasAndMobParametersCommon_supportedGapPattern, Gap patterns 13,14,17,18 and 19 are checked if UE is only NR SA FR2 In the function f_MIMO_LayersDLParameterSet , else case verdict is corrected to match the function scope |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC, 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function fl_Check_MeasAndMobParametersCommon_supportedGapPattern	3

2.2	Correction to the Function fl_Check_maxNumberMIMO_LayersPDSCH	6

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_Check_MeasAndMobParametersCommon_supportedGapPattern

| Function name | fl_Check_MeasAndMobParametersCommon_supportedGapPattern |
| --- | --- |
| Reason for change | - In the Current TTCN Implementation of the function fl_Check_MeasAndMobParametersCommon_supportedGapPattern , Gap Patterns 13,14,17,18 and 19 are expected to be “1” if UE configuration is NR_SA and it supports FR2 . Looks like this implementation is based on 38.306 broader statement “The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2“ More detailed explanation is given in 38.133 As per 38.133 , UE shall set Gap patterns 12-23 only if UE is NR SA FR2. . |
| Summary of change | - In the function fl_Check_MeasAndMobParametersCommon_supportedGapPattern, Gap patterns 13,14,17,18 and 19 are checked if UE is only NR SA FR2. For change (1) , R5s230297 is covering part of the correction but which is more intended to replace the check “f_NR_UESupportsFR2Band” with pc_nrFR2 check ; as f_NR_UESupportsFR2Band always returns True |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (f_NR_UESupportsFR2Band()) // This check applies only to option 2 and FR2 Band supported. }

After Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { var UE_NR_Capability v_Received_NRCapabilityMsg; var B20_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; var B11_Type v_SupportedGapPatternsFromPics_1; var B9_Type v_SupportedGapPatternsFromPics_2; var B2_Type v_PICS_GapPatterns_13and14_nr := '11'B; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if ((p_Configuration_Type == NR_SA) and (/*f_NR_UESupportsFR2Band()*/((not pc_nrFR1) and pc_nrFR2))) // This check applies only to option 2 and FR2 Band supported. //WA#WI=1078071 //WA#WI=1081777 { << SKIPPED CODE >> }

## Correction to the Function fl_Check_maxNumberMIMO_LayersPDSCH

| Function name | f_MIMO_LayersDLParameterSet |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_MIMO_LayersDLParameterSet , else case verdict has to be corrected to match the scope of the function |
| Summary of change | In the function f_MIMO_LayersDLParameterSet , else case verdict is corrected to match the function scope |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MIMO_LayersDLParameterSet () return integer { var integer v_MIMO_LayersDLParameterSet := 0; if (f_UptoOneParameterSet(pc_maxNumberMIMO_LayersPDSCH_twoLayers, pc_maxNumberMIMO_LayersPDSCH_fourLayers, pc_maxNumberMIMO_LayersPDSCH_eightLayers)) { // Case where as much only one is set if (pc_maxNumberMIMO_LayersPDSCH_twoLayers){ v_MIMO_LayersDLParameterSet := 2; } if (pc_maxNumberMIMO_LayersPDSCH_fourLayers) { v_MIMO_LayersDLParameterSet := 4; } if (pc_maxNumberMIMO_LayersPDSCH_eightLayers){ v_MIMO_LayersDLParameterSet := 8; } } else { // Fail case f_SetVerdict(inconc,__FILE__, __LINE__, "More than one NR Measurement gap patterns PICS are set simultaneously"); } return v_MIMO_LayersDLParameterSet; }

After Change:

function f_MIMO_LayersDLParameterSet () return integer { var integer v_MIMO_LayersDLParameterSet := 0; if (f_UptoOneParameterSet(pc_maxNumberMIMO_LayersPDSCH_twoLayers, pc_maxNumberMIMO_LayersPDSCH_fourLayers, pc_maxNumberMIMO_LayersPDSCH_eightLayers)) { // Case where as much only one is set if (pc_maxNumberMIMO_LayersPDSCH_twoLayers){ v_MIMO_LayersDLParameterSet := 2; } if (pc_maxNumberMIMO_LayersPDSCH_fourLayers) { v_MIMO_LayersDLParameterSet := 4; } if (pc_maxNumberMIMO_LayersPDSCH_eightLayers){ v_MIMO_LayersDLParameterSet := 8; } } else { // Fail case f_SetVerdict(inconc,__FILE__, __LINE__, "More than one MIMO_LayersPDSCH PICS are set simultaneously"); //WA#WI=1081777 } return v_MIMO_LayersDLParameterSet; }